3GPP TSG-SA5 Meeting #128 	S5-197322

# Zhuhai (CN), 18 - 22 November 2019  	             	                       			 Revision of S5-19xxxx

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28.552 | CR |  |  |  | 0158 |  |  |  | rev |  |  | - |  |  | Current version: |  |  |  |  |  |  | 16.3.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | UICC apps |  |  |  |  |  |  | ME |  |  | Radio Access Network |  |  |  |  | X |  | Core Network |  |  |
| Title: |  |  |  | Packet Drop Rate measurements update |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  |  |  | Ericsson, Verizon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  | S5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  | 5G_SLICE_ePA |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  | 2019-11-04 |  |  |  |  |  |
| Category: |  |  |  | C | B |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-16 |  |  |  |  |  |
|  |  |  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |  |  |
| Reason for change: |  |  | The Packet Drop Rate measurements per S-NSSAI is of importance to the operator to monitor the Drop Rate for different S-NSSAIs. The existing definition of the Packet Drop Rate measurements are missing measurements per S-NSSAI. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  | The following Packet Drop Rate measurements are updated in line with the other measurements in 28.552 (that supports S-NSSAI) to optionally have subcounters per S-NSSAI: - DL PDCP SDU Drop rate in gNB-CU-UP - DL Packet Drop Rate in gNB-DU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  | The Packet Drop Rate measurements in 28.552 will not be defined for network slicing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  | 5.1.3.2.1, 5.1.3.2.2, A.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Y |  |  |  |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  |  |  |  | X | Other core specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| affected: |  |  |  |  |  |  |  | X | Test specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  |  |  |  |  | X | O&M Specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1st modified section

##### 5.1.3.2.1	DL PDCP SDU Drop rate in gNB-CU-UP

a)	This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per S-NSSAI.

NOTE: this measurement may include packets that were supposed to be sent via the eUtran air interface if using NR split bearer option 3, 4 or 7.

b)	SI.

c)	This measurement is obtained as: 1000000*Number of DL packets, for which no part has been transmitted over the F1-U or Xn-U or X2-U interface, of a data radio bearer, that are discarded in the PDCP layer, divided by Number of DL packets for data radio bearers that has entered PDCP upper SAP. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3) and per S-NSSAI.

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement is perfomed, the measurements are equal to the number of mapped 5QIs and the number of S-NSSAIs.

e)	The measurement name has the form DRB.PdcpPacketDropRateDl or optionally DRB.PdcpPacketDropRateDl.QOS 
where QOS identifies the target quality of service class, or optinally DRB.PdcpPacketDropRateDl.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

Next modified section

##### 5.1.3.2.2	DL Packet Drop Rate in gNB-DU

a)	This measurement provides the fraction of RLC SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-DU. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-DU without any part of it having been transmitted on the air interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per S-NSSAI.

b)	SI.

c)	This measurement is obtained as: 1000000*Number of DL packets, for which no part has been transmitted over the air, of a data radio bearer, that are discarded in the gNB-DU divided by Number of DL packets for data radio bearers that were received from gNB-CU-UP. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3) and per S-NSSAI.

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement is perfomed, the measurements are equal to the number of mapped 5QIs and the number of S-NSSAIs.

e)	The measurement name has the form DRB.RlcPacketDropRateDl or optionally DRB.RlcPacketDropRateDl.QOS 
where QOS identifies the target quality of service class, or optionally DRB.RlcPacketDropRateDl.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

Next modified section

# A.3	Monitoring of DL packet drop in NG-RAN

Keeping track of DL packet drops in the NG-RAN is essential, since for certain services packets that are dropped along the way through the system may have a noticeable impact on the end user. DL packet drop measurements can be useful for evaluation, optimization and for performance assurance of the network. Subcounters per QoS Level as well as per S-NSSAI is helpful for operator to pinpoint the reason for high packet drop rate.

For gNBs that are deployed in a split architecture, e.g. when parts of a gNB are deployed in a virtualized environment, the DL packet drops may occur in two parts; the gNB CU-UP and the gNB DU. Therefore, it is important to measure this separately.

End of modified section