3GPP TSG-SA5 Meeting # 128	S5-197361

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0784 | rev |  | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Caller and Callee information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | VBCLTE |  |  |  |  |  | Date: |  |  | 2019-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PCNs cannot collect caller information and callee information for VoLTE service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the caller information and callee information for volume based charging for VoLTE in PS domain CDRs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete definition for volume based charging for VoLTE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.2.X (new), 5.1.2.2.Y (new), 5.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS32.251 CR 0516 TS32.299 CR 0823 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.1.2.2.X	Callee Information

This field contains the Callee Information of VoLTE.

The Callee Information contains the following elements:

-	Called Party Address (described in clause 5.1.3.1.9);

-	Requested Party Address (described in clause 5.1.3.1.43);

-	List of Called Asserted Identity (described in clause 5.1.3.1.23).

Second change

##### 5.1.2.2.Y	Caller Information

This field contains the Caller Information of VoLTE as described in clause 5.1.3.1.24.

Third change

#### 5.2.2.2	PS domain CDRs

This subclause contains the abstract syntax definitions that are specific to the GPRS and EPC CDR types defined in TS 32.251 [11].

.$GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

# BEGIN

-- EXPORTS everything

# IMPORTS

AddressString,

CallDuration,

CallingNumber,

CauseForRecClosing,

CellId,

ChargingID,

CivicAddressInformation,

Diagnostics,

DiameterIdentity,

DynamicAddressFlag,

EnhancedDiagnostics,

GSNAddress,

InvolvedParty,

IPAddress,

LCSCause,

LCSClientIdentity,

LCSQoSInfo,

LevelOfCAMELService,

LocalSequenceNumber,

LocationAreaAndCell,

LocationAreaCode,

ManagementExtensions,

MBMSInformation,

MessageReference,

MSISDN,

MSTimeZone,

NodeID,

PDPAddress,

PLMN-Id,

PositioningData,

RATType,

RecordingEntity,

RecordType,

RoutingAreaCode,

SCSASAddress,

ServiceSpecificInfo,

SMSResult,

SmsTpDestinationNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

DefaultGPRS-Handling,

DefaultSMS-Handling,

NotificationToMSUser,

ServiceKey

FROM MAP-MS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)

gsm-Network (1) modules (3) map-MS-DataTypes (11) version18 (18)}

-- from TS 29.002 [214]

IMEI,

IMSI,

ISDN-AddressString,

RAIdentity

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CommonDataTypes (18) version18 (18)}

-- from TS 29.002 [214]

# CallReferenceNumber

FROM MAP-CH-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0)gsm-Network (1) modules (3) map-CH-DataTypes (13) version18 (18)}

-- from TS 29.002 [214]

Ext-GeographicalInformation,

LCSClientType,

LCS-Priority,

LocationType

FROM MAP-LCS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-LCS-DataTypes (25) version18 (18) }

-- from TS 29.002 [214]

# LocationMethod

FROM SS-DataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Access (2) modules (3) ss-DataTypes (2) version12 (12)}

-- from TS 24.080 [209]

;

--

--  GPRS RECORDS

--

GPRSRecord	::= CHOICE

--

-- Record values 20, 22..27 are specific

-- Record values 76, 77, 86 are MBMS specific

-- Record values 78,79 and 92, 95, 96 are EPC specific

--

{

sgsnPDPRecord			[20] SGSNPDPRecord,

sgsnMMRecord			[22] SGSNMMRecord,

sgsnSMORecord			[23] SGSNSMORecord,

sgsnSMTRecord			[24] SGSNSMTRecord,

sgsnMTLCSRecord			[25] SGSNMTLCSRecord,

sgsnMOLCSRecord			[26] SGSNMOLCSRecord,

sgsnNILCSRecord			[27] SGSNNILCSRecord,

sgsnMBMSRecord			[76] SGSNMBMSRecord,

ggsnMBMSRecord			[77] GGSNMBMSRecord,

sGWRecord				[78] SGWRecord,

pGWRecord				[79] PGWRecord,

gwMBMSRecord			[86] GWMBMSRecord,

tDFRecord				[92] TDFRecord,

iPERecord				[95] IPERecord,

ePDGRecord				[96] EPDGRecord,

tWAGRecord				[97] TWAGRecord

}

SGWRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

s-GWAddress					[4] GSNAddress,

chargingID					[5] ChargingID,

servingNodeAddress			[6] SEQUENCE OF GSNAddress,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servingNodePLMNIdentifier	[27] PLMN-Id OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

mSTimeZone 					[31] MSTimeZone OPTIONAL,

userLocationInformation		[32] OCTET STRING OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

servingNodeType				[35] SEQUENCE OF ServingNodeType,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 	[41] NULL OPTIONAL,

userCSGInformation			[42] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

lowPriorityIndicator		[44] NULL OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

s-GWiPv6Address				[48] GSNAddress OPTIONAL,

servingNodeiPv6Address		[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

userLocationInfoTime		[52] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[53] CNOperatorSelectionEntity OPTIONAL,

presenceReportingAreaInfo	[54] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation	[55] OCTET STRING OPTIONAL,

lastMSTimeZone				[56] MSTimeZone OPTIONAL,

enhancedDiagnostics			[57] EnhancedDiagnostics OPTIONAL,

cPCIoTEPSOptimisationIndicator [59] CPCIoTEPSOptimisationIndicator OPTIONAL,

uNIPDUCPOnlyFlag			[60] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl		[61] ServingPLMNRateControl OPTIONAL,

pDPPDNTypeExtension			[62] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter		[63] MOExceptionDataCounter OPTIONAL,

listOfRANSecondaryRATUsageReports [64] SEQUENCE OF RANSecondaryRATUsageReport OPTIONAL

}

PGWRecord 	::= SET

--

--	List of traffic volumes is only applicable when Charging per IP-CAN session is active and

--	IP-CAN bearer charging is being performed for the session.

--

--	EPC QoS Information is only applicable when Charging per IP-CAN session is active.

--

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

chargingID						[5] ChargingID,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext				[25] NULL OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

cAMELChargingInformation		[33] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID OPTIONAL,

iMSIunauthenticatedFlag 		[42] NULL OPTIONAL,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

lowPriorityIndicator				[46] NULL OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

userLocationInfoTime				[53] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		    	[54] CNOperatorSelectionEntity OPTIONAL,

ePCQoSInformation					[55] EPCQoSInformation OPTIONAL,

presenceReportingAreaInfo			[56] PresenceReportingAreaInfo OPTIONAL,

lastUserLocationInformation			[57] OCTET STRING OPTIONAL,

lastMSTimeZone						[58] MSTimeZone OPTIONAL,

enhancedDiagnostics					[59] EnhancedDiagnostics OPTIONAL,

nBIFOMMode							[60] NBIFOMMode OPTIONAL,

nBIFOMSupport						[61] NBIFOMSupport OPTIONAL,

uWANUserLocationInformation			[62] UWANUserLocationInfo OPTIONAL,

sGiPtPTunnellingMethod				[64] SGiPtPTunnellingMethod OPTIONAL,

uNIPDUCPOnlyFlag					[65] UNIPDUCPOnlyFlag OPTIONAL,

servingPLMNRateControl				[66] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[67] APNRateControl OPTIONAL,

pDPPDNTypeExtension					[68] PDPPDNTypeExtension OPTIONAL,

mOExceptionDataCounter				[69] MOExceptionDataCounter OPTIONAL,

chargingPerIPCANSessionIndicator    [70] ChargingPerIPCANSessionIndicator OPTIONAL,

threeGPPPSDataOffStatus				[71] ThreeGPPPSDataOffStatus OPTIONAL,

sCSASAddress						[72] SCSASAddress OPTIONAL,

listOfRANSecondaryRATUsageReports	[73] SEQUENCE OF RANSecondaryRATUsageReport OPTIONAL,

callerInformation					[74]  SEQUENCE OF InvolvedParty OPTIONAL,

calleeInformation					[75] CalleePartyInformation OPTIONAL

}

TDFRecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

p-GWAddress						[4] GSNAddress,

servingNodeAddress				[6] SEQUENCE OF GSNAddress,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

pdpPDNType						[8] PDPType OPTIONAL,

servedPDPPDNAddress				[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode				[21] APNSelectionMode OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

servingNodePLMNIdentifier		[27] PLMN-Id OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI						[29] IMEI OPTIONAL,

rATType							[30] RATType OPTIONAL,

mSTimeZone 						[31] MSTimeZone OPTIONAL,

userLocationInformation			[32] OCTET STRING OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servingNodeType					[35] SEQUENCE OF ServingNodeType,

servedMNNAI						[36] SubscriptionID OPTIONAL,

p-GWPLMNIdentifier				[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

served3gpp2MEID					[40] OCTET STRING OPTIONAL,

pDNConnectionChargingID			[41] ChargingID,

userCSGInformation				[43] UserCSGInformation OPTIONAL,

threeGPP2UserLocationInformation	[44] OCTET STRING OPTIONAL,

servedPDPPDNAddressExt 				[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt				[47] DynamicAddressFlag OPTIONAL,

servingNodeiPv6Address				[49] SEQUENCE OF GSNAddress OPTIONAL,

p-GWiPv6AddressUsed					[50] GSNAddress OPTIONAL,

tWANUserLocationInformation			[51] TWANUserLocationInfo OPTIONAL,

retransmission   					[52] NULL OPTIONAL,

tDFAddress							[53] GSNAddress,

tDFiPv6AddressUsed					[54] GSNAddress OPTIONAL,

tDFPLMNIdentifier					[55] PLMN-Id OPTIONAL,

servedFixedSubsID					[56] FixedSubsID OPTIONAL,

accessLineIdentifier				[57] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation		[59] FixedUserLocationInformation OPTIONAL

}

IPERecord 	::= SET

{

recordType						[0] RecordType,

servedIMSI						[3] IMSI OPTIONAL,

iPEdgeAddress					[4] GSNAddress,

chargingID						[5] ChargingID,

accessPointNameNI				[7] AccessPointNameNI OPTIONAL,

iPCANsessionType				[8] PDPType OPTIONAL,

servedIPCANsessionAddress		[9] PDPAddress OPTIONAL,

dynamicAddressFlag				[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes			[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime				[13] TimeStamp,

duration						[14] CallDuration,

causeForRecClosing				[15] CauseForRecClosing,

diagnostics						[16] Diagnostics OPTIONAL,

recordSequenceNumber			[17] INTEGER OPTIONAL,

nodeID							[18] NodeID OPTIONAL,

recordExtensions				[19] ManagementExtensions OPTIONAL,

localSequenceNumber				[20] LocalSequenceNumber OPTIONAL,

servedMSISDN					[22] MSISDN OPTIONAL,

chargingCharacteristics			[23] ChargingCharacteristics,

chChSelectionMode				[24] ChChSelectionMode OPTIONAL,

pSFurnishChargingInformation	[28] PSFurnishChargingInformation OPTIONAL,

servedIMEI		   				[29] IMEI OPTIONAL,

listOfServiceData				[34] SEQUENCE OF ChangeOfServiceCondition OPTIONAL,

servedMNNAI						[36] SubscriptionID OPTIONAL,

iPEdgeOperatorIdentifier		[37] PLMN-Id OPTIONAL,

startTime						[38] TimeStamp OPTIONAL,

stopTime						[39] TimeStamp OPTIONAL,

servedIPCANsessionAddressExt 	[45] PDPAddress OPTIONAL,

dynamicAddressFlagExt			[47] DynamicAddressFlag OPTIONAL,

iPEdgeiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission   				[52] NULL OPTIONAL,

servedFixedSubsID				[55] FixedSubsID OPTIONAL,

accessLineIdentifier			[56] AccessLineIdentifier OPTIONAL,

fixedUserLocationInformation	[57] FixedUserLocationInformation OPTIONAL

}

EPDGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

ePDGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

iMSsignalingContext			[25] NULL OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

ePDGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

uWANUserLocationInformation	[53] UWANUserLocationInfo OPTIONAL,

userLocationInfoTime		[54] TimeStamp OPTIONAL,

iMSIunauthenticatedFlag 	[55] NULL OPTIONAL

}

TWAGRecord 	::= SET

{

recordType					[0] RecordType,

servedIMSI					[3] IMSI OPTIONAL,

tWAGAddressUsed				[4] GSNAddress,

chargingID					[5] ChargingID,

accessPointNameNI			[7] AccessPointNameNI OPTIONAL,

pdpPDNType					[8] PDPType OPTIONAL,

servedPDPPDNAddress			[9] PDPAddress OPTIONAL,

dynamicAddressFlag			[11] DynamicAddressFlag OPTIONAL,

listOfTrafficVolumes		[12] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[13] TimeStamp,

duration					[14] CallDuration,

causeForRecClosing			[15] CauseForRecClosing,

diagnostics					[16] Diagnostics OPTIONAL,

recordSequenceNumber		[17] INTEGER OPTIONAL,

nodeID						[18] NodeID OPTIONAL,

recordExtensions			[19] ManagementExtensions OPTIONAL,

localSequenceNumber			[20] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[21] APNSelectionMode OPTIONAL,

servedMSISDN				[22] MSISDN OPTIONAL,

chargingCharacteristics		[23] ChargingCharacteristics,

chChSelectionMode			[24] ChChSelectionMode OPTIONAL,

servedIMEI					[29] IMEI OPTIONAL,

rATType						[30] RATType OPTIONAL,

sGWChange					[34] SGWChange OPTIONAL,

p-GWAddressUsed				[36] GSNAddress OPTIONAL,

p-GWPLMNIdentifier			[37] PLMN-Id OPTIONAL,

startTime					[38] TimeStamp OPTIONAL,

stopTime					[39] TimeStamp OPTIONAL,

pDNConnectionChargingID		[40] ChargingID OPTIONAL,

servedPDPPDNAddressExt 		[43] PDPAddress OPTIONAL,

dynamicAddressFlagExt		[47] DynamicAddressFlag OPTIONAL,

tWAGiPv6AddressUsed			[48] GSNAddress OPTIONAL,

p-GWiPv6AddressUsed			[50] GSNAddress OPTIONAL,

retransmission				[51] NULL OPTIONAL,

enhancedDiagnostics			[52] EnhancedDiagnostics OPTIONAL,

tWANUserLocationInformation	[53] TWANUserLocationInfo OPTIONAL,

iMSIunauthenticatedFlag 	[54] NULL OPTIONAL

}

SGSNMMRecord	::= SET

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

sgsnAddress					[3] GSNAddress OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

routingArea					[5] RoutingAreaCode OPTIONAL,

locationAreaCode			[6] LocationAreaCode OPTIONAL,

cellIdentifier				[7] CellId OPTIONAL,

changeLocation				[8] SEQUENCE OF ChangeLocation OPTIONAL,

recordOpeningTime			[9] TimeStamp,

duration					[10] CallDuration OPTIONAL,

sgsnChange					[11] SGSNChange OPTIONAL,

causeForRecClosing			[12] CauseForRecClosing,

diagnostics					[13] Diagnostics OPTIONAL,

recordSequenceNumber		[14] INTEGER OPTIONAL,

nodeID						[15] NodeID OPTIONAL,

recordExtensions			[16] ManagementExtensions OPTIONAL,

localSequenceNumber			[17] LocalSequenceNumber OPTIONAL,

servedMSISDN				[18] MSISDN OPTIONAL,

chargingCharacteristics		[19] ChargingCharacteristics,

cAMELInformationMM 			[20] CAMELInformationMM OPTIONAL,

rATType						[21] RATType OPTIONAL,

chChSelectionMode			[22] ChChSelectionMode OPTIONAL,

cellPLMNId					[23] PLMN-Id OPTIONAL,

servingNodePLMNIdentifier	[24] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[25] CNOperatorSelectionEntity OPTIONAL

}

SGSNPDPRecord 	::= SET

{

recordType					[0] RecordType,

networkInitiation			[1] NetworkInitiatedPDPContext OPTIONAL,

servedIMSI					[3] IMSI OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

sgsnAddress					[5] GSNAddress OPTIONAL,

msNetworkCapability			[6] MSNetworkCapability OPTIONAL,

routingArea					[7] RoutingAreaCode OPTIONAL,

locationAreaCode			[8] LocationAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

chargingID					[10] ChargingID,

ggsnAddressUsed				[11] GSNAddress,

accessPointNameNI			[12] AccessPointNameNI OPTIONAL,

pdpType						[13] PDPType OPTIONAL,

servedPDPAddress			[14] PDPAddress OPTIONAL,

listOfTrafficVolumes		[15] SEQUENCE OF ChangeOfCharCondition OPTIONAL,

recordOpeningTime			[16] TimeStamp,

duration					[17] CallDuration,

sgsnChange					[18] SGSNChange OPTIONAL,

causeForRecClosing			[19] CauseForRecClosing,

diagnostics					[20] Diagnostics OPTIONAL,

recordSequenceNumber		[21] INTEGER OPTIONAL,

nodeID						[22] NodeID OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

apnSelectionMode			[25] APNSelectionMode OPTIONAL,

accessPointNameOI			[26] AccessPointNameOI OPTIONAL,

servedMSISDN				[27] MSISDN OPTIONAL,

chargingCharacteristics		[28] ChargingCharacteristics,

rATType						[29] RATType OPTIONAL,

cAMELInformationPDP  		[30] CAMELInformationPDP OPTIONAL,

rNCUnsentDownlinkVolume		[31] DataVolumeGPRS OPTIONAL,

chChSelectionMode			[32] ChChSelectionMode OPTIONAL,

dynamicAddressFlag			[33] DynamicAddressFlag OPTIONAL,

iMSIunauthenticatedFlag 	[34] NULL OPTIONAL,

userCSGInformation			[35] UserCSGInformation OPTIONAL,

servedPDPPDNAddressExt 		[36] PDPAddress OPTIONAL,

lowPriorityIndicator		[37] NULL OPTIONAL,

servingNodePLMNIdentifier	[38] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		[39] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMORecord	::= SET

--

--   also for MME UE originated SMS record

--

{

recordType					[0] RecordType,

servedIMSI					[1] IMSI,

servedIMEI					[2] IMEI OPTIONAL,

servedMSISDN				[3] MSISDN OPTIONAL,

msNetworkCapability			[4] MSNetworkCapability OPTIONAL,

serviceCentre				[5] AddressString OPTIONAL,

recordingEntity				[6] RecordingEntity OPTIONAL,

locationArea				[7] LocationAreaCode OPTIONAL,

routingArea					[8] RoutingAreaCode OPTIONAL,

cellIdentifier				[9] CellId OPTIONAL,

messageReference			[10] MessageReference,

eventTimeStamp				[11] TimeStamp,

smsResult					[12] SMSResult OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

nodeID						[14] NodeID OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		[16] ChargingCharacteristics,

rATType						[17] RATType OPTIONAL,

destinationNumber			[18] SmsTpDestinationNumber OPTIONAL,

cAMELInformationSMS			[19] CAMELInformationSMS OPTIONAL,

chChSelectionMode			[20] ChChSelectionMode OPTIONAL,

servingNodeType				[21] ServingNodeType,

servingNodeAddress			[22] GSNAddress OPTIONAL,

servingNodeiPv6Address		[23] GSNAddress OPTIONAL,

mMEName						[24] DiameterIdentity OPTIONAL,

mMERealm					[25] DiameterIdentity OPTIONAL,

userLocationInformation		[26] OCTET STRING OPTIONAL,

retransmission				[27] NULL OPTIONAL,

servingNodePLMNIdentifier	[28] PLMN-Id OPTIONAL,

userLocationInfoTime		[29] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		[30] CNOperatorSelectionEntity OPTIONAL

}

SGSNSMTRecord	::= SET

--

--   also for MME UE terminated SMS record

--

{

recordType					 [0] RecordType,

servedIMSI					 [1] IMSI,

servedIMEI					 [2] IMEI OPTIONAL,

servedMSISDN				 [3] MSISDN OPTIONAL,

msNetworkCapability			 [4] MSNetworkCapability OPTIONAL,

serviceCentre				 [5] AddressString OPTIONAL,

recordingEntity				 [6] RecordingEntity OPTIONAL,

locationArea				 [7] LocationAreaCode OPTIONAL,

routingArea					 [8] RoutingAreaCode OPTIONAL,

cellIdentifier				 [9] CellId OPTIONAL,

eventTimeStamp				 [10] TimeStamp,

smsResult					 [11] SMSResult OPTIONAL,

recordExtensions			 [12] ManagementExtensions OPTIONAL,

nodeID						 [13] NodeID OPTIONAL,

localSequenceNumber			 [14] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [15] ChargingCharacteristics,

rATType						 [16] RATType OPTIONAL,

chChSelectionMode			 [17] ChChSelectionMode OPTIONAL,

cAMELInformationSMS			 [18] CAMELInformationSMS OPTIONAL,

originatingAddress			 [19] AddressString OPTIONAL,

servingNodeType				 [20] ServingNodeType,

servingNodeAddress			 [21] GSNAddress OPTIONAL,

servingNodeiPv6Address		 [22] GSNAddress OPTIONAL,

mMEName						 [23] DiameterIdentity OPTIONAL,

mMERealm					 [24] DiameterIdentity OPTIONAL,

userLocationInformation		 [25] OCTET STRING OPTIONAL,

retransmission				 [26] NULL OPTIONAL,

servingNodePLMNIdentifier	 [27] PLMN-Id OPTIONAL,

userLocationInfoTime		 [28] TimeStamp OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMTLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType,

lcsClientIdentity			 [3] LCSClientIdentity,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationType				 [7] LocationType,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

notificationToMSUser		 [13] NotificationToMSUser OPTIONAL,

privacyOverride				 [14] NULL OPTIONAL,

location					 [15] LocationAreaAndCell OPTIONAL,

routingArea					 [16] RoutingAreaCode OPTIONAL,

locationEstimate			 [17] Ext-GeographicalInformation OPTIONAL,

positioningData				 [18] PositioningData OPTIONAL,

lcsCause					 [19] LCSCause OPTIONAL,

diagnostics					 [20] Diagnostics OPTIONAL,

nodeID						 [21] NodeID OPTIONAL,

localSequenceNumber			 [22] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [23] ChargingCharacteristics,

chChSelectionMode			 [24] ChChSelectionMode OPTIONAL,

rATType						 [25] RATType OPTIONAL,

recordExtensions			 [26] ManagementExtensions OPTIONAL,

causeForRecClosing			 [27] CauseForRecClosing,

servingNodePLMNIdentifier	 [28] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [29] CNOperatorSelectionEntity OPTIONAL

}

SGSNMOLCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

locationMethod				 [7] LocationMethod,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNNILCSRecord			::= SET

{

recordType					 [0] RecordType,

recordingEntity				 [1] RecordingEntity,

lcsClientType				 [2] LCSClientType OPTIONAL,

lcsClientIdentity			 [3] LCSClientIdentity OPTIONAL,

servedIMSI					 [4] IMSI OPTIONAL,

servedMSISDN				 [5] MSISDN OPTIONAL,

sgsnAddress					 [6] GSNAddress OPTIONAL,

servedIMEI					 [7] IMEI OPTIONAL,

lcsQos						 [8] LCSQoSInfo OPTIONAL,

lcsPriority					 [9] LCS-Priority OPTIONAL,

mlcNumber					 [10] ISDN-AddressString OPTIONAL,

eventTimeStamp				 [11] TimeStamp,

measurementDuration			 [12] CallDuration OPTIONAL,

location					 [13] LocationAreaAndCell OPTIONAL,

routingArea					 [14] RoutingAreaCode OPTIONAL,

locationEstimate			 [15] Ext-GeographicalInformation OPTIONAL,

positioningData				 [16] PositioningData OPTIONAL,

lcsCause					 [17] LCSCause OPTIONAL,

diagnostics					 [18] Diagnostics OPTIONAL,

nodeID						 [19] NodeID OPTIONAL,

localSequenceNumber			 [20] LocalSequenceNumber OPTIONAL,

chargingCharacteristics		 [21] ChargingCharacteristics,

chChSelectionMode			 [22] ChChSelectionMode OPTIONAL,

rATType						 [23] RATType OPTIONAL,

recordExtensions			 [24] ManagementExtensions OPTIONAL,

causeForRecClosing			 [25] CauseForRecClosing,

servingNodePLMNIdentifier	 [26] PLMN-Id OPTIONAL,

cNOperatorSelectionEnt		 [27] CNOperatorSelectionEntity OPTIONAL

}

SGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofRAs					[3] SEQUENCE OF RAIdentity OPTIONAL,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

sgsnPLMNIdentifier			[15] PLMN-Id OPTIONAL,

numberofReceivingUE			[16] INTEGER OPTIONAL,

mbmsInformation				[17] MBMSInformation OPTIONAL

}

GGSNMBMSRecord	::= SET

{

recordType					[0] RecordType,

ggsnAddress					[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

servedPDPAddress			[5] PDPAddress OPTIONAL,

listOfTrafficVolumes		[6] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[7] TimeStamp,

duration					[8] CallDuration,

causeForRecClosing			[9] CauseForRecClosing,

diagnostics					[10] Diagnostics OPTIONAL,

recordSequenceNumber		[11] INTEGER OPTIONAL,

nodeID						[12] NodeID OPTIONAL,

recordExtensions			[13] ManagementExtensions OPTIONAL,

localSequenceNumber			[14] LocalSequenceNumber OPTIONAL,

mbmsInformation				[15] MBMSInformation OPTIONAL

}

GWMBMSRecord	::= SET

{

recordType					[0] RecordType,

mbmsGWAddress				[1] GSNAddress,

chargingID					[2] ChargingID,

listofDownstreamNodes		[3] SEQUENCE OF GSNAddress,

accessPointNameNI			[4] AccessPointNameNI OPTIONAL,

pdpPDNType					[5] PDPType OPTIONAL,

servedPDPPDNAddress			[6] PDPAddress OPTIONAL,

listOfTrafficVolumes		[7] SEQUENCE OF ChangeOfMBMSCondition OPTIONAL,

recordOpeningTime			[8] TimeStamp,

duration					[9] CallDuration,

causeForRecClosing			[10] CauseForRecClosing,

diagnostics					[11] Diagnostics OPTIONAL,

recordSequenceNumber		[12] INTEGER OPTIONAL,

nodeID						[13] NodeID OPTIONAL,

recordExtensions			[14] ManagementExtensions OPTIONAL,

localSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

mbmsInformation				[16] MBMSInformation OPTIONAL,

commonTeid   				[17] CTEID OPTIONAL,

iPMulticastSourceAddress	[18] PDPAddress OPTIONAL

}

--

--  PS DATA TYPES

--

AccessAvailabilityChangeReason		::= INTEGER (0..4294967295)

--

-- 0 (RAN rule indication)   : This value shall be used to indicate that the availability

--                             of an access is changed due to the RAN rule indication.

-- 1 (Access usable/unusable): This value shall be used to indicate that the availability

--                             of an access is changed due to the access is unusable or usable

--                             again.

--

AccessLineIdentifier	::= SEQUENCE

--

-- "Physical Access Id" includes a port identifier and the identity of the access node where the

--  port resides. "logical Access Id" contains a Circuit-ID. Both are defined ETSI TS 283 034 [314]

--

{

physicalAccessID	[0] UTF8String OPTIONAL,

logicalAccessID		[1] OCTET STRING OPTIONAL

}

AccessPointNameNI	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of APN in  dot representation.

-- For example, if the complete APN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- NI is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

AccessPointNameOI	::= IA5String (SIZE(1..37))

--

-- Operator Identifier part of APN in dot representation.

-- In the 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs' example, the OI portion is 'mnc022.mcc111.gprs'

-- and is presented in this form in the CDR.

--

ADCRuleBaseName		::= IA5String

--

-- identifier for the group of charging rules

-- see ADC-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

AdditionalExceptionReports		::= ENUMERATED

{

notAllowed		(0),

allowed			(1)

}

AFChargingIdentifier	::= OCTET STRING

--

-- see AF-Charging-Identifier AVP as defined in TS 29.214[221]

--

AFRecordInformation	::= SEQUENCE

{

aFChargingIdentifier		[1] AFChargingIdentifier,

flows						[2] Flows OPTIONAL

}

APNRateControl		::= SEQUENCE

--

-- See TS 24.008 [208] for more information

--

{

aPNRateControlUplink	[0] APNRateControlParameters OPTIONAL,

aPNRateControlDownlink	[1] APNRateControlParameters OPTIONAL

}

APNRateControlParameters		::= SEQUENCE

{

additionalExceptionReports	[0] AdditionalExceptionReports OPTIONAL,

rateControlTimeUnit			[1] RateControlTimeUnit OPTIONAL,

rateControlMaxRate			[2] INTEGER OPTIONAL,

rateControlMaxMessageSize	[3] DataVolumeGPRS OPTIONAL -- aPNRateControlDownlink only

}

APNSelectionMode	::= ENUMERATED

--

-- See Information Elements TS 29.060 [215], TS 29.274 [223] or TS 29.275 [224]

--

{

mSorNetworkProvidedSubscriptionVerified				(0),

mSProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

CalleePartyInformation	::= SEQUENCE

{

called-Party-Address				[0] InvolvedParty OPTIONAL,

requested-Party-Address				[1] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 	[2] SEQUENCE OF InvolvedParty OPTIONAL

}

CAMELAccessPointNameNI	::= AccessPointNameNI

CAMELAccessPointNameOI	::= AccessPointNameOI

CAMELInformationMM		::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

numberOfDPEncountered  			[4] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[5] LevelOfCAMELService OPTIONAL,

freeFormatData					[6] FreeFormatData OPTIONAL,

fFDAppendIndicator				[7] FFDAppendIndicator OPTIONAL

}

CAMELInformationPDP	::= SET

{

sCFAddress						[1] SCFAddress OPTIONAL,

serviceKey						[2] ServiceKey OPTIONAL,

defaultTransactionHandling		[3] DefaultGPRS-Handling OPTIONAL,

cAMELAccessPointNameNI			[4] CAMELAccessPointNameNI OPTIONAL,

cAMELAccessPointNameOI			[5] CAMELAccessPointNameOI OPTIONAL,

numberOfDPEncountered			[6] NumberOfDPEncountered OPTIONAL,

levelOfCAMELService				[7] LevelOfCAMELService OPTIONAL,

freeFormatData					[8] FreeFormatData OPTIONAL,

fFDAppendIndicator				[9] FFDAppendIndicator OPTIONAL

}

CAMELInformationSMS		::= SET

{

sCFAddress								[1] SCFAddress OPTIONAL,

serviceKey								[2] ServiceKey OPTIONAL,

defaultSMSHandling						[3] DefaultSMS-Handling OPTIONAL,

cAMELCallingPartyNumber					[4] CallingNumber OPTIONAL,

cAMELDestinationSubscriberNumber		[5] SmsTpDestinationNumber OPTIONAL,

cAMELSMSCAddress						[6] AddressString OPTIONAL,

freeFormatData							[7] FreeFormatData OPTIONAL,

smsReferenceNumber						[8] CallReferenceNumber	OPTIONAL

}

ChangeCondition	::= ENUMERATED

{

qoSChange						(0),

tariffTime						(1),

recordClosure					(2),

cGI-SAICHange					(6),	-- bearer modification. "CGI-SAI Change"

rAIChange						(7),	-- bearer modification. "RAI Change"

dT-Establishment				(8),

dT-Removal						(9),

eCGIChange						(10),	-- bearer modification. "ECGI Change"

tAIChange						(11),	-- bearer modification. "TAI Change"

userLocationChange				(12),	-- bearer modification. "User Location Change"

userCSGInformationChange		(13), 	-- bearer modification. "User CSG info Change"

presenceInPRAChange				(14),	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

removalOfAccess					(15),	-- NBIFOM "Removal of Access"

unusabilityOfAccess				(16),	-- NBIFOM "Unusability of Access"

indirectChangeCondition			(17),	-- NBIFOM "Indirect Change Condition"

userPlaneToUEChange				(18),	-- bearer modification. "Change of user plane to UE"

servingPLMNRateControlChange	(19),

-- bearer modification “Serving PLMN Rate Control Change"

threeGPPPSDataOffStatusChange	(20),   -- "Change of 3GPP PS DataO ff Status"

aPNRateControlChange			(21)	-- bearer modification. "APN Rate ControlChange"

}

ChangeOfCharCondition	::= SEQUENCE

--

-- qosRequested and qosNegotiated are used in S-CDR only

-- ePCQoSInformation used in SGW-CDR,PGW-CDR, IPE-CDR, TWAG-CDR and ePDG-CDR only

-- userLocationInformation is used only in S-CDR, SGW-CDR and PGW-CDR

-- chargingID used in PGW-CDR only when Charging per IP-CAN session is active

-- accessAvailabilityChangeReason and relatedChangeOfCharCondition applicable only in PGW-CDR

-- cPCIoTOptimisationIndicator is used in SGW-CDR only

-- aPNRateControl is valid for PGW-CDR only

--

{

qosRequested					[1] QoSInformation OPTIONAL,

qosNegotiated							[2] QoSInformation OPTIONAL,

dataVolumeGPRSUplink					[3] DataVolumeGPRS OPTIONAL,

dataVolumeGPRSDownlink					[4] DataVolumeGPRS OPTIONAL,

changeCondition							[5] ChangeCondition,

changeTime								[6] TimeStamp,

userLocationInformation					[8] OCTET STRING OPTIONAL,

ePCQoSInformation						[9] EPCQoSInformation OPTIONAL,

chargingID								[10] ChargingID OPTIONAL,

presenceReportingAreaStatus 			[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation						[12] UserCSGInformation OPTIONAL,

diagnostics								[13] Diagnostics OPTIONAL,

enhancedDiagnostics						[14] EnhancedDiagnostics OPTIONAL,

rATType									[15] RATType OPTIONAL,

accessAvailabilityChangeReason			[16] AccessAvailabilityChangeReason OPTIONAL,

uWANUserLocationInformation				[17] UWANUserLocationInfo OPTIONAL,

relatedChangeOfCharCondition			[18] RelatedChangeOfCharCondition OPTIONAL,

cPCIoTEPSOptimisationIndicator			[19] CPCIoTEPSOptimisationIndicator OPTIONAL,

servingPLMNRateControl					[20] ServingPLMNRateControl OPTIONAL,

threeGPPPSDataOffStatus					[21] ThreeGPPPSDataOffStatus OPTIONAL,

listOfPresenceReportingAreaInformation	[22] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL,

aPNRateControl							[23] APNRateControl OPTIONAL

}

ChangeOfMBMSCondition	::= SEQUENCE

--

-- Used in MBMS record

--

{

qosRequested				[1] QoSInformation OPTIONAL,

qosNegotiated				[2] QoSInformation OPTIONAL,

dataVolumeMBMSUplink		[3] DataVolumeMBMS OPTIONAL,

dataVolumeMBMSDownlink		[4] DataVolumeMBMS,

changeCondition				[5] ChangeCondition,

changeTime					[6] TimeStamp,

failureHandlingContinue		[7] FailureHandlingContinue OPTIONAL

}

ChangeOfServiceCondition	::= SEQUENCE

--

-- Used for Flow based Charging and Application based Charging service data container

-- presenceReportingAreaStatus is used in PGW-CDR Only

--

{

ratingGroup 						[1] RatingGroupId,

chargingRuleBaseName				[2] ChargingRuleBaseName OPTIONAL,

resultCode							[3] ResultCode OPTIONAL,

localSequenceNumber					[4] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage					[5] TimeStamp OPTIONAL,

timeOfLastUsage						[6] TimeStamp OPTIONAL,

timeUsage 							[7] CallDuration OPTIONAL,

serviceConditionChange				[8] ServiceConditionChange,

qoSInformationNeg					[9] EPCQoSInformation OPTIONAL,

servingNodeAddress 					[10] GSNAddress OPTIONAL,

datavolumeFBCUplink					[12] DataVolumeGPRS OPTIONAL,

datavolumeFBCDownlink				[13] DataVolumeGPRS OPTIONAL,

timeOfReport						[14] TimeStamp,

failureHandlingContinue				[16] FailureHandlingContinue OPTIONAL,

serviceIdentifier					[17] ServiceIdentifier OPTIONAL,

pSFurnishChargingInformation		[18] PSFurnishChargingInformation OPTIONAL,

aFRecordInformation					[19] SEQUENCE OF AFRecordInformation OPTIONAL,

userLocationInformation				[20] OCTET STRING OPTIONAL,

eventBasedChargingInformation		[21] EventBasedChargingInformation OPTIONAL,

timeQuotaMechanism					[22] TimeQuotaMechanism OPTIONAL,

serviceSpecificInfo					[23] SEQUENCE OF ServiceSpecificInfo OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

sponsorIdentity						[25] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[26] OCTET STRING OPTIONAL,

aDCRuleBaseName						[27] ADCRuleBaseName OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedChangeOfServiceCondition		[33] RelatedChangeOfServiceCondition OPTIONAL,

servingPLMNRateControl				[35] ServingPLMNRateControl OPTIONAL,

aPNRateControl						[36] APNRateControl OPTIONAL,

threeGPPPSDataOffStatus             [37] ThreeGPPPSDataOffStatus OPTIONAL,

trafficSteeringPolicyIDDownlink     [38] TrafficSteeringPolicyIDDownlink OPTIONAL,

trafficSteeringPolicyIDUplink       [39] TrafficSteeringPolicyIDUplink OPTIONAL,

tWANUserLocationInformation			[40] TWANUserLocationInfo OPTIONAL,

listOfPresenceReportingAreaInformation	[41] SEQUENCE OF PresenceReportingAreaInfo OPTIONAL

}

ChangeLocation	::= SEQUENCE

--

-- used in SGSNMMRecord only

--

{

locationAreaCode		[0] LocationAreaCode,

routingAreaCode			[1] RoutingAreaCode,

cellId					[2] CellId OPTIONAL,

changeTime				[3] TimeStamp,

mCC-MNC					[4] PLMN-Id OPTIONAL

}

ChargingCharacteristics	::= OCTET STRING (SIZE(2))

ChargingPerIPCANSessionIndicator	::= ENUMERATED

{

inactive		(0),

active               (1)

}

ChargingRuleBaseName	::= IA5String

--

-- identifier for the group of charging rules

-- see Charging-Rule-Base-Name AVP as desined in TS 29.212 [220]

--

ChChSelectionMode		::= ENUMERATED

{

servingNodeSupplied			(0),	-- For S-GW/P-GW

subscriptionSpecific		(1),	-- For SGSN only

aPNSpecific					(2),	-- For SGSN only

homeDefault					(3),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

roamingDefault				(4),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

visitingDefault				(5),	-- For SGSN, S-GW, P-GW, TDF and IP-Edge

fixedDefault				(6)		-- For TDF and IP-Edge

}

CNOperatorSelectionEntity	::= ENUMERATED

{

servCNSelectedbyUE		(0),

servCNSelectedbyNtw		(1)

}

# CPCIoTEPSOptimisationIndicator ::= BOOLEAN

CSGAccessMode		::= ENUMERATED

{

closedMode  (0),

hybridMode  (1)

}

CSGId		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 23.003 [200]. Coded according to TS 29.060 [215] for GTP, and

-- in TS 29.274 [223] for eGTP.

--

CTEID		::= OCTET STRING (SIZE(4))

--

-- Defined in TS 32.251[11] for MBMS-GW-CDR. Common Tunnel Endpoint Identifier

-- of MBMS GW for user plane, defined in TS 23.246 [207].

--

DataVolumeGPRS		::= INTEGER

--

-- The volume of data transferred in octets.

--

# DataVolumeMBMS ::= INTEGER

--

-- The volume of data transferred in octets.

--

EPCQoSInformation	::= SEQUENCE

--

-- See TS 29.212 [220] for more information

--

{

qCI							[1] INTEGER,

maxRequestedBandwithUL		[2] INTEGER OPTIONAL,

maxRequestedBandwithDL		[3] INTEGER OPTIONAL,

guaranteedBitrateUL			[4] INTEGER OPTIONAL,

guaranteedBitrateDL			[5] INTEGER OPTIONAL,

aRP							[6] INTEGER OPTIONAL,

aPNAggregateMaxBitrateUL	[7] INTEGER OPTIONAL,

aPNAggregateMaxBitrateDL	[8] INTEGER OPTIONAL,

extendedMaxRequestedBWUL	[9] INTEGER OPTIONAL,

extendedMaxRequestedBWDL	[10] INTEGER OPTIONAL,

extendedGBRUL				[11] INTEGER OPTIONAL,

extendedGBRDL				[12] INTEGER OPTIONAL,

extendedAPNAMBRUL			[13] INTEGER OPTIONAL,

extendedAPNAMBRDL			[14] INTEGER OPTIONAL

}

EventBasedChargingInformation		::= SEQUENCE

{

numberOfEvents		[1] INTEGER,

eventTimeStamps  	[2] SEQUENCE OF TimeStamp OPTIONAL

}

FailureHandlingContinue		::= BOOLEAN

--

-- This parameter is included when the failure handling procedure has been executed and new

-- containers are opened. This parameter shall be included in the first and subsequent

-- containers opened after the failure handling execution.

--

FFDAppendIndicator	::= BOOLEAN

FixedSubsID	::= OCTET STRING

--

-- The fixed subscriber Id identifier is defined in Broadband Forum TR 134 [601].

--

FixedUserLocationInformation	::= SEQUENCE

--

-- See format in IEEE Std 802.11-2012 [408] for "SSID" and "BSSID".

--

{

sSID					[0] OCTET STRING OPTIONAL ,

bSSID					[1] OCTET STRING OPTIONAL,

accessLineIdentifier	[2] AccessLineIdentifier OPTIONAL

}

Flows		::= 	SEQUENCE

--

-- See Flows AVP as defined in TS 29.214 [221]

--

{

mediaComponentNumber	[1] INTEGER,

flowNumber  			[2] SEQUENCE OF INTEGER OPTIONAL

}

FreeFormatData		::=	OCTET STRING (SIZE(1..160))

--

-- Free formatted data as sent in the FurnishChargingInformationGPRS

-- see TS 29.078 [217]

--

GSNAddress		::= IPAddress

MOExceptionDataCounter		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

counterValue		[0] INTEGER,

counterTimestamp	[1] TimeStamp

}

MSNetworkCapability	::= OCTET STRING (SIZE(1..8))

--

-- see TS 24.008 [208]

--

NBIFOMMode		::= ENUMERATED

{

uEINITIATED				(0),

nETWORKINITIATED		(1)

}

NBIFOMSupport 	::= ENUMERATED

{

nBIFOMNotSupported			(0),

nBIFOMSupported				(1)

}

NetworkInitiatedPDPContext		::= BOOLEAN

--

-- Set to true if PDP context was initiated from network side

--

# NumberOfDPEncountered  ::= INTEGER

PDPType		::= OCTET STRING (SIZE(2))

--

-- OCTET 1: PDP Type Organization

-- OCTET 2: PDP/PDN Type Number

-- See TS 29.060 [215] for encoding details.

--

PDPPDNTypeExtension	::= INTEGER

--

-- This integer is 1:1 copy of the PDP type value as defined in TS 29.061 [215].

--

# PresenceReportingAreaElementsList ::= OCTET STRING

--

-- For EPC see Presence-Reporting-Area-Elements-List AVP defined in TS 29.212 [220]

-- For 5GC see PresenceInfo defined in TS 29.571 [249] excluding praId and presenceState

--

PresenceReportingAreaInfo	::= SEQUENCE

{

presenceReportingAreaIdentifier	 [0] OCTET STRING,

presenceReportingAreaStatus		 [1] PresenceReportingAreaStatus OPTIONAL,

presenceReportingAreaElementsList[2] PresenceReportingAreaElementsList OPTIONAL,

presenceReportingAreaNode		 [3] PresenceReportingAreaNode OPTIONAL

}

# PresenceReportingAreaNode ::= BIT STRING

{

oCS   		 (0),

pCRF  		 (1)

}

--

--	This bit mask has the same format as Presence-Reporting-Area-Node AVP in TS 29.212 [220]

--

# PresenceReportingAreaStatus ::= ENUMERATED

{

insideArea   (0),

outsideArea  (1),

inactive 	 (2),

unknown      (3)

}

PSFurnishChargingInformation		::= SEQUENCE

{

pSFreeFormatData		[1] FreeFormatData,

pSFFDAppendIndicator	[2] FFDAppendIndicator OPTIONAL

}

QoSInformation	::= OCTET STRING (SIZE (4..255))

--

-- This  octet string

-- is a 1:1 copy of the contents (i.e. starting with octet 5) of the "Bearer Quality of

-- Service" information element specified in TS 29.274 [223].

--

RANSecondaryRATUsageReport	::= SEQUENCE

--

{

dataVolumeUplink				[1] DataVolumeGPRS,

dataVolumeDownlink				[2] DataVolumeGPRS,

rANStartTime					[3] TimeStamp,

rANEndTime						[4] TimeStamp,

secondaryRATType				[5] SecondaryRATType OPTIONAL,

chargingID						[6] ChargingID OPTIONAL

}

# RateControlTimeUnit ::= INTEGER

{	unrestricted	(0),

minute			(1),

hour			(2),

day				(3),

week			(4)

}

RatingGroupId		::= INTEGER

--

-- IP service flow identity (DCCA), range of 4 byte (0... 4294967295)

-- see Rating-Group AVP as used in TS 32.299 [50]

--

RelatedChangeOfCharCondition	::= SEQUENCE

{

changeCondition					[5] ChangeCondition,

changeTime						[6] TimeStamp,

userLocationInformation			[8] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 	[11] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation				[12] UserCSGInformation OPTIONAL,

rATType							[15] RATType OPTIONAL,

uWANUserLocationInformation		[17] UWANUserLocationInfo OPTIONAL

}

RelatedChangeOfServiceCondition	::= SEQUENCE

{

userLocationInformation				[20] OCTET STRING OPTIONAL,

threeGPP2UserLocationInformation	[24] OCTET STRING OPTIONAL,

presenceReportingAreaStatus 		[28] PresenceReportingAreaStatus OPTIONAL,

userCSGInformation					[29] UserCSGInformation OPTIONAL,

rATType								[30] RATType OPTIONAL,

uWANUserLocationInformation			[32] UWANUserLocationInfo OPTIONAL,

relatedServiceConditionChange		[33] ServiceConditionChange OPTIONAL

}

ResultCode		::= INTEGER

--

-- charging protocol return value, range of 4 byte (0... 4294967295)

-- see Result-Code AVP as used in 32.299 [40]

--

SecondaryRATType	::= INTEGER

{

nR				(0)		-- New Radio 5G

}

ServiceConditionChange	::= BIT STRING

{

qoSChange 								 (0),	-- bearer modification

sGSNChange 								 (1),	-- bearer modification:

-- apply to Gn-SGSN /SGW Change

sGSNPLMNIDChange 						 (2),	-- bearer modification

tariffTimeSwitch 						 (3),	-- tariff time change

pDPContextRelease 						 (4),	-- bearer release

rATChange 								 (5),	-- bearer modification

serviceIdledOut 						 (6),	-- IP flow idle out, DCCA QHT expiry

reserved 								 (7),	-- old: QCTexpiry is no report event

configurationChange 					 (8),	-- configuration change

serviceStop 							 (9),	-- IP flow termination.From "Service Stop" in

-- Change-Condition AVP

dCCATimeThresholdReached 				(10),	-- DCCA quota reauthorization

dCCAVolumeThresholdReached 				(11),	-- DCCA quota reauthorization

dCCAServiceSpecificUnitThresholdReached	(12),	-- DCCA quota reauthorization

dCCATimeExhausted 						(13),	-- DCCA quota reauthorization

dCCAVolumeExhausted 					(14),	-- DCCA quota reauthorization

dCCAValidityTimeout 					(15),	-- DCCA quota validity time (QVT expiry)

reserved1								(16),	-- reserved due to no use case,

-- old: return Requested is covered by (17),(18)

dCCAReauthorisationRequest 				(17),	-- DCCA quota reauthorization request by OCS

dCCAContinueOngoingSession 				(18),	-- DCCA failure handling (CCFH),

-- continue IP flow

dCCARetryAndTerminateOngoingSession		(19),	-- DCCA failure handling (CCFH),

-- terminate IP flow after DCCA retry

dCCATerminateOngoingSession 			(20),	-- DCCA failure handling,

-- terminate IP flow

cGI-SAIChange							(21),	-- bearer modification. "CGI-SAI Change"

rAIChange								(22),	-- bearer modification. "RAI Change"

dCCAServiceSpecificUnitExhausted		(23),	-- DCCA quota reauthorization

recordClosure							(24),	-- PGW-CDR closure

timeLimit								(25),	-- intermediate recording. From "Service Data

--	Time Limit" Change-Condition AVP value

volumeLimit								(26),	-- intermediate recording.From "Service Data

--	Volume Limit" Change-Condition AVP value

serviceSpecificUnitLimit				(27),	-- intermediate recording

envelopeClosure 						(28),

eCGIChange								(29),	-- bearer modification. "ECGI Change"

tAIChange								(30),	-- bearer modification. "TAI Change"

userLocationChange						(31),	-- bearer modification. "User Location Change"

userCSGInformationChange				(32),	-- bearer modification. "User CSG info Change"

presenceInPRAChange						(33), 	-- bearer modification. "Change of UE Presence

-- in Presence Reporting Area"

accessChangeOfSDF						(34),	-- "access change of service data flow"

indirectServiceConditionChange			(35),	-- NBIFOM: "indirect service condition change"

servingPLMNRateControlChange			(36), 	-- bearer modification. "Serving PLMNRate

-- Control Change"

aPNRateControlChange					(37)	-- bearer modification. "APN Rate ControlChange

}

--

-- Trigger and cause values for IP flow level recording are defined for support of independent

-- online and offline charging and also for tight interworking between online and offline charging.

-- Unused bits will always be zero.

-- Some of the values are non-exclusive (e.g. bearer modification reasons).

--

SCFAddress	::= AddressString

--

-- See TS 29.002 [214]

--

ServiceIdentifier	::= INTEGER (0..4294967295)

--

-- The service identifier is used to identify the service or the service component

-- the service data flow relates to. See Service-Identifier AVP as defined in TS 29.212 [220]

--

ServingNodeType	::= ENUMERATED

{

sGSN		(0),

pMIPSGW		(1),

gTPSGW		(2),

ePDG		(3),

hSGW		(4),

mME			(5),

tWAN		(6)

}

ServingPLMNRateControl		::= SEQUENCE

--

-- See TS 29.128 [244] for more information

--

{

sPLMNDLRateControlValue	[0] INTEGER,

sPLMNULRateControlValue	[1] INTEGER

}

SGiPtPTunnellingMethod		::= ENUMERATED

{

uDPIPbased		(0),

others			(1)

}

SGSNChange	::= BOOLEAN

--

-- present if first record after inter SGSN routing area update in new SGSN

--

SGWChange		::= BOOLEAN

--

-- present if first record after inter serving node change (SGW, ePDG, TWAG, HSGW)

--

TimeQuotaMechanism		::= SEQUENCE

{

timeQuotaType					[1] TimeQuotaType,

baseTimeInterval				[2] INTEGER

}

TimeQuotaType		::= ENUMERATED

{

dISCRETETIMEPERIOD			(0),

cONTINUOUSTIMEPERIOD		(1)

}

TrafficSteeringPolicyIDDownlink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-DL AVP as defined in TS 29.212[220]

TrafficSteeringPolicyIDUplink	::= OCTET STRING

--

-- see Traffic-Steering-Policy-Identifier-UL AVP as defined in TS 29.212[220]

TWANUserLocationInfo		::= SEQUENCE

{

sSID					 [0] OCTET STRING,			-- see format in IEEE Std 802.11-2012 [408]

bSSID					 [1] OCTET STRING OPTIONAL, 	-- see format in IEEE Std 802.11-2012 [408]

civicAddressInformation	[2] CivicAddressInformation OPTIONAL,

wLANOperatorId			[3] WLANOperatorId OPTIONAL,

logicalAccessID			[4] OCTET STRING OPTIONAL

}

UNIPDUCPOnlyFlag	::= BOOLEAN

UserCSGInformation		::= SEQUENCE

{

cSGId						[0] CSGId,

cSGAccessMode				[1] CSGAccessMode,

cSGMembershipIndication		[2] NULL OPTIONAL

}

UWANUserLocationInfo		::= SEQUENCE

{

uELocalIPAddress		 [0] IPAddress,

uDPSourcePort			 [1] OCTET STRING (SIZE(2)) OPTIONAL,

sSID					 [2] OCTET STRING OPTIONAL,	-- see format in IEEE Std 802.11-2012 [408]

bSSID					 [3] OCTET STRING OPTIONAL,	-- see format in IEEE Std 802.11-2012 [408]

tCPSourcePort			[4] OCTET STRING (SIZE(2)) OPTIONAL,

civicAddressInformation	[5] CivicAddressInformation OPTIONAL,

wLANOperatorId			[6] WLANOperatorId OPTIONAL,

logicalAccessID			[7] OCTET STRING OPTIONAL

}

# WLANOperatorId ::= SEQUENCE

{

wLANOperatorName	[0] OCTET STRING,

wLANPLMNId			[1] PLMN-Id

}

.#END

End of changes