3GPP TSG-SA5 Meeting # 128	S5-197418

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0004 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify design pattern for scoping and filtering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | REST_SS |  |  |  |  |  | Date: |  |  | 08-11-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The design patterns for scoping and filtering is described only on conceptual level. A detailed description how to use query parameters for that purpose is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing description how to use query parameters for scoping and filtering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Scoping and filtering cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

## 6.1	Design pattern for scoping and filtering

## 6.1.1	Introduction

Scoping is the process of targeting more than one resource for manipulation with HTTP methods. The URI query component shall be used for scoping resources below and including the resource identified by the URI path component (base resource).

Filtering is the process of selecting a subset of the scoped resources based on filtering criteria applied to the scoped resources. The URI query component shall be used for filtering.

Multiple query parameters shall be separated by an ampersand character ("&").

Scoping and filtering may be supported by the HTTP GET method or the HTTP DELETE method. It is not supported by any other method.

When multiple resources are targeted for retrieval by HTTP GET, the respone message body with the selected resource set shall be constructed according to one of the following rules.

Flat response construction method: The resources are basically returned as a flat list of JSON objects. Their location in the hierarchical containment tree needs to be specified by e.g. their URI which needs to be returned for each resource.

Hierarchical response construction method: The resources are returned inside the containment tree as specified by the JSON schema definition of the information model. The resources not selected are either not returned at all or returned empty, except for the resource identifiers, when their presence is required in the containment tree. The containment tree present in the response message shall always start with the root resource of the information model (document root) or the base resource.

### 6.1.2	Query parameters for scoping

Scoping is typically based on the resource hierarchy as specified by the name-containment relationships. A base resource is specified by the target URI, and one or more query parameters select one or more levels below and including the base object.

With one parameter the base resource and all resources until the level indicated by the query parameter can be selected. When the value of the query parameter is set to inifinite, the complete subtree starting at the base resource is selected.

Two query parameters for scoping allow for more sophisticated selection methods.

An example scoping method uses a "scopeType" and a "scopeLevel" query parameter. The allowed values are defined in Table 6.1.2-1.

Table 6.1.2-1: Allowed values of the "scopeType" query parameter

| Value | Description |
| --- | --- |
| BASE_ONLY | Selects only the base resource. The "scopeLevel" parameter shall be absent or ignored if present. |
| BASE_NTH_LEVEL | Selects all resources on the level, which is indicated by the "scopeLevel" parameter, below the base resource. The base resource is at "scopeLevel" zero. |
| BASE_SUBTREE | Selects the base resource and all of its subordinate resources down to and including the resources on the level indicated by the "scopeLevel" parameter. The base resource is at "scopeLevel" zero. |
| BASE_ALL | Selects the base resource and all of its subordinate resources (incl. the leaf resources). The "scopeLevel" parameter shall be absent or ignored if present. |

### 6.1.3	Query parameters for filtering

XPath 1.0 [15] shall be used for filtering scoped resources. The context resource for the XPath path expression is the resource identified by the targrt URI of the GET request.

A valid XPath expression returns a flat list of selected resources. Name-contained resources included in the selected resources shall be removed before constructing the response message according to clause 6.1.1.

The name of the query parameter shall be "filter".

End of modificatios