3GPP TSG-SA5 Meeting # 128	S5-197429

# Zhuhai,China,  18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0071 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct and update the RESTful HTTP-based solution set of provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE |  |  |  |  |  | Date: |  |  | 08-11-2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The solution set has some errors and inconsistencies with the latest version of TS 32.158 |  |  |  |  |  |  |  |  |
| Summary of change: |  | The errors are corrected and the inconsistencies with TS 32.158 are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CRUD operations cannot be implemented properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.1.1.1, 12.1.1.1.2, 12.1.1.1.3, 12.1.1.1.4, 12.1.1.1.4.1 (new), 12.1.1.1.4.2 (new), 12.1.1.1.5, A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First modification

#### 12.1.1.1	Mapping of operations

##### 12.1.1.1.1	Introduction

The IS operations are mapped to SS equivalents according to table 12.1.1.1.1-1.

Table 12.1.1.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | SQ |
| --- | --- | --- | --- |
| createMOI | PUT | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/{LDN-first-part}/{className}={id} | M |
| getMOIAttributes | GET | http://{URI-DN-PREFIX}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/{LDN-FIRST-PAT}/{className}={id} | M |
| modifyMOIAttributes | PUT PATCH | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/{LDN-first-part}/{className}={id} | M |
| deleteMOI | DELETE | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/{LDN-first-part}/{className}={id} | M |
| subscribe | POST | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/{URI-LDN}/subscriptions | M |
| unSubscribe | DELETE DELETE | http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/subscriptions http://{URI-DN-prefix}/{root}/{ProvMnS}/{MnSVersion}/subscriptions/{subscriptionId} | M M |

##### 12.1.1.1.2	Operation "createMOI"

This operation creates a single resource representing a managed object instance.

Table 12.1.1.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP PUT)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| managedObjectClass managedObjectInstance | path | /{className=id} | className: string id: string | M |
| referenceObjectInstance | n/a | n/a | n/a | n/a |
| attributeListIn | request body | data | resourcePut-RequestType | M |

The IS parameter "referenceObjectInstance" has no SS equivalent in the present document.

Table 12.1.1.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP PUT)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | data | resourceCreation-ResponseType | M |
| status | response status codes response body | n/a error | n/a error-ResponseType | M M |

The message flow for creating a resource is as follows:

1.	The Service Consumer sends a HTTP PUT request to the Service Provider.

The target URI identifies the location of the new resource to be created.

The message body shall carry the complete representation of the resource to be created.

2.	The Service Provider sends a HTTP PUT response to the Service Consumer.

On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource and the message body the complete representation of the new resource.

On failure, an appropriate error code shall be returned. The response message body should provide additional error information

##### 12.1.1.1.3	Operation "getMOIAttributes"

This operation retrieves one or multiple resources representing managed object instances.

Table 12.1.1.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| baseObjectInstance | path | /{className}={id} | className: string id: string | M |
| scope | query | scope | scope-QueryType style: form explode: true | O |
| filter | query | filter | filter-QueryType | O |
| attributeListIn | query | attributes fields | attributes-QueryType style: form explode: false fields-QueryType style: form explode: false | O O |

The SS parameters "scope", "filter", "attributes" and "fields" are defined in TS 32.158 [15].

Table 12.1.1.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP GET)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | data | resourceRetrieval-ResponseType | M |
| status | response status codes response body | n/a error | n/a error-ResponseType | M M |

The message flow for retrieval of one or multiple resources is as follows:

1.	The Service Consumer sends a HTTP GET request to the Service Provider.

The path component of the target URI identifies the base resource.

The "scope" query parameter identifies other resources besides the base resource.

The "filter" query parameter is applied to the set of scoped resources. Only resources passing the filter criteria are targeted.

The fields query parameter identifies the attributes to be returned.

2.	The Service Provider sends a HTTP GET response to the Service Consumer.

On success, "200 OK" shall be returned. The response message body is constructed according to the hierarchical response construction method (TS 32.158 [15]).

On failure, an appropriate error code shall be returned. The response message body shall provide additional error information

##### 12.1.1.1.4	Operation "modifyMOIAttributes"

###### 12.1.1.1.4.1	Mapping to HTTP PUT

HTTP PUT is used for a full update of a single resource.

Table 12.1.1.1.4.1-1: Mapping of IS operation input parameters to SS equivalents (HTTP PUT)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| baseObjectInstance | path | /{className}={id} | className: string id: string | M |
| scope | n/a | n/a | n/a | n/a |
| filter | n/a | n/a | n/a | n/a |
| modificationList | request body | data | resourcePut-RequestType | M |

The IS parameters "scope" and "filter" have no meaning when targerting a single resource with the target URI and are not mapped.

Table 12.1.1.1.4.1-2: Mapping of IS operation output parameters to SS equivalents (HTTP PUT)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | data | resourceUpdate-ResponseType | O |
| status | response status codes response body | n/a error | n/a error-ResponseType | M O |

The message flow is as follows:

1.	The Service Consumer sends a HTTP PUT request to the Service Provider.

The target URI identifies the target resource.

The message body shall contain the representation the target resource shall be replaced with.

2.	The Service Provider sends a HTTP PUT response to the Service Consumer.

On success, "200 OK" or "204 No Content" shall be returned. In the former case the response carries the representation of the updated resource in the message body. In the latter case the response has no message body. A "200 OK" response including the representation of the updated resource shall be sent in case the updated representation of the resource is not identical to the representation received in the request.

On failure, an appropriate error code shall be returned. The response message body may provide additional error information

###### 12.1.1.1.4.2	Mapping to HTTP PATCH

### HTTP PATCH is used to create, update or delete one or multiple resources.

Table 12.1.1.1.4.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP PATCH)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| baseObjectInstance | path | /{className}={id} | className: string id: string | M |
| scope | n/a | n/a | n/a | n/a |
| filter | n/a | n/a | n/a | n/a |
| modificationList | request body | n/a | jsonMergePatch-RequestType jsonEnhancedMergePatch-RequestType jsonPatch-RequestType | M |

The IS parameters "scope" and "filter" have no SS equivalents in the present document.

The MnS producer shall support at least one media type out of

-	application/merge-patch+json (RFC 7396 [12])

-	application/json-patch+json (RFC 6902 [13])

-	application/enhanced-merge-patch+json (TS 32.158 [15])

Table 12.1.1.1.4.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP PATCH)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| attributeListOut | n/a | n/a | n/a | n/a |
| status | response status codes response body | n/a error | n/a error-ResponseType | M O |

The message flow for modification of one or multiple resources is as follows:

1.	The Service Consumer sends a HTTP PATCH request to the Service Provider.

The path component of the target URI identifies the base resource.

The message body shall contain the patch document.

2.	The Service Provider sends a HTTP PATCH response to the Service Consumer.

On success, "200 OK" or "204 No Content" shall be returned. When "200 OK" is returned the message body shall include a representation of the updated resources constructed according to the hierarchical response construction method (TS 32.158 [15]).

On failure, an appropriate error code shall be returned. The response message body may provide additional error information

##### 12.1.1.1.5	Operation "deleteMOI"

This operation deletes one or multiple resources representing managed object instances.

Table 12.1.1.1.5-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| baseObjectInstance | path | /{className}={id} | className: string id: string | M |
| scope | query | scope | scope-QueryType | O |
| filter | query | filter | filter-QueryType | O |

Table 12.1.1.1.5-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | SQ |
| --- | --- | --- | --- | --- |
| deletionlist | response body | data | resourceDeletion-ResponseType | O |
| status | response status codes response body | n/a error | n/a error-ResponseType | M O |

The message flow for deletion of one or multiple resources is as follows:

1.	The Service Consumer sends a HTTP DELETE request to the Service Provider.

The path component of the target URI identifies the base resource.

If present, the "scope" query parameter identifies other resources besides the base resource.

The "filter" query parameter may be applied to the set of scoped resources. Only resources passing the filter criteria are targeted.

When no query parameters are present only the resource identified by the target URI shall be deleted.

2.	The Service Provider sends a HTTP DELETE response to the Service Consumer.

On success, when no query parameters are present in the request and only one resource is deleted, "204 No Content" shall be returned. Otherwise, when query parameters are present in the request, "200 OK" shall be returned and the response message body shall carry the URIs of the deleted resources.

On failure, an appropriate error code shall be returned. The response message body may provide additional error information

##### 12.1.1.1.6	Operation "subscribe"

See clause 12.2.1.1.8.

##### 12.1.1.1.7	Operation "unsubscribe"

See clause 12.2.1.1.9.

Next modificatios

## A.1	Provisioning management service

{

"openapi": "3.0.1",

"info": {

"title": "TS 28.532 Provisioning Management Service",

"version": "16.2.0",

"description": "OAS 3.0.1 specification of the Provisioning Management Service"

},

"servers": [

{

"url": "http://{URI-DN-prefix}/{root}/ProvMnS/v1620/{LDN-first-part}",

"variables": {

"URI-DN-prefix": {

"description": "See subclause 4.4 of TS 32.158",

"default": "example.com"

},

"root": {

"description": "See subclause 4.4 of TS 32.158",

"default": "3GPPManagement"

},

"LDN-first-part": {

"description": "See subclause 4.4 of TS 32.158",

"default": ""

}

}

}

],

"paths": {

"/{className}={id}": {

"parameters": [

{

"name": "className",

"in": "path",

"required": true,

"schema": {

"$ref": "#/components/schemas/className-PathType"

}

},

{

"name": "id",

"in": "path",

"required": true,

"schema": {

"$ref": "#/components/schemas/id-PathType"

}

}

],

"put": {

"summary": "Updates a single resource or creates it if it does not exist",

"description": "With HTTP PUT a complete resource is updated or created if it does not exist. The target resource is identified by the target URI.",

"requestBody": {

"required": true,

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourcePut-RequestType"

}

}

}

},

"responses": {

"200": {

"description": "Success case (\"200 OK\") for resource update. The representation of the updated resource is returned in the message body. This response shall be returned when the updated resource representation is not identical to the representation in the request.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceUpdate-ResponseType"

}

}

}

},

"201": {

"description": "Success case (\"201 Created\") for resource creation. The representation of the created resource is returned in the message body.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceCreation-ResponseType"

}

}

}

},

"204": {

"description": "Success case (\"204 No Content\") for resource update. The response has no message body. This response may be returned only when the updated resource representation is identical to the representation in the request."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"get": {

"summary": "Read resources",

"description": "With HTTP GET resources are read. The resources to be read are identified with the path component (base resource) and the query component (scope, filer) of the URI. The fields query component allows to select the resource properties to be returned.",

"parameters": [

{

"name": "scope",

"in": "query",

"description": "This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI. No scoping mechanism is specified in the present document.",

"required": false,

"schema": {

"$ref": "#/components/schemas/scope-QueryType"

},

"style": "form",

"explode": true

},

{

"name": "filter",

"in": "query",

"description": "This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to \"true\" are targeted. No filter language is specified in the present document.",

"required": false,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

},

{

"name": "attributes",

"in": "query",

"description": "This parameter specifies the attributes of the scoped resources that are returned.",

"required": true,

"schema": {

"$ref": "#/components/schemas/attributes-QueryType"

},

"style": "form",

"explode": false

},

{

"name": "fields",

"in": "query",

"description": "This parameter specifies the attribute field of the scoped resources that are returned.",

"required": false,

"schema": {

"$ref": "#/components/schemas/fields-QueryType"

},

"style": "form",

"explode": false

}

],

"responses": {

"200": {

"description": "Success case (\"200 OK\"). The resources identified in the request for retrieval are returned in the response message body. In case the fields query parameter is used, the selected attributes are returned.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceRetrieval-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"patch": {

"summary": "Patch one or multiple resources",

"description": "With HTTP PATCH resources are created, updated or deleted. The resources to be modified are identified with the path component of the URI (base resource) and with the patch document.",

"requestBody": {

"description": "The request body describes changes to be made to the target resources as defined in RFC 7396 (JSON Merge Patch).",

"required": true,

"content": {

"application/merge-patch+json": {

"schema": {

"$ref": "#/components/schemas/jsonMergePatch-RequestType"

}

},

"application/enhanced-merge-patch+json": {

"schema": {

"$ref": "#/components/schemas/jsonEnhancedMergePatch-RequestType"

}

},

"application/json-patch+json": {

"schema": {

"$ref": "#/components/schemas/jsonPatch-RequestType"

}

}

}

},

"responses": {

"200": {

"description": "Success case (\"200 OK\")."

},

"204": {

"description": "Success case (\"204 No Content\")."

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

},

"delete": {

"summary": "Delete one or multiple resources",

"description": "With HTTP DELETE resources are deleted. The resources to be deleted are identified with the path component (base resource) and the query component (scope, filer) of the URI.",

"parameters": [

{

"name": "scope",

"in": "query",

"description": "This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI. No scoping mechanism is specified in the present document.",

"required": false,

"schema": {

"$ref": "#/components/schemas/scope-QueryType"

}

},

{

"name": "filter",

"in": "query",

"description": "This parameter reduces the targeted set of resources by applying a filter to the scoped set of resource representations. Only resources representations for which the filter construct evaluates to \"true\" are returned. No filter language is specified in the present document.",

"required": false,

"schema": {

"$ref": "#/components/schemas/filter-QueryType"

}

}

],

"responses": {

"200": {

"description": "On success, when query parameters are present in the request, \"200 OK\" is returned. The message body carries the URIs of the deleted resources."

},

"204": {

"description": "On success, when no query parameters are present in the request and only one resource is deleted, \"204 No Content\" is returned. The message body is empty.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/resourceDeletion-ResponseType"

}

}

}

},

"default": {

"description": "Error case.",

"content": {

"application/json": {

"schema": {

"$ref": "#/components/schemas/error-ResponseType"

}

}

}

}

}

}

}

},

"components": {

"schemas": {

"uri-Type": {

"type": "string"

},

"nrm-Type": {

"type": "object"

},

"scopeType-Type": {

"type": "string",

"enum": [

"BASE_ONLY",

"BASE_NTH_LEVEL",

"BASE_SUBTREE",

"BASE_ALL"

]

},

"scopeLevel-Type": {

"type": "integer"

},

"className-PathType": {

"type": "string"

},

"id-PathType": {

"type": "string"

},

"attributes-QueryType": {

"type": "array",

"items": {

"type": "string"

}

},

"fields-QueryType": {

"type": "array",

"items": {

"type": "string"

}

},

"filter-QueryType": {

"type": "string"

},

"scope-QueryType": {

"type": "object",

"properties": {

"scopeType": {

"$ref": "#/components/schemas/scopeType-Type"

},

"scopeLevel": {

"$ref": "#/components/schemas/scopeLevel-Type"

}

}

},

"resourcePut-RequestType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

},

"jsonMergePatch-RequestType": {

"$ref": "#/components/schemas/nrm-Type"

},

"jsonEnhancedMergePatch-RequestType": {

"$ref": "#/components/schemas/nrm-Type"

},

"jsonPatch-RequestType": {

"type": "object"

},

"error-ResponseType": {

"type": "object",

"properties": {

"error": {

"type": "object",

"properties": {

"errorInfo": {

"type": "string"

}

}

}

}

},

"resourceCreation-ResponseType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

},

"resourceUpdate-ResponseType": {

"type": "object",

"properties": {

"data": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

},

"resourceDeletion-ResponseType": {

"type": "object",

"properties": {

"data": {

"type": "array",

"items": {

"$ref": "#/components/schemas/uri-Type"

}

}

}

},

"resourceRetrieval-ResponseType": {

"type": "object",

"properties": {

"data": {

"type": "array",

"items": {

"$ref": "#/components/schemas/resourceRepresentation-Type"

}

}

}

},

"resourceRepresentation-Type": {

"type": "object",

"properties": {

"href": {

"$ref": "#/components/schemas/uri-Type"

},

"class": {

"type": "string"

},

"id": {

"type": "string"

},

"attributes": {

"type": "object"

}

}

}

}

}

}

End of modificatios