3GPP TSG-SA5 Meeting #128	S5-197571

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019                                          Revision of S5-197286

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0034 | rev | 4 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS 28.550 Add UC and requirements for KPI job control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecommunications, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA-KPI |  |  |  |  |  | Date: |  |  | 2019-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the objective of the new work item KPI reporting, it mentioned that “enhance (with maximum reuse) the performance assurance MnS to support KPI collection and reporting”. In the current specfication TS 28.550, it is lacking the use cases and requirements for KPI job control. So it is proposed to add new KPI job control related use cases and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add usecase and requirements for KPI job control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no requirements and corresponding use cases for KPI job control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.X (new), 5.1.X.1 (new), 5.1.X.2 (new), 5.1.X.3 (new), 5.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.1.X	MnS responsible for KPI job control

5.1.X.1	 Creation of KPI job

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to create a KPI job for collecting the KPIs of one or more object instance(s). |  |
| Actors and Roles | An authorized consumer of the MnS responsible for KPI job control. |  |
| Telecom resources | NSSI(s), Network slice(s), Network(s)/sub-network(s); Producer of the MnS responsible for KPI job control. Producers of measurement job control services |  |
| Assumptions | N/A |  |
| Pre-conditions | - The NF(s), NSSI(s), Network slice(s) and Network(s)/sub-network(s) have been deployed. - The producer of the MnS responsible for KPI job control is in operation. |  |
| Begins when | The authorized consumer needs to create KPI job for collecting the KPIs of one or more object instance(s). |  |
| Step 1 (M) | The authorized consumer requests the producer of the MnS responsible for KPI job control to create KPI job to calculate and collect the KPIs of the subject object instance(s). The request needs to indicate that the KPIs needs to be reported by performance data file or by performance data streaming. |  |
| Step 2 (M) | The producer of the MnS responsible for KPI job control identifies the performance measurement type(s) used for the KPI(s) and checks whether the corresponding performance measurement type(s) can be collected by the existing measurement job(s). |  |
| Step 3 (M) | If measurement(s) are missing for the calculation of KPI(s), the management function hosting the producer of the MnS responsible for KPI job control would consume the necessary measurement MnS, for the missing measurement(s). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The KPI job for the subject object instance(s) has been created, and the producer of the MnS responsible for KPI job control generates the KPIs for the KPI job. |  |
| Traceability | REQ- KJCS_FUN-1, REQ- KJCS_FUN-2, REQ- KJCS_FUN-3 |  |

5.1.X.2	Termination of KPI job

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request the producer of the MnS responsible for KPI job control to terminate a KPI job. |  |
| Actors and Roles | An authorized consumer of the MnS responsible for KPI job control. |  |
| Telecom resources | NSSI(s), Network slice(s), Network(s)/sub-network(s); The producer of the MnS responsible for KPI job control. |  |
| Assumptions | N/A |  |
| Pre-conditions | The KPI job has been created. |  |
| Begins when | The authorized consumer does not need the KPI job that is calculating or collecting the KPIs. |  |
| Step 1 (M) | The authorized consumer requests the producer of the MnS responsible for KPI job control to terminate a KPI job that is calculating or collecting the KPIs. |  |
| Step 2 (M) | The producer of the MnS responsible for KPI job control terminates the KPI job. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The KPI job is terminated |  |
| Traceability | REQ- KJCS_FUN-4 |  |

5.1.X.3	Query of KPI jobs

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to query which KPI jobs are ongoing (i.e. the KPI jobs that have been created by the subject consumer and not terminated). |  |
| Actors and Roles | An authorized consumer of the MnS responsible for KPI job control. |  |
| Telecom resources | NSSI(s), Network slice(s), Network(s)/sub-network(s); The producer of the MnS responsible for KPI job control. |  |
| Assumptions | N/A |  |
| Pre-conditions | The producer of the MnS responsible for KPI job control is in operation. |  |
| Begins when | The authorized consumer needs to query the ongoing KPI jobs. |  |
| Step 1 (M) | The authorized consumer queries the information which KPI jobs are ongoing from the producer of the MnS responsible for KPI job control. |  |
| Step 2 (M) | The producer of the MnS responsible for KPI job control provides the information which KPI jobs are ongoing to the consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The information about the ongoing KPI jobs are available to the consumer. |  |
| Traceability | REQ- KJCS_FUN-5 |  |

Next change

5.2	Requirements

5.2.X	Requirements for MnS responsible for KPI production

REQ-KJCS_FUN-1		The management service producer responsible for KPI production shall have the capability allowing its authorized consumer to request production of KPI(s) of one or more object instance(s).

REQ-KJCS_FUN-2		The management service producer responsible for KPI production shall have the capability allowing its authorized consumer to indicate the reporting method (i.e. by performance data file or by performance data streaming) for the KPI(s).

REQ-KJCS_FUN-3		The management service producer responsible for KPI production shall have the capability to produce KPI(s) of one or more object instance(s) according to request of its authorized consumer.

REQ-KJCS_FUN-4		The management service producer responsible for KPI production shall have the capability allowing its authorized consumer to request termination of the production of KPI(s).

REQ-KJCS_FUN-5		The management service producer responsible for KPI production shall have the capability allowing its authorized consumer to query the information which KPI jobs are ongoing.

End of changes