3GPP TSG-SA5 Meeting #128	S5-197692

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019     							    Revision of S5-197214

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.274 | CR | 0075 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SMS Submission - ECUR flow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SMSCH |  |  |  |  |  | Date: |  |  | 2019-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The ECUR description on submit and delivery scenario is confused for converged charging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct on description on submit and delivery scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible for correctly use ECUR for SMS submission and delivery |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.4.2.4	SMS Submission - ECUR

Figure 5.4.2.4.1 describes the scenario where a SMS is submitted to the SMSF for ECUR mode.

# Figure 5.4.2.4.1: Converged charging SMS Submission using ECUR

1.	Initial procedures: see applicable flows.

2.	The SMSF receives a "SMS Submit or Forward SM" incoming message.

2ch-a.	The SMSF sends Charging Data Request [Initial] to CHF for authorization to submit or forward the SMS message.

2ch-b. The CHF opens CDR for this SMS submission.

2ch-c. The CHF acknowledges by sending Charging Data Response [Initial] to the SMSF

3. 	The SMSF

forward SMS or SMS deliver per applicable flows.

4.	"SMS to deliver"(i.e. Submit report, SMS Deliver answer) received by SMSF: see applicable flows.

4ch-a. The SMSF sends Charging Data Request [Termination] to the CHF for terminating the charging associated with SMS submission.

4ch-b. The CHF closes the CDR for this SMS submission.

4ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the SMSF.

5.

Termination procedures: see applicable flows.

The correspondence between "SMS Submit/Forward SM" and "SMS to deliver/SMS Deliver" messages in this scenario, and the messages in the different network scenario for which they are applicable, is described in table 5.4.2.2.1 and table 5.4.2.3.1 respectively.

End of changes