3GPP TSG-SA5 Meeting #128	S5-197717

# Zhuhai, China, 18th Nov 2019 - 22nd Nov 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0070 | rev | 3 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | YANG_Netconf CM Notifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson España S.A. Telecom Italia Orange China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNRM |  |  |  |  |  | Date: |  |  | 2019-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An NRM solution set includes a modeling methodology, a set of models, a way to manipulate these models (operations) and a way to notify interested parties about MOI/Attribute changes. The YANG/Netconf solution set needs a mapping for the the notifications : notifyMOICreation, notifyMOIDeletion, notifyMOIAttributeValueChanges. A notification mapping is proposed using the principles and methodology followed by Netconf and YANG, based on RFC 8641. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Netconf/YANG SS for notifyMOICreation, notifyMOIDeletion, notifyMOIAttributeValueChanges. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A notification solution that has a more complicated mapping to the YANG NRM. A mixed operation style for the YANG solution set: CRUD operations based on Netconf while CM notifications based on some other paradigm. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.x.2, 12.1.x.2.1 , 12.1.x.2.2 , 12.1.x.2.3 , 12.1.x.2.4 , 12.1.x.2.5 , 12.1.x.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The embedded YANG file at the end will be removed, it is only included for convinience during the review period of the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-197236 |  |  |  |  |  |  |  |  |

1st Change

# 2	References

-	The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.526: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[3]	3GPP TS 28.541: "Management and orchestration ; 5G Network Resource Model (NRM); Stage 2 and stage3".

[4]	ITU-T Recommendation X.733 (02/92): "Information technology - Open Systems Interconnection - Systems Management: Alarm reporting function".

[5]	3GPP TS 28.531: "Management and orchestration ; Provisioning; ".

[6]	3GPP TS 28.554: "Management and orchestration ; 5G end to end Key Performance Indicators (KPI)".

[7]	3GPP TS 22.261: "Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1".

[8]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects; System Architecture for the 5G System; Stage 2".

[9]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[10]	ETSI GS NFV-IFA 013 V2.4.1 (2018-02) "Network Function Virtualization (NFV); Management and Orchestration; Os-Ma-nfvo Reference Point - Interface and Information Model Specification".

[11]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[12]	ETSI GS NFV-IFA 015 (V2.4.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[13]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[14]	ITU-T Recommendation X.734 (1992): "Information technology - Open Systems Interconnection - Systems management: Event report management function".

[15]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[16]	3GPP TS 32.302:	 "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[18]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[19]	3GPP TS 32.401: "Telecommunication management; Perfomance Measurement (PM); Concept and requirements".

[20]	ISO 8601:2004: "Data elements and interchange formats – Information interchange – Representation of dates and times".

[21]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text: https://docs.onap.org/en/latest/_downloads/2c2b5962df52a0c1f2862f3bba3d67c7/CommonEventFormat_30.1_ONAP.json, accessed 21.03.2019.

[22]	Figure Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the figure: https://docs.onap.org/en/latest/submodules/vnfsdk/model.git/docs/files/ves7_1spec.html?highlight=heartbeatIntervalChange#resource-structure, accessed 21.03.2019).

[23]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text:	https://docs.onap.org/en/latest/submodules/vnfsdk/model.git/docs/files/VESEventListener_7_0_1.html?highlight=ves%207#naming-standards-for-eventname, accessed 11.04.2019).

[24]	Text Attribution: Creator: ONAP, under Creative Commons Attribution 4.0 International License, https://creativecommons.org/licenses/by/4.0/, URI to access the text:	https://docs.onap.org/en/latest/submodules/vnfsdk/model.git/docs/files/VESEventListener_7_0_1.html?highlight=ves%207#datatype-commoneventheader, accessed 11.04.2019).

[25]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects ".

[26]	W3C REC-xmlschema-0-20010502: "XML Schema Part 0: Primer".

[27]	W3C REC-xmlschema-1-20010502: "XML Schema Part 1: Structures".

[28]	W3C REC-xmlschema-2-20010502: "XML Schema Part 2: Datatypes".

[29]	W3C REC-xml-names-19990114: "Namespaces in XML".

[x1]	RFC 8639 "Subscription to YANG Notifications"

[x2]	RFC 8641 "Subscription to YANG Notifications for Datastore Updates"

[x3]	RFC 7951 "JSON Encoding of Data Modeled with YANG"

2nd Change

#### 12.1.x.2	Mapping of notifications

##### 12.1.x.2.1	Introduction

Notifications are implemented as a YANG “ietf-yang-push:push-change-update” notification as defined by RFC 8641 using the on-change feature. "ietf-yang-push:push-change-update"is augmented by the module "_3gpp-common-https-notif" with additional data items.

The notifications shall be carried in a RestFul manner.

- Notifications shall be carried over the HTTP protocol over TLS using the POST method

- HTTP result codes shall be used

- Notifications shall use a URI to address the receiver/consumer.

- Notification data shall be encoded as JSON data according to RFC 7951.

- Outgoing notifications will be subject to access control as specified in RFC8641 section “Receiver Authorization”. The user-id used by Network Configuration Access Control Model [RFC8341], is that of the receiver and is derived from the certificate presented by the receiver.

##### 12.1.x.2.2	Configuring notification subscriptions

Notifications are emited by the producer when a subscription is configured. If the subscription is removed notifications are no longer sent for that subscription.

The data models used for configuring the subscription are the ietf-subscribed-notifications.yang with support for the configured, encode-json and subtree or xpath features and the _3gpp-common-https-notif.yang. The configuration subtree identified by ietf-subscribed-notifications:/subscriptions/subscription/id shall be configured. Subscription filters can also be specified in the /filters/selection-filter subtree.

##### 12.1.x.2.3	Notification notifyMOICreation

This IS notification notifies the subscribed consumers that a new Managed Object Instance has been created. Creating a MOI in the Yang/Netconf Solution set means creating the list entry representing the MOI including creating the YANG data nodes representing its attributes.

The IS notification parameters are mapped to SS equivalents according to table 12.1.x.2.3-1.

Table 12.1.X.2.3-1: Mapping of IS notifyMOICreation parameters to SS equivalents

| IS notification parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| objectClass | Part of the/push-change-update/datastore-changes/yang-patch/edit/target element | M |  |
| objectInstance | Identified by /push-change-update/datastore-changes/yang-patch/edit/target element | M |  |
| notificationId | /push-change-update/datastore-changes/yang-patch/patch-id | M |  |
| notificationType | Represented by an element under <notification> element | M |  |
| systemDN | /push-change-update/systemDN | M |  |
| correlatedNotifications | /push-change-update/correlatedNotifications | CM |  |
| attributeList | Contained inside the element /push-change-update/datastore-changes/yang-patch/edit/value | O |  |
| additionalText | /push-change-update/additionalText | O |  |

##### 12.1.x.2.4	Notification notifyMOIDeletion

This IS notification notifies the subscribed consumers that an existing Managed Object Instance has been deleted.  Deleting a MOI in the Yang/Netconf Solution set means deleting the list entry representing the MOI.

The IS notification parameters are mapped to SS equivalents according to table 12.1.x.2.4-1.

Table 12.1.X.2.4-1: Mapping of IS notifyMOIDeletion parameters to SS equivalents

| IS notification parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| objectClass | Part of the/push-change-update/datastore-changes/yang-patch/edit/target element | M |  |
| objectInstance | Identified by /push-change-update/datastore-changes/yang-patch/edit/target element | M |  |
| notificationId | /push-change-update/datastore-changes/yang-patch/patch-id | M |  |
| notificationType | Represented by an element under <notification> element | M |  |
| eventTime | /Notification/eventTime | M |  |
| systemDN | /push-change-update/systemDN | M |  |
| correlatedNotifications | /push-change-update/correlatedNotifications | CM |  |
| attributeList | Not supported, not needed | O |  |
| additionalText | /push-change-update/additionalText | O |  |

##### 12.1.x.2.5	Notification notifyMOIAttributeValueChange

This IS notification notifies the subscribed consumers about changes of one or several attributes of a Managed Object Instance in the NRM.

The IS notification parameters are mapped to SS equivalents according to table 12.1.x.2.5-1.

Table 12.1.X.2.5-1: Mapping of IS notifyMOIAttributeValueChange parameters to SS equivalents

| IS notification parameter name | SS parameter name | SQ | Remark |
| --- | --- | --- | --- |
| objectClass | Part of the/push-change-update/datastore-changes/yang-patch/edit/target element | M |  |
| objectInstance | Identified by /push-change-update/datastore-changes/yang-patch/edit/target element | M |  |
| notificationId | /push-change-update/datastore-changes/yang-patch/patch-id | M |  |
| notificationType | Represented by an element under <notification> element | M |  |
| eventTime | /Notification/eventTime | M |  |
| systemDN | /push-change-update/systemDN | M |  |
| correlatedNotifications | /push-change-update/correlatedNotifications | CM |  |
| attributeValueChange | Contained inside the element /push-change-update/datastore-changes/yang-patch/edit/value | M |  |
| additionalText | /push-change-update/additionalText | O |  |

##### 12.1.x.2.6	_3gpp-common-https-notif@2019-10-11.yang

module _3gpp-common-https-notif {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-common-https-client;

prefix hnot3gpp;

import ietf-inet-types { prefix inet; }

import ietf-subscribed-notifications { prefix sn; }

import ietf-yang-push { prefix yp; }

import _3gpp-common-yang-types { prefix  types3gpp ; }

organization "3gpp SA5";

description "The model defines a extensions to IETF subscribed nodification

(RFC8639) and IETF YANG-Push (RFC8641) for realizing 3GPP https notifications";

reference "3GPP TS 28.532 Generic management services";

revision 2019-10-11 {

description "Initial revision";

}

identity https-3gpp {

base sn:transport;

base sn:configurable-encoding;

description

"3GPP HTTPS transport for notifications.";

}

augment "/sn:subscriptions/sn:subscription/sn:receivers/sn:receiver" {

description "Augment the subscriptions container to define the receiver.";

leaf uri-for-https-notifications {

when derived-from-or-self(../../../sn:transport,"hnot3gpp:https-3gpp") ;

type inet:uri;

description "The presence of this URI indicates that the

notifications for this subscription SHALL be sent over

HTTPS as specified in 3GPP TS 28.532 to the receiver.

A uniform resource locator that SHALL identify the file

transfer protocol, destination path.

E.g.

https://myserver.mydomain.com/cmChangeNotifications ";

}

}

augment "/yp:push-change-update" {

description "Augment the notification with 3GPP specific data.";

leaf additionalText {

type string ;

description "It can contain further information in text on the

event of the ManagedEntity(s).";

}

leaf systemDN {

type types3gpp:DistinguishedName ;

description "It shall carry the DN of management service providers.";

}

list correlatedNotifications {

leaf source {

type types3gpp:DistinguishedName ;

description "The entity emiting the notification as used in

/push-change-update/systemDN." ;

}

leaf-list notificationId {

type string;

description "Id of a correlated notification as used in

/push-change-update/datastore-changes/yang-patch/patch-id." ;

}

}

}

}

End of Change