3GPP TS 28.622 V16.2.0 (2019-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

Generic, NRM, IRP, Converged Management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	9

4	Model	10

4.1	Imported information entities and local labels	10

4.2	Class diagrams	10

4.2.1	Relationships	10

4.2.2	Inheritance	14

4.3	Class definitions	15

4.3.1	Any	15

4.3.1.1	Definition	15

4.3.1.2	Attributes	15

4.3.1.3	Attribute constraints	15

4.3.1.4	Notifications	15

4.3.2	IRPAgent	15

4.3.2.1	Definition	15

4.3.2.2	Attributes	16

4.3.2.3	Attribute constraints	16

4.3.2.4	Notifications	16

4.3.3	ManagedElement	16

4.3.3.1	Definition	16

4.3.3.2	Attributes	16

4.3.3.3	Attribute constraints	17

4.3.3.4	Notifications	17

4.3.4	ManagedFunction	17

4.3.4.1	Definition	17

4.3.4.2	Attributes	17

4.3.4.3	Attribute constraints	17

4.3.4.4	Notifications	17

4.3.5	ManagementNode	18

4.3.5.1	Definition	18

4.3.5.2	Attributes	18

4.3.5.3	Attribute constraints	18

4.3.5.4	Notifications	18

4.3.6	MeContext	18

4.3.6.1	Definition	18

4.3.6.2	Attributes	18

4.3.6.3	Attribute constraints	19

4.3.6.4	Notifications	19

4.3.7	SubNetwork	19

4.3.7.1	Definition	19

4.3.7.2	Attributes	19

4.3.7.3	Attribute constraints	19

4.3.7.4	Notifications	19

4.3.8	Top	20

4.3.8.1	Definition	20

4.3.8.2	Attributes	20

4.3.8.3	Attribute constraints	20

4.3.8.4	Notifications	20

4.3.9	VsDataContainer	20

4.3.9.1	Definition	20

4.3.9.2	Attributes	20

4.3.9.3	Attribute constraints	20

4.3.9.4	Notifications	20

4.3.10	Link	20

4.3.10.1	Definition	20

4.3.10.2	Attributes	21

4.3.10.3	Attribute constraints	21

4.3.10.4	Notifications	21

4.3.11	EP_RP	21

4.3.11.1	Definition	21

4.3.11.2	Attributes	21

4.3.11.3	Attribute constraints	22

4.3.11.4	Notifications	22

4.3.12	MeasurementControl	22

4.3.12.1	Definition	22

4.3.12.2	Attributes	22

4.3.12.3	Attribute constraints	22

4.3.12.4	Notifications	22

4.3.13	MeasurementReader	22

4.3.13.1	Definition	22

4.3.13.2	Attributes	23

4.3.13.3	Attribute constraints	23

4.3.13.4	Notifications	23

4.3.14	Measurements <<dataType>>	23

4.3.14.1	Definition	23

4.3.14.2	Attributes	24

4.3.14.3	Attribute constraints	24

4.3.14.4	Notifications	24

4.3.15	ThresholdMonitoringCapability	24

4.3.15.1	Definition	24

4.3.15.2	Attributes	24

4.3.15.3	Attribute constraints	24

4.3.15.4	Notifications	24

4.3.16	ThresholdMonitor	24

4.3.16.1	Definition	24

4.3.16.2	Attributes	25

4.3.16.3	Attribute constraints	25

4.3.16.4	Notifications	25

4.3.17	ManagedNFService	25

4.3.17.1	Definition	25

4.3.17.2	Attributes	25

4.3.17.3	Attribute constraints	25

4.3.17.4	Notifications	26

4.3.18	Operation <<dataType>>	26

4.3.18.1	Definition	26

4.3.18.2	Attributes	26

4.3.18.3	Attribute constraints	26

4.3.18.4	Notifications	26

4.3.19	SAP <<dataType>>	26

4.3.19.1	Definition	26

4.3.19.2	Attributes	26

4.3.19.3	Attribute constraints	26

4.3.19.4	Notifications	26

4.3.20	ManagedEntity <<ProxyClass>>	26

4.3.20.1	Definition	26

4.3.20.2	Attributes	27

4.3.20.3	Attribute constraints	27

4.3.20.4	Notifications	27

4.3.21	HeartbeatControl	27

4.3.21.1	Definition	27

4.3.21.2	Attributes	27

4.3.21.3	Attribute constraints	27

4.3.21.4	Notifications	27

4.3.22	NtfSubscriptionControl	28

4.3.22.1	Definition	28

4.3.22.2	Attributes	28

4.3.22.3	Attribute constraints	28

4.3.22.4	Notifications	28

4.3.23	Scope <<dataType>>	28

4.3.23.1	Definition	28

4.3.23.2	Attributes	29

4.3.23.3	Attribute constraints	29

4.3.23.4	Notifications	29

4.4	Attribute definitions	30

4.4.1	Attribute properties	30

4.4.2	Constraints	42

4.5	Common notifications	42

4.5.1	Alarm notifications	42

4.5.2	Configuration notifications	42

Annex A (informative):	Alternate class diagram	44

Annex B (informative):	Change history	45

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

28.621	Generic Network Resource Model (NRM) Integration Reference Point (IRP); Requirements;

28.622	Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ;

28.623	Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions.

The interface Itf-N, defined in 3GPP TS 32.102 [2], is built up by a number of Integration Reference Points (IRPs) and a related Name Convention, which realise the functional capabilities over this interface. The basic structure of the IRPs is defined in 3GPP TS 32.150 [4].

The present document is part of a set that has been developed for converged management solutions.

The present document is part of a set that is used for management and orchestration of 5G networks and network slicing.

# 1	Scope

The present document specifies the Generic network resource information that can be communicated between an IRPAgent and an IRPManager for telecommunication network management purposes, including management of converged networks and networks that include virtualized network functions.

This document specifies the semantics of information object class attributes and relations visible across the reference point in a protocol and technology neutral way.  It does not define their syntax and encoding.

This document supports the Federated Network Information Model (FNIM) concept described in [8] in that the relevant Information Object Class (IOC)s defined in this specification are directly or indirectly inherited from those specified in the Umbrella Information Model (UIM) of [9].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[4]	3GPP TS 32.150: "Telecommunication management; Integration Reference Point (IRP) Concept and Definitions".

[5]	3GPP TS 23.003: "Technical Specification Group Core Network and Terminals; Numbering, addressing and identification"

[6]	3GPP TS 32.532: " Telecommunication management; Software Management Integration Reference Point (IRP); Information Service (IS) "

[7]	ITU-T Recommendation X.710 (1991): "Common Management Information Service Definition for CCITT Applications".

[8]	TS 32.107: "Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM)"

[9]	TS 28.620: "Telecommunication management; Fixed Mobile Convergence (FMC) Federated Network Information Model (FNIM) Umbrella Information Model (UIM)"

[10]	TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire"

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM Information Service (IS)".

[13]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[14]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[15]	ETSI GS NFV 003 V1.1.1: "Network Functions Virtualisation (NFV); Terminology for Main Concepts in NFV".

[16] 	ETSI GS NFV-IFA 008 v2.1.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[17]	ETSI GS NFV-IFA 015 v2.1.2: "Network Functions Virtualisation (NFV); Management and Orchestration; Report on NFV Information Model".

[18]	ETSI ES 202 336-12 V1.1.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[19]	ITU-T Recommendation X.731: "Information technology - Open Systems Interconnection - Systems Management: State management function".

[20]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[21]	3GPP TS 28.625: "State Management Data Definition Integration Reference Point (IRP); Information Service (IS) ".

[22]	3GPP TS 23.501: "System Architecture for the 5G System".

[23]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[24]	IETF RFC 791: "Internet Protocol".

[25]	IETF RFC 2373: "IP Version 6 Addressing Architecture".

[26]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply. For terms and definitions not found here, please refer to 3GPP TS 32.101 [1], 3GPP TS 32.102 [2], 3GPP TS 32.150 [4] and 3GPP TS 32.600 [14].

Association: In general, it is used to model relationships between Managed Objects. Associations can be implemented in several ways, such as:

1)	name bindings,

2)	reference attributes, and

3)	association objects.

This IRP stipulates that name containment associations shall be expressed through name bindings, but it does not stipulate the implementation for other types of associations as a general rule. These are specified as separate entities in the object models (UML diagrams). Currently however, all (non-containment) associations are modelled by means of reference attributes of the participating MOs.

Information Object Class (IOC): An IOC represents the management aspect of a network resource. It describes the information that can be passed/used in management interfaces. Their representations are technology agnostic software objects. IOC has attributes that represents the various properties of the class of objects. See the term "attribute" defined in [10]. Furthermore, IOC can support operations providing network management services invocable on demand for that class of objects. An IOC may support notifications that report event occurrences relevant for that class of objects. It is modelled using the stereotype "Class" in the UML meta-model. See TS 32.156 [10] for additional information on IOC.

Managed Object (MO): A MO is an instance of a Managed Object Class (MOC) representing the management aspects of a network resource. Its representation is a technology specific software object. It is sometimes called MO instance (MOI). The MOC is a class of such technology specific software objects. An MOC is the same as an IOC except that the former is defined in technology specific terms and the latter is defined in technology agnostic terms. MOCs are used/defined in SS level specifications. IOCs are used/defined in IS level specifications.

Management Information Base (MIB): A MIB is an instance of an NRM and has some values on the defined attributes and associations specific for that instance. In the context of the present document, an MIB consists of:

1)	a Name space (describing the MO containment hierarchy in the MIB through Distinguished Names),

2)	a number of Managed Objects with their attributes and

3)	a number of Associations between these MOs. Also note that TMN (ITU-T Recommendation X.710 [7]) defines a concept of a Management Information Tree (also known as a Naming Tree) that corresponds to the name space (containment hierarchy) portion of this MIB definition. Figure 3.1 depicts the relationships between a Name space and a number of participating MOs (the shown association is of a non-containment type)

Figure 3.1: Relationships between a Name space and a number of participating MOs

Name space: A name space is a collection of names. The IRP name convention (see 3GPP TS 32.300 [13]) restricts the name space to a hierarchical containment structure, including its simplest form - the one-level, flat name space. 
All Managed Objects in a MIB are included in the corresponding name space and the MIB/name space shall only support a strict hierarchical containment structure (with one root object). A Managed Object that contains another is said to be the superior (parent); the contained Managed Object is referred to as the subordinate (child). The parent of all MOs in a single name space is called a Local Root. The ultimate parent of all MOs of all managed systems is called the Global Root.

Network resource:  discrete entity represented by an Information Object Class (IOC) for the purpose of network and service management.

NOTE:	A network resource may represent intelligence, information, hardware and software of a telecommunication network.

Network Resource Model (NRM): A collection of IOCs, inclusive of their associations, attributes and operations, representing a set of network resources under management.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [26] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [26].

DN	Distinguished Name (see 3GPP TS 32.300 [13])

IOC 	Information Object Class

MO	Managed Object

MOC	Managed Object Class

MOI	Managed Object Instance

NFVI	Network Functions Virtualisation Infrastructure (NFVI): Defined in ETSI GS NFV 003 [15].

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [13])

SS	Solution Set

VNF	Virtualised Network Function

# 4	Model

## 4.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 32.111-2 [11], notification, notifyAckStateChanged | notifyAckStateChanged |
| 3GPP TS 32.662 [12], notification, notifyAttributeValueChanged | notifyAttributeValueChanged |
| 3GPP TS 32.111-2 [11], notification, notifyChangedAlarm | notifyChangedAlarm |
| 3GPP TS 32.111-2 [11], notification, notifyClearedAlarm | notifyClearedAlarm |
| 3GPP TS 32.111-2 [11], notification, notifyComments | notifyComments |
| 3GPP TS 32.111-2 [11], notification, notifyNewAlarm | notifyNewAlarm |
| 3GPP TS 32.662 [12], notification, notifyObjectCreation | notifyObjectCreation |
| 3GPP TS 32.662 [12], notification, notifyObjectDeletion | notifyObjectDeletion |
| 3GPP TS 32.111-2 [11], notification, notifyAlarmListRebuilt | notifyAlarmListRebuilt |
| 3GPP TS 32.111-2 [11], notification, notifyPotentialFaultyAlarmList | notifyPotentialFaultyAlarmList |
| 3GPP TS 32.532 [6], notification, notifyDownloadNESwStatusChanged | notifyDownloadNESwStatusChanged |
| 3GPP TS 32.532 [6], notification, notifyInstallNESwStatusChanged | notifyInstallNESwStatusChanged |
| 3GPP TS 32.532 [6], notification, notifyActivateNESwStatusChanged | notifyActivateNESwStatusChanged |
| 3GPP TS 28.620 [9], IOC, Domain_ | Domain_ |
| 3GPP TS 28.620 [9], IOC, ManagedElement_ | ManagedElement_ |
| 3GPP TS 28.620 [9], IOC, Function_ | Function_ |
| 3GPP TS 28.620 [9], IOC, ManagementSystem_ | ManagementSystem_ |
| 3GPP TS 28.620 [9], IOC, TopologicalLink_ | TopologicalLink_ |
| 3GPP TS 28.620 [9], IOC, Top_ | Top_ |

## 4.2	Class diagrams

### 4.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for this IRP. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

The following figure shows the containment/naming hierarchy and the associations of the classes defined in the present document. See Annex A of a class diagram that combines this figure with Figure 1 of [2], the class diagram of UIM.

NOTE 1:	ManagedElement may be contained either

-	in a SubNetwork (since SubNetwork inherits from Domain_ and ManagedElement inherits from ManagedElement_ and Domain_ name-contained ManagedElement_ as observed in the figure of Annex A) or

-	in a MeContext instance as observed by the above figure or in the figure of Annex A.

This either-or relation cannot be shown by using an {xor} constraint in the above figure.

ManagedElement may also have no parent instance at all.

NOTE 2:	Each instance of the VsDataContainer shall only be contained under one IOC. The VsDataContainer can be contained under IOCs defined in other NRMs.

NOTE 3:	If the configuration contains several instances of SubNetwork, exactly one SubNetwork instance shall directly or indirectly contain all the other SubNetwork instances.

NOTE 4:	The SubNetwork instance not contained in any other instance of SubNetwork is referred to as "the root SubNetwork instance".

NOTE 5:	ManagementNode shall be contained in the root SubNetwork instance.

NOTE 6:	If contained in a SubNetwork instance, IRPAgent shall be contained in the root SubNetwork instance.

NOTE 7:	For a clarification on the choice of containment of the IRPAgent (since it has three possible parents), see the def. of IRPAgent.

NOTE 8:    Cardinality * is identical to multiplicity 0..*.

- Figure 4.2.1-1: Containment/Naming and Association NRM fragment

Each Managed Object is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [13] that expresses its containment hierarchy. As an example, the DN of a ManagedElement instance could have a format like:

SubNetwork=Sweden,MeContext =MEC-Gbg-1, ManagedElement=RNC-Gbg-1.

NOTE 8:	Each instance of the VsDataContainer shall only be contained under one IOC. The VsDataContainer can be contained under IOCs defined in other NRMs by virtue of inheritance from the GENERIC NRM.

NOTE 9:	The VsDataContainer is only used for the Bulk CM IRP.

Figure 4.2.1-2: VsDataContainer Containment/Naming and Association in  NRM fragment

Figure 4.2.1-3: Performance measurement control NRM fragment

Figure 4.2.1-4: Measurement threshold monitoring control fragment

Figure 4.2.1-5: Heartbeat notification control fragment

Figure 4.2.1-6: Notification subscription control fragment

### 4.2.2	Inheritance

- This clause depicts the inheritance relationships.

- Figure 4.2.2-1: Inheritance Hierarchy NRM fragment

- Figure 4.2.2-2: Performance measurement control NRM fragment

- Figure 4.2.2-3: Measurement threshold monitoring NRM fragment

- Figure 4.2.2-4: Heartbeat notification control fragment

Figure 4.2.2-5: Notification subscription fragment

## 4.3	Class definitions

### 4.3.1	Any

#### 4.3.1.1	Definition

This class represents the classes (e.g. IOC) that are not defined in this specification but are or will be defined in other IRP specification(s).

#### 4.3.1.2	Attributes

None

#### 4.3.1.3	Attribute constraints

None

#### 4.3.1.4	Notifications

This class does not support any notification.

### 4.3.2	IRPAgent

#### 4.3.2.1	Definition

This IOC represents the functionality of an IRPAgent. It shall be present. For a definition of IRPAgent, see 3GPP TS 32.102 [2].

The IRPAgent will be contained under an IOC as follows (only one of the options shall be used):

1)	ManagementNode, if the configuration contains a ManagementNode;

2)	SubNetwork, if the configuration contains a SubNetwork and no ManagementNode;

3)	ManagedElement, if the configuration contains no ManagementNode or SubNetwork.

#### 4.3.2.2	Attributes

The IRPAgent IOC includes the attributes inherited from Top_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| systemDN | M | T | F | F | T |

#### 4.3.2.3	Attribute constraints

None

#### 4.3.2.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.3	ManagedElement

#### 4.3.3.1	Definition

This IOC represents telecommunications equipment or TMN entities within the telecommunications network providing support and/or service to the subscriber. 
An ME communicates with a manager (directly or indirectly) over one or more management interfaces for the purpose of being monitored and/or controlled. MEs may or may not additionally perform element management functionality. 
An ME contains equipment that may or may not be geographically distributed. An ME is often referred to as a "Network Element".

A telecommunication equipment has software and hardware components. The IOC described above represents the case when the software component is designed to run on dedicated hardware component. In the case when the software is designed to run on ETSI NFV defined NFVI [15], the IOC description would exclude the NFVI component supporting the above mentioned subject software. A ManagedElement may be contained in either a SubNetwork or in a MeContext instance. A single ManagedElement may also exist stand-alone with no parent at all.

The ManagedElement IOC may be used to represent combined ME functionality (as indicated by the managedElementType attribute and the contained instances of different functional IOCs).

Single function ManagedElement IOC instances will have a 1..1 containment relationship to a function IOC instance (in this context a function IOC instance is an instance of an IOC derived from the ManagedFunction IOC). Multiple function ManagedElement instances will have a 1..N containment relationship to function IOC instances.

NOTE:	For some specific functional IOCs a 1..N containment relationship is permitted.  The specific functional entities are identified in the NRMs that define subclasses of ManagedFunction.

#### 4.3.3.2	Attributes

The ManagedElement IOC includes the attributes inherited from ManagedElement_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| vendorName | M | T | F | F | T |
| userDefinedState | M | T | T | F | T |
| swVersion | M | T | F | F | T |
| priorityLabel | O | T | F | T | F |
| measurementsList | M | T | F | F | T |

#### 4.3.3.3	Attribute constraints

Attribute constrains for dnPrefix: The attribute dnPrefix shall be supported if an instance of ManagedElement is the local root instance of the MIB. Otherwise the attribute shall be absent or carry no information.

#### 4.3.3.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC. In addition, the following set of notifications is also valid.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyDownloadNESwStatusChanged | See Software Management IRP (3GPP TS 32.532 [6]) |  |
| notifyInstallNESwStatusChanged | See Software Management IRP (3GPP TS 32.532 [6]) |  |
| notifyActivateNESwStatusChanged | See Software Management IRP (3GPP TS 32.532 [6]) |  |

### 4.3.4	ManagedFunction

#### 4.3.4.1	Definition

This IOC is provided for sub-classing only. It provides attribute(s) that are common to functional IOCs. Note that a ManagedElement may contain several managed functions. The ManagedFunction may be extended in the future if more common characteristics to functional objects are identified.

This IOC can represent a telecommunication function either realized by software running on dedicated hardware or realized by software running on NFVI. Each ManagedFunction instance communicates with a manager (directly or indirectly) over one or more management interfaces exposed via its containing ME instance.

#### 4.3.4.2	Attributes

The ManagedFunction IOC includes the attributes inherited from Function_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| vnfParametersList | CM | T | T | F | T |
| peeParametersList | CM | T | T | F | T |
| priorityLabel | O | T | F | T | F |
| measurementsList | M | T | F | F | T |

#### 4.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| vnfParametersList Support Qualifier | Condition: The ManagedFunction instance is realized by one or more VNF instance(s). Otherwise this attribute shall be absent. |
| peeParametersList Support Qualifier | Condition: The control and monitoring of PEE parameters is supported by the ManagedFunction or sub-class instance. |

#### 4.3.4.4	Notifications

There is no notification defined.

### 4.3.5	ManagementNode

#### 4.3.5.1	Definition

This IOC represents a telecommunications management system (EM) within the TMN that contains functionality for managing a number of ManagedElements (MEs). The management system communicates with the MEs directly or indirectly over one or more interfaces for the purpose of monitoring and/or controlling these MEs.

This class has similar characteristics as the ManagedElement. The main difference between these two classes is that the ManagementNode has a special association to the managed elements that it is responsible for managing.

#### 4.3.5.2	Attributes

The ManagementNode IOC includes the attributes inherited from ManagementSystem_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| vendorName | M | T | F | F | T |
| userDefinedState | M | T | T | F | T |
| locationName | M | T | F | F | T |
| swVersion | M | T | F | F | T |

#### 4.3.5.3	Attribute constraints

None

#### 4.3.5.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.6	MeContext

#### 4.3.6.1	Definition

This IOC is introduced for naming purposes. It may support creation of unique DNs in scenarios when some MEs have the same RDNs due to the fact that they have been manufacturer pre-configured. 
If some MEs have the same RDNs (for the above mentioned reason) and they are contained in the same SubNetwork instance, some measure shall be taken in order to assure the global uniqueness of DNs for all IOC instances under those MEs. One way could be to set different dnPrefix for those NEs, but that would require either that:

a)	all LDNs or DNs are locally modified using the new dnPrefix for the upper portion of the DNs, or

b)	a mapping (translation) of the old LDNs or DNs to the new DNs every time they are used externally, e.g. in alarm notifications.

As both the two alternatives above may involve unacceptable drawbacks (as the old RDNs for the MEs then would have to be changed or mapped to new values), using MeContext offers a new alternative to resolve the DN creation. Using MeContext as part of the naming tree (and thus the DN) means that the dnPrefix, including a unique MeContext for each ME, may be directly concatenated with the LDNs, without any need to change or map the existing ME RDNs to new values.

MeContext have 0..N instances. It may exist even if no SubNetwork exists. Every instance of MeContext contains exactly one ManagedElement during steady-state operations.

#### 4.3.6.2	Attributes

The MeContext IOC includes the attributes inherited from Top_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dnPrefix | CM | T | F | F | T |

#### 4.3.6.3	Attribute constraints

| Name | Definition |
| --- | --- |
| dnPrefix Support Qualifier | Condition: The instance of MeContext is the local root instance of the MIB. Otherwise the attribute shall be absent or carry no information. |

#### 4.3.6.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.7	SubNetwork

#### 4.3.7.1	Definition

This IOC represents a set of managed entities.There may be zero or more instances of a SubNetwork. It shall be present if either a ManagementNode or multiple ManagedElements are present (i.e. ManagementNode and multiple ManagedElement instances shall have SubNetwork as parent).

The SubNetwork instance not contained in any other instance of SubNetwork is referred to as "the root SubNetwork instance".

#### 4.3.7.2	Attributes

The SubNetwork IOC includes the attributes inherited from Domain_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| setOfMcc | CM | T | F | F | T |
| priorityLabel | O | T | F | T | F |
| measurementsList | M | T | F | F | T |

#### 4.3.7.3	Attribute constraints

| Name | Definition |
| --- | --- |
| dnPrefix (inherited from Domain_) Support Qualifier | Condition: The instance of SubNetwork is the local root instance of the MIB. Otherwise the attribute shall be absent or carry no information. |
| setOfMcc Support Qualifier | Condition: There is more than one value in setOfMcc of the SubNetwork ; otherwise the support is optional. |

#### 4.3.7.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.8	Top

#### 4.3.8.1	Definition

This IOC is provided for sub-classing only. All information object classes defined in all TS that claim to be conformant to 32.102 [2] shall inherit from Top.

#### 4.3.8.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| objectClass | M | T | F | T | T |
| objectInstance | M | T | F | T | T |

#### 4.3.8.3	Attribute constraints

None

#### 4.3.8.4	Notifications

There is no notification defined.

### 4.3.9	VsDataContainer

#### 4.3.9.1	Definition

The VsDataContainer managed object is a container for vendor specific data. The number of instances of the VsDataContainer can differ from vendor to vendor. This IOC shall only be used by the Bulk CM IRP for all the NRMs.

#### 4.3.9.2	Attributes

The VsDataContainer IOC includes the attributes inherited from Top_ IOC (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| vsDataType | M | T | F | F | O |
| vsData | M | T | O | F | O |
| vsDataFormatVersion | M | T | F | F | O |

#### 4.3.9.3	Attribute constraints

None

#### 4.3.9.4	Notifications

Support for notification on the change of attribute value is vendor-specific.

### 4.3.10	Link

#### 4.3.10.1	Definition

This IOC is provided for sub-classing only. This IOC represents a communication link or reference point between two network entities. The Link IOC does not indicate whether the represented communication link or reference point is a physical or logical entity.

For the subclasses of Link, the following rules apply:

1)	The subclass names shall have the form “Link_<X>_<Y>”, where <X> is a string that represents the IOC at one end of the association related to the particular Link subclass, and <Y> is a string that represents the IOC at the other end of the association. For the order of the two strings, <X> shall come alphabetically before <Y>.

2)	In case <X> and <Y> are YyyFunction IOCs (inheriting from ManagedFunction and on first level below ManagedElement), the <X> and <Y> strings shall have the same form as the legal values of the managedElementType attribute (see clause 4.5.1), e.g. “Auc”.  Otherwise <X> and <Y> shall be the full IOC names.

Thus, two valid examples of Link subclass names would be: Link_As_Cscf and Link_Mrfc_Mrfp.

#### 4.3.10.2	Attributes

The Link IOC includes the attributes inherited from TopologicalLink_ (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| userLabel | M | T | T | F | T |
| linkType | O | T | F | F | T |
| protocolVersion | O | T | F | F | T |

#### 4.3.10.3	Attribute constraints

| Name | Definition |
| --- | --- |
| aEnd and zEnd (inherited from TopologicalLink_) Support Qualifier | Condition: The property multiplicity is 1. |

#### 4.3.10.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.11	EP_RP

#### 4.3.11.1	Definition

This IOC is provided for sub-classing only. This IOC represents an end point of a link used across a reference point between two network entities.

The detailed subclassed IOC, e.g. EP_X2, will be defined in E-UTRAN NRM IRP, by inheriting from this EP_RP.

For naming the subclasses of EP_RP, the following rules shall apply:

-	The name of the subclassed IOC shall have the form “EP_<rp>”, where <rp> is a string that represents the name of the reference point.

Thus, two valid examples of EP_RP subclassed IOC names would be: EP_S1 and EP_X2.

#### 4.3.11.2	Attributes

The EP_RP IOC includes the attributes inherited from Top_ (defined in TS 28.620 [9]), attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| farEndEntity | O | T | - | - | T |
| userLabel | O | T | T | - | T |
| measurementsList | M | T | F | F | T |

#### 4.3.11.3	Attribute constraints

None

#### 4.3.11.4	Notifications

This class does not support any notification.

### 4.3.12	MeasurementControl

#### 4.3.12.1	Definition

This IOC captures the properties of the file-based and stream-based delivery methods.

The file-based delivery method has properties for the file location, the file reporting period and the file-based GP. The stream-based delivery method has properties for the stream target and the stream-based GP.

These properties are labelled as default (e.g. defaultFileBasedGP) in that they will be ignored in case the same properties captured in MeasurementReader are used.

Instance of this IOC shall not be created nor deleted by consumer. It shall be created and deleted by the producer.

Depending on particular deployment context and agreement between operator and vendor, the isWritable for all attributes could be ‘F’.

#### 4.3.12.2	Attributes

The MeasurementControl IOC includes attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pMAdministrativeState | M | T | T | F | T |
| pMOperationalState | M | T | F | F | T |
| defaultFileBasedGP | M | T | T | F | F |
| defaultFileReportingPeriod | M | T | T | F | F |
| defaultFileLocation | M | T | T | F | F |
| streamBasedGP | O | T | T | F | F |
| defaultStreamTarget | O | T | T | F | F |

#### 4.3.12.3	Attribute constraints

None.

#### 4.3.12.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.13	MeasurementReader

#### 4.3.13.1	Definition

This IOC identifies the entity (derivatives of ManagedFunction) whose Measurements are required by consumer to be produced and captured. The types of Measurements required are identified by the attribute measurementTypes.

This IOC captures the properties of the two delivery methods for delivering the Measurements.

The file-based delivery method has properties: fileLocation, reportingPeriod and fileBasedGP. The stream-based delivery method has properties: streamTarget and streamBasedGP.

The MeasurementControl also can capture the properties of the file-based and stream-based delivery methods.

If the MeasurementReader instance’s file-based delivery method has valid properties, the file-based delivery method is used and the MeasurementControl instance’s file-based delivery method is ignored

If the MeasurementReader instance’s stream-based delivery method has valid properties, the stream-based delivery method is used and the MeasurementControl instance’s stream-based delivery is ignored.

A file-based and stream-based delivery methods can be active at the same time for a MeasurementReader instance.

The activity of a MeasurementReader instance is independent of that of other MeasurementReader instances.

This IOC uses attributes managedObjectDNs or managedObjectDNsBasic to identify specific managed entities whose Measurements are required by consumer.

#### 4.3.13.2	Attributes

The MeasurementReader IOC includes attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| measurementTypes | M | T | T | F | F |
| fileBasedGP | O | T | T | F | F |
| fileReportingPeriod | O | T | T | F | F |
| fileLocation | O | T | T | F | F |
| streamBasedGP | O | T | T | F | F |
| streamTarget | O | T | T | F | F |
| managedObjectDNsBasic | CM | T | T | F | F |
| managedObjectDNs | CM | T | T | F | F |

#### 4.3.13.3	Attribute constraints

| Name | Definition |
| --- | --- |
| managedObjectDNsBasic, managedObjectDNs Support Qualifier | Condition: It shall be supported if the MeasurementReader is a subordinate of SubNetwork or NetworkSliceSubnet. If both attributes contain information, the information in managedObjectDNs is ignored. |

#### 4.3.13.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.14	Measurements <<dataType>>

#### 4.3.14.1	Definition

This <<dataType>> captures the Measurement types supported by its associated IOC, i.e. ManagedFunction or ManagedElement or EP_RP. It also captures the list of GPs supported, on per Measurement type. This <<datatype>> indicates a capability of the associated IOC and therefore its isWritable property is False and its isInvariant property is False (since vendor product upgrade can increase or decrease the number of supported Measurement types without the need to deleting the older-version instance and creating a newer-version instance.)

#### 4.3.14.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| measurementTypes | M | T | F | F | F |
| gPs | M | T | F | F | F |

#### 4.3.14.3	Attribute constraints

None

#### 4.3.14.4	Notifications

Not applicable.

### 4.3.15	ThresholdMonitoringCapability

#### 4.3.15.1	Definition

This IOC represents the capability of threshold monitoring(s) allowed to be created by <<IOC>>ThresholdMonitor to monitor some or all of the measurements identified by Measurements<<datatype>>.

One instance of this IOC can be name-contained by one instance of <<IOC>>SubNetwork or one instance of <<IOC>>ManagedElement or one instance of <<IOC>>ManagedFunction.

This IOC instance provides the capability of the threshold monitor(s) allowed to be created for the measurements of the (tree) nodes of a name-containment tree whose top (tree) node is the IOC instance name-containing the said <<IOC>>ThresholdMonitoringCapability instance. In case one instance (say A) of this IOC is contained by a tree node (say X), and another instance (say B) of this IOC is contained by a subordinate tree node (of tree node X), the instance (B) of this IOC contained by the subordinate tree node (Y) prevail.

Instance of this IOC shall not be created nor deleted by consumer. It shall be created and deleted by the producer.

#### 4.3.15.2	Attributes

The ThresholdMonitoringCapability IOC includes attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedMonitoringGPs | M | T | F | F | T |

#### 4.3.15.3	Attribute constraints

None.

#### 4.3.15.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.16	ThresholdMonitor

#### 4.3.16.1	Definition

This IOC identifies the threshold monitor that is created by the consumer for the monitored entities whose measurements are required by consumer to monitor. The monitored entities are identified by the attribute monitoredObjectDNs.

The MOI creation request for this IOC may be rejected, if

the measurements to be monitered are being collected (e.g., by measurement job or NRM configurations) with  the GP different from the monitoringGP; or

the measurements to be monitered are not being collected.

One instance of this IOC can be name-contained by one instance of <<IOC>>SubNetwork, one instance of <<IOC>>ManagedElement, one instance of <<IOC>>ManagedFunction or one instance of <<IOC>>EP_RP.

In case one instance (say A) of this IOC is contained by a tree node (say X), and another instance (say B) of this IOC is contained by a subordinate tree node (of tree node X), when these two instances have overlaps the instance (B) of this IOC contained by the subordinate tree node (Y) prevail for the overlapped parts.

#### 4.3.16.2	Attributes

The ThresholdMonitor IOC includes attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| thresholdInfoList | M | T | T | F | F |
| monitoringGP | M | T | T | F | F |
| monitoringNotifTarget | M | T | T | F | F |
| monitoredIOCName | M | T | T | F | F |
| monitoredObjectDNs | M | T | T | F | F |

#### 4.3.16.3	Attribute constraints

None.

#### 4.3.16.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.17	ManagedNFService

#### 4.3.17.1	Definition

A ManagedNFService represents a Network Function (NF) service as defined in clause 7 of 3GPP TS 23.501[22].

#### 4.3.17.2	Attributes

The ManagedNFService IOC includes attributes inherited from Top IOC (defined in clause 4.3.8) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| userLabel | O | T | T | F | T |
| nFServiceType | M | T | F | T | F |
| sAP | M | T | T | F | T |
| operations | M | T | T | F | T |
| administrativeState | M | T | T | F | T |
| operationalState | M | T | F | T | T |
| usageState | M | T | F | T | T |
| registrationState | CM | T | F | F | T |

#### 4.3.17.3	Attribute constraints

Attribute constraint for registrationState: The attribute registrationState should be supported by instance of a ManagedNFService if the service is designed for being publicshed and discovered by other NFs, and need to be registered to a repository function. E.g. Authentication service provided by AUSF should include this attribute. NF management services provided by NRF don’t include this attribute.

#### 4.3.17.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.18	Operation <<dataType>>

#### 4.3.18.1	Definition

This data type represents an Operation. An Operation is comprised of a name, an allowedNFType and an operationSemantics (See TS 23.502 [23]).

#### 4.3.18.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| name | M | T | F | T | F |
| allowedNFTypes | M | T | T | F | T |
| operationSemantics | M | T | F | T | T |

#### 4.3.18.3	Attribute constraints

None

#### 4.3.18.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.19	SAP <<dataType>>

#### 4.3.19.1	Definition

This data type represents the access point of a managed NF service which is comprised of a host and a port.

#### 4.3.19.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| host | M | T | T | F | T |
| port | M | T | T | F | T |

#### 4.3.19.3	Attribute constraints

None

#### 4.3.19.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

### 4.3.20	ManagedEntity <<ProxyClass>>

#### 4.3.20.1	Definition

This represents an <<IOC>>SubNetwork, <<IOC>>ManagedElement or <<IOC>>ManagedFunction.

#### 4.3.20.2	Attributes

See that defined in <<IOC>>SubNetwork, <<IOC>>ManagedElement or <<IOC>>ManagedFunction.

#### 4.3.20.3	Attribute constraints

See that defined in <<IOC>>SubNetwork, <<IOC>>ManagedElement or <<IOC>>ManagedFunction.

#### 4.3.20.4	Notifications

See respective IOCs.

### 4.3.21	HeartbeatControl

#### 4.3.21.1	Definition

MnS consumers (i.e. notification recipients) use heartbeat notifications to monitor the communication channels between them and data report MnS producers emitting notifications such as notifyNewAlarm and notifyFileReady.

A HeartbeatControl instance allows controlling the emission of heartbeat notifications by MnS producers. The recipients of heartbeat notifications are not specified by an attribute of the Heartbeat instance, but by an attribute of the IOC name-containing the HeartbeatControl IOC.

Note that the MnS consumer managing the HeartbeatControl instance and the MnS consumer receiving the heartbeat notifications may not be the same.

As a pre-condition for the emission of heartbeat notifications, a HeartbeatControl instance needs to be created. Creation of an instance with an initial non-zero value of the heartbeatNtfPeriod attribute triggers an immediate heartbeat notification emission. Creation of an instance with an initial zero value of the heartbeatPeriod attribute does not trigger an emission of a heartbeat notification. Deletion of this instance does not trigger an emission of a heartbeat notification.

Creation and deletion of HeartbeatControl instances by MnS Consumers is optional; when not supported, the HeartbeatControl instances may be created and deleted by the system or be pre-installed.

#### 4.3.21.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| heartbeatNtfPeriod | M | T | T | F | T |
| triggerHeartbeatNtf | M | F | T | F | F |

#### 4.3.21.3	Attribute constraints

None.

#### 4.3.21.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC. In addition, the following set of notifications is also valid.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyHeartbeat | M | -- |

### 4.3.22	NtfSubscriptionControl

#### 4.3.22.1	Definition

A NtfSubscriptionControl instance represents the notification subscription of a particular notification recipient.

The scope attribute is used to select managed object instances. The base object instance of the scope is the object instance name-containing the NtfSubscriptionControl instance. The notifications related to the selected managed object instances are candidates to be sent to the address specified by the notificationRecipientAddress attribute.

The notificationType attribute and notificationFilter attribute allow MnS consumers to exercise control over which candidate notifications are sent to the notificationRecipientAddress.

If the notificationType attribute is supported, its value identifies the types of notifications that are candidate to be sent to the notificationRecipientAddress. If the notificationType attribute is not supported, all types of notifications are candidate to be sent to notificationRecipientAddress.

If supported, the notificationFilter attribute defines a filter that is applied to the set of candidate notifications. Only candidate notifications that pass the filter criteria are sent to the notificationRecipientAddress. If the notificationFilter attribute is not supported all candidate notificatios are sent to the notificationRecipientAddress.

To receive notifications, a MnS consumer has to create NtfSubscriptionControl object instancess on the MnS producer. A MnS consumer can create a subscription for another MnS consumer since it is not required the notificationRecipientAddress be his own address.

When a MnS consumer does not wish to receive notifications any more the MnS consumer shall delete the corresponding NtfSubscriptionControl instance.

Creation and deletion of NtfSubscriptionControl instances by MnS consumers is optional; when not supported, the NtfSubscriptionControl instances may be created and deleted by the system or be pre-installed.

#### 4.3.22.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| notificationRecipientAddress | M | T | T | F | T |
| notificationTypes | O | T | T | F | T |
| scope | M | T | T | F | T |
| notificationFilter | O | T | T | F | T |

#### 4.3.22.3	Attribute constraints

None.

#### 4.3.22.4	Notifications

The common notifications defined in clause 4.5 are valid for this IOC, without exceptions or additions

### 4.3.23	Scope <<dataType>>

#### 4.3.23.1	Definition

The scope attribute has the data type Scope. The scope describes which object instances are selected with respect to a base object instance. The base object instance needs to be specified using a dedicated attribute.

#### 4.3.23.2	Attributes

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scopeType | M | T | T | F | F |
| scopeLevel | O | T | T | F | F |

#### 4.3.23.3	Attribute constraints

None.

#### 4.3.23.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name |  | Documentation and Allowed Values |  | Properties |  |
| --- | --- | --- | --- | --- | --- |
| heartbeatNtfPeriod |  | This attribute specifies the periodicity of heartbeat notification emission. The value of zero has the special meaning of stopping the heartbeat notification emission. Unit is in minutes. AllowedValues: non-negative integers |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |  |
| triggerHeartbeatNtf |  | Setting this attribute to TRUE triggers an immediate additional heartbeat notification emission. Setting the value to FALSE has no observable result. The periodicity of notifyHeartbeat emission is not changed. AllowedValues: TRUE, FALSE |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |  |
| notificationRecipientAddress |  | This attribute defines the address of the notification recipient. |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| notificationTypes |  | This attribute defines the types of notifications that are candidates for being forwarding to the notification recipient If the notificationFilter attribute is not supported all candidate notifications are forwarded to the notification recipient, otherwise the candidate notifications are discriminated by the filter specified by the notificationFilter attribute. |  | type: string multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: NOne isNullable: False |  |
| notificationFilter |  | This attribute defines a filter to be applied to candidate notifications identified by the notificationTypes attribute. Only notifications that pass the filter criteria are forwarded to the notification recipient. All other notifications are discarded. The filter can be applied to any field of a notification. |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| scope |  | This attribute defines the scope. It is a structured parameter with the data type Scope. |  | type: Scope multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| scopeType |  | If the optional scopeLevel parameter is not supported or absent, allowed values of scopeType are BASE_ONLY and BASE_ALL. The value BASE_ONLY indicates only the base object is selected. The value BASE_ALL indicates the base object and all of its subordinate objects (incl. the leaf objects) are selected. If the scopeLevel parameter is supported and present, allowed values of scopeType are BASE_ALL, BASE_ONLY, BASE_NTH_LEVEL and BASE_SUBTREE. The value BASE_NTH_LEVEL indicates all objects on the level, which is specified by the scopeLevel parameter, below the base object are selected. The base object is at scopeLevel zero. The value BASE_SUBTREE indicates the base object and all of its subordinate objects down to and including the objects on the level, which is specified by the scopeLevel parameter, are selected. The base object is at scopeLevel zero. |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| scopeLevel |  | See definition of scopeType parameter. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| farEndEntity |  | The value of this attribute shall be the Distinguished Name of the far end network entity to which the reference point is related. As an example, with EP_Iucs, if the instance of EP_Iucs is contained by one RncFunction instance, the farEndEntity is the Distinguished Name of the MscServerFunction instance to which this Iucs reference point is related. allowedValues: N/A |  | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No isNullable: False |  |
| linkType |  | This attribute defines the type of the link. allowedValues: Signalling, Bearer, OAM&P, Other or multiple combinations of this type. |  | type: String multiplicity: 0..* isOrdered: False isUnique: True defaultValue: No isNullable: False |  |
| locationName |  | The physical location of this entity (e.g. an address). allowedValues: N/A |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No isNullable: False |  |
| monitoringGP |  | This attribute defines the monitoring granularity period (in seconds). allowedValues: see NOTE 5 |  | type: Integer multiplicity: 1 isOrdered: False isUnique: True defaultValue: No isNullable: False |  |
| monitoredIOCName |  | It specifies one object class name for which the threshold monitor is created. When this attribute take effective, the threshold monitor is created for all of its instances in the name-containment tree whose top (tree) node is the IOC instance name-containing the subject <<IOC>>ThresholdMonitor instance containing this attribute. This attribute takes effective when the monitoredObjectDNs contained by the same <<IOC>>ThresholdMonitor instance is empty. allowedValues: The IOC names defined in the NRMs specifications. |  | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: No isNullable: False |  |
| monitoringNotifTarget |  | This identifies the target of the notifications when the monitored measurement crosses or reaches the threshold set by the subject threshold monitor. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |
| monitoredObjectDNs |  | It specifies the object instance(s) for threshold monitoring. This attribute monitoredIOCName contained by the same <<IOC>>ThresholdMonitor instance does not take effective when this attribute is not empty. allowedValues: list of DN |  | type: dataType multiplicity: 1 isOrdered: False isUnique: True defaultValue: No default value isNullable: False |  |
| objectClass |  | An attribute which captures the name of the class from which the object instance is an occurrence of. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| objectInstance |  | An information which captures the Distinguished Name of any object. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| peeParametersList |  | This attribute contains the parameter list for the control and monitoring of power, energy and environmental parameters of ManagedFunction instance(s). This list contains the following parameters: - siteIdentification - siteLatitude (optional) - siteLongitude (optional) - siteDescription - equipmentType - environmentType - powerInterface siteIdentification: The identification of the site where the ManagedFunction resides. allowedValues: N/A siteLatitude: The latitude of the site where the ManagedFunction instance resides, based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to the northern hemisphere. This attribute is optional in case of BTSFunction and RNCFunction instance(s). allowedValues: -90.0000 to +90.0000 siteLongitude: The longitude of the site where the ManagedFunction instance resides, based on World Geodetic System (1984 version) global reference frame (WGS 84). Positive values correspond to degrees east of 0 degrees longitude. This attribute is optional in case of BTSFunction and RNCFunction instance(s). allowedValues: -180.0000 to +180.0000 siteDescription: An operator defined description of the site where the ManagedFunction instance resides. allowedValues: N/A equipmentType: The type of equipment where the managedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. environmentType: The type of environment where the managedFunction instance resides. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. powerInterface: The type of power. allowedValues: see clause 4.4.1 of ETSI ES 202 336-12 [18]. |  | type: String multiplicity: 0..* isOrdered: N/A isUnique: True defaultValue: No default value isNullable: True |  |
| priorityLabel | This is a label that consumer would assign a value on a concrete instance of the managed object. The management system takes the value of this attribute into account. The effect of this attribute value to the subject managed entity is not standardized |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |  |
| protocolVersion |  | Versions(s) and additional descriptive information for the protocol(s) used for the associated communication link. Syntax and semantic is not specified. allowedValues: N/A |  | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: no default value isNullable: False |  |
| setOfMcc |  | Set of Mobile Country Code (MCC). The MCC uniquely identifies the country of domicile of the mobile subscriber. MCC is part of the IMSI (TS 23.003 [5]) This list contains all the MCC values in subordinate object instances to this SubNetwork instance. allowedValues: See clause 2.3 of TS 23.003 [5] for MCC allocation principles. |  | type: Integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: No default value isNullable: False |  |
| swVersion |  | The software version of the ManagementNode or ManagedElement (this is used for determining which version of the vendor specific information is valid for the ManagementNode or ManagedElement). allowedValues: N/A |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| systemDN |  | The Distinguished Name (DN) of IRPAgent. Defined in 3GPP TS 32.300. allowedValues: N/A |  | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| supportedMonitoringGPs |  | The monitoring granularity periods (in seconds) supported by the producer for the monitored entities. AllowedValues: see NOTE 5. |  | type: integer multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| thresholdInfoList |  | It specifies the list of threshold information. It is list of < measurementType, direction, thresholdPack >, where thresholdPack is list of < thresholdLevel, thresholdValue, hysteresis (optional) > The measurementType shall be in one of the following form: - "family.measurementName.subcounter" for monitoring the measurement types with subcounters defined. - "family.measurementName" for monitoring the measurement types without subcounters defined. The direction can be 'Increasing' or ‘Decreasing' of the measurement values. - If it is "Increasing", the threshold crossing notification is triggered when the measurement value equals or exceeds a thresholdValue. - If it is "Decreasing", the threshold crossing notification is triggered when the measurement value equals or below a thresholdValue. The thresholdLevel indicates the level for the threshold. See NOTE 6 for the supported threshold levels. The hysteresis has a threshold-high and a threshold-low value that are different from the threshold value. A hysteresis, therefore, defines the threshold-high and threshold-low levels within which the measurementType value is allowed to oscillate without triggering the threshold crossing notification. AllowedValues: N/A |  | type: dataType multiplicity: 1s..* isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| userDefinedState |  | An operator defined state for operator specific usage. allowedValues: N/A |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| userLabel |  | A user-friendly (and user assignable) name of this object. allowedValues: N/A |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| vendorName |  | The name of the vendor. allowedValues: N/A |  | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| vnfParametersList |  | This attribute contains the parameter set of the VNF instance(s) corresponding to an NE. Each entry in the list contains: - vnfInstanceId - vnfdId (optional) - flavourId (optional) - autoScalable vnfInstanceId: VNF instance identifier (vnfInstanceId, see section 9.4.2 of [16] and section B2.4.2.1.2.3 of [17]). See Note 1. vnfdId: Identifier of the VNFD on which the VNF instance is based, see section 9.4.2 of [16]. This attribute is optional. Note: the value of this attribute is identical to that of the same attribute in clause 9.4.2 of ETSI GS NFV-IFA 008 [16]. flavourId: Identifier of the VNF Deployment Flavour applied to this VNF instance, see section 9.4.3 of [16]. This attribute is optional. Note: the value of this attribute is identical to that of the same attribute in clause 9.4.3 of ETSI GS NFV-IFA 008 [16]. autoScalable: Indicator of whether the auto-scaling of this VNF instance is enabled or disabled. The type is Boolean. See Note2. The presence of this attribute indicates that the ManagedFunction represented by the MOI is a virtualized function. See Note 3. allowedValues: N/A A string length of zero for vnfInstanceId means the VNF instance(s) corresponding to the MOI does not exist (e.g. has not been instantiated yet, has already been terminated). |  | type: String multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: True |  |
| vsData |  | Vendor specific attributes of the type vsDataType. The attribute definitions including constraints (value ranges, data types, etc.) are specified in a vendor specific data format file. allowedValues: -- |  | type: -- multiplicity: -- isOrdered: -- isUnique: -- defaultValue: -- isNullable: False |  |
| vsDataFormatVersion |  | Name of the data format file, including version. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No isNullable: False |  |
| vsDataType |  | Type of vendor specific data contained by this instance, e.g. relation specific algorithm parameters, cell specific parameters for power control or re-selection or a timer. The type itself is also vendor specific. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No isNullable: False |  |
| pMAdministrativeState |  | It indicates the administrative state of MeasurementControl. It describes the permission to use or prohibition against using the capability of MeasurementControl, imposed through the consumer of OAM services produced by MeasurementControl, The measurement report production would begin when pMadministrativeState is UNLOCKED and pMoperationalState is ENABLED. The meaning of these values is as defined in 3GPP TS 28.625 [21] and ITU-T X.731 [19]. allowedValues: "LOCKED", "SHUTTING DOWN", "UNLOCKED". |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: “LOCKED” isNullable: False |  |
| pMOperationalState |  | It indicates the operational state of MeasurementControl. It describes if the resource is physically installed and working. allowedValues: “ENABLED”, “DISABLED”. The meaning of these values is as defined in 3GPP TS 28.625 [21] and ITU-T X.731 [19]. |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No allowedValues: N/A isNullable: False |  |
| managedObjectDNs |  | It identifies the managed entities whose Measurements are required to be produced. It identifies specific management entities say X, Y, Z . These are called the base. In case the base is SubNetwork, it identifies all, including the base, managed entities that are subordinates, in the sense of name-containment, of the base. In case the base is NetworkSliceSubnet, it identifies all, including the base, managed entities that has aggregation association relation with the base. The identified entities are called a collection. TIt would mean Measurement types specified in attribute MeasurementReader.measurementTypes, are required to be produced if the member (of the collection) is capable of supporting the Measurement types. allowedValues: N/A |  | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: No isNullable: False |  |
| managedObjectDNsBasic |  | It identifies the managed entities whose Measurements are required to be produced. It identifies specific managed entities (say X, Y, Z). It would mean Measurements type specified in MeasurementReader.measurementTypes, are required to be produced if X, Y, Z are capable of supporting the Measurement types. If managedObjectDNs of the same MeasurementReader instance has valid information, the information of this attribute is ignored. allowedValues: N/A |  | type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: No isNullable: False |  |
| streamBasedGP |  | It defines the frequency of producing and sending the Measurement to the consumer. allowedValues: See Note 4. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| measurementsList |  | It specifies a list of supported measurements and their GPs. A NULL value indicates there is no measurement supported. allowedValues: N/A |  | type: Measurements multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: True |  |
| measurementTypes |  | It identifies one or more Measurement types. The Measurement type can be those specified in TS 28.552 [20], TS 32.404 [21] and can be those specified by other SDOs or can be vendor-specific. allowedValues: N/A |  | type: String multiplicity: * isOrdered: N/A isUnique: True defaultValue: No isNullable: False |  |
| streamTarget |  | This identifies the target of the notification carrying the content of the measurement report. There are two delivery methods (i.e. file-based and stream-based) via which the consumer(s) can receive the Measurements. This attribute is used for the stream-based delivery method. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |  |
| gPs |  | It identifies the supported GPs, see Note 4. allowedValues: N/A |  | type: Integer multiplicity: * isOrdered: False isUnique: True defaultValue: No isNullable: False |  |
| nFServiceType |  | The parameter defines the type of the managed NF service instance allowedValues: See clause 7.2 of TS 23.501[22] |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: No default value isNullable: False |  |
| operations |  | This parameter defines set of operations supported by the managed NF service instance. allowedValues: See TS 23.502[23] for supporting operations |  | type: Operation multiplicity: 1..* isOrdered: False isUnique: False defaultValue: No default value isNullable: False |  |
| Operation.name |  | This parameter defines the name of the operation of the managed NF service instance. allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: False isUnique: False defaultValue: no default value isNullable: True |  |
| allowedNFTypes |  | This parameter identifies the type of network functions allowed to access the operation of the managed NF service instance. allowedValues: See TS 23.501[22] for NF types |  | type: ENUM multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| operationSemantics |  | This paramerter identifies the semantics type of the operation. See TS 23.502[23] allowedValues: “Request/Response”, “Subscribe/Notify”. |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| sAP |  | This parameter specifies the service access point of the managed NF service instance. allowedValues: N/A |  | type: SAP multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No default value isNullable: False |  |
| host |  | This parameter specifies the host address of the managed NF service instance. It can be FQDN (See TS 23.003 [5]) or an IPv4 address (See RFC 791 [24]) or an IPv6 address (See RFC 2373 [25]). allowedValues: N/A |  | type: String multiplicity: 1 isOrdered: False isUnique: N/A defaultValue: None isNullable: False |  |
| port |  | This parameter specifies the transport port of the managed NF service instance. allowedValues: 1 - 65535 |  | type: Integer multiplicity: 1 isOrdered: False isUnique: False defaultValue: None isNullable: False |  |
| ManagedNFService.operationalState |  | This parameter indicates the operational state of the managed NF service instance. It describes whether the resource is physically installed and working. allowedValues: "ENABLED", "DISABLED". The meaning of these values is as defined in 3GPP TS 28.625 [21] and ITU-T X.731 [19]. |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| ManagedNFService.administrativeState |  | This parameter indicates the administrative state of the manaed object instance. It describes the permission to use or prohibition against using the instance, imposed through the management services. allowedValues: “LOCKED”, “UNLOCKED”, SHUTTINGDOWN” The meaning of these values is as defined in 3GPP TS 28.625 [21] and ITU-T X.731 [19]. |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| ManagedNFService.usageStae |  | This parameter indicates the usage state of the object instance. It describes whether the resource is actively in use at a specific instant, and if so, whether or not it has spare capacity for additional users at that instant. allowedValues: "IDLE", "ACTIVE", "BUSY". The meaning of these values is as defined in 3GPP TS 28.625 [21] and ITU-T X.731 [19]. |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |  |
| registrationState |  | This parameter defines the registration status of the managed NF service instance. allowedValues: "Registered", "Deregistered". |  | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Deregistered isNullable: False |  |
| defaultFileBasedGP |  | This is a property of the file-based delivery method. See definition of fileBasedGP. This value is ignored in case the property captured in MeasurementReader is in use. GP unit is in minute. |  | Same as in fileBasedGP |  |
| defaultFileReportingPeriod |  | This is a property of the file-based delivery method. See definition of fileReportingGP. This value is ignored in case the property captured in MeasurementReader is in use. |  | Same as in fileReportingPeriod |  |
| defaultFileLocation |  | This is a property of the file-based delivery method. See definition of fileLocation. This value is ignored in case the property captured in MeasurementReader is in use. |  | Same as in fileLocation |  |
| defaultStreamBasedGP |  | This is a property of the stream-based delivery method. See definition of streamBasedGP. This value is ignored in case the property captured in MeasurementReader is in use. |  | Same as in streamBasedGP |  |
| defaultStreamTarget |  | This is a property of the stream-based delivery method. See definition of streamTarget. This value is ignored in case the property captured in MeasurementReader is in use. |  | Same as in streamTarget |  |
| fileBasedGP |  | This defines the frequency of producing the measurement data. The measurement data would be produced immediately at the end of each fileBasedGP.A measurement report file contains multiple measurement data. GP unit is in minute. allowedValues: See Note 4. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No isNullable: False |  |
| fileReportingPeriod |  | This defines the frequency of producing the measurement report files, in fileLocation, that hold the measurement reports. allowedValues: Its value is a multiple of fileBasedGP. |  | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No isNullable: False |  |
| fileLocation |  | This is used for the file-based delivery method. It is the path to a location on either the producer’s file system or a URI to a network file location that is not part of the producer’s file system. In case it points to a location on the producer’s file system, it is a relative path based on a vendor-specified root directory for measurement files. The size of this fileLocation is decided by consumer and producer. The producer is expected to remove old files to make room for new files, when necessary. allowedValues: Not applicable. |  | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No isNullable: True |  |
| NOTE 1: The value of this attribute is identical to that of the same attribute in clause 9.4.2 of ETSI GS NFV-IFA 008 [16]. NOTE 2: The value of this attribute is identical to that of the same attribute included in vnfConfigurableProperty in clause 9.4.2 of ETSI GS NFV-IFA 008 [16]. NOTE 3:The presence of the attribute vnfParametersList, whose vnfInstanceId with a string length of zero, in createMO operation can trigger the instantiation of the related VNF/VNFC instances. NOTE 4: The GP defines the measurement data production rate. The supported rates are dependent on the capacity of the producer involved (e.g. the processing power of the producer, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported GPs reflects the agreement between producer and the consumer involved. NOTE 5: The monitoring granularity period defines the measurements monitoring period. The supported monitoring periods are dependent on the capacity of the producer involved (e.g. the processing power of the producer, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported monitoring GPs reflect the agreement between producer and the consumer involved. NOTE 6: The supported threshold levels are dependent on the capacity of the producer involved (e.g. the processing power of the producer, number of measurements being measured by the producer at the time, the complexity of the measurement type involved etc) and therefore, it cannot be standardized for all producers involved. The supported levels can only reflect the negotiated agreement between producer and the consumer involved. |  |  |  |  |  |

### 4.4.2	Constraints

None

## 4.5	Common notifications

### 4.5.1	Alarm notifications

This clause presents a list of notifications, defined in [11], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in [3], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |

### 4.5.2	Configuration notifications

This clause presents a list of notifications, defined in [12], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in [3], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

Annex A (informative): Alternate class diagram

This class diagram combines the Figure 4.2.1-1 of this document with Figure 1 of [9], the class diagram of UIM.

Figure A-1: Alternate class diagram

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2012-12 |  |  |  |  | New version after approval | 2.0.0 | 11.0.0 |
| 2012-02 |  |  |  |  | MCC update of TOC | 11.0.0 | 11.0.1 |
| 2014-06 | SA#64 | SP-140332 | 001 | - | Correction of reference | 11.0.1 | 11.1.0 |
|  |  | SP-140358 | 002 | - | Remove the feature support statements |  |  |
| 2014-09 | SA#65 |  |  |  | Upgrade to Rel-12 | 11.1.0 | 12.0.0 |
| 2015-12 | SA#70 | SP-150691 | 005 | 1 | Add missing id attribute for 28.622 | 12.0.0 | 12.1.0 |
| 2016-01 |  |  |  |  | Upgrade to Rel-13 (MCC) | 12.1.0 | 13.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2016-12 | SA#74 | SP-160853 | 0010 | - | A | Clarification on the need to show VsDataContainer self-containing itself several times | 13.1.0 |
| 2017-03 | SA#75 | SP-170139 | 0012 | 2 | A | Clarify notification triggered by VsDataContainer change | 13.2.0 |
| 2017-03 | SA#75 | SP-170143 | 0015 | 1 | B | Modify definitions of ME and MF to support virtualized network element | 14.0.0 |
| 2017-03 | SA#75 | SP-170142 | 0016 | 3 | B | Adding an attribute for ManagedFunction to support management of virtualized NE | 14.0.0 |
| 2017-06 | SA#76 | SP-170510 | 0019 | 2 | B | Add VNFInfo related attributes in IOC ManagedFunction | 14.1.0 |
| 2018-01 | SA#78 | SP-170969 | 0021 | - | F | Missing note in table of Attribute Properties | 14.2.0 |
| 2018-03 | SA#79 | SP-180060 | 0022 | - | B | Add new attribute peeParametersList to IOC ManagedFunction | 15.0.0 |
| 2018-06 | SA#80 | SP-180421 | 0024 | 1 | B | Remove references to Itf-N | 15.1.0 |
| 2018-12 | SA#82 | SP-181156 | 0027 | - | F | Add the missing NRM fragment supporting network performance management | 15.2.0 |
| 2018-12 | SA#82 | SP-181042 | 0028 | 1 | F | Replace MF with ManagedFunction | 15.2.0 |
| 2018-12 | SA#82 | SP-181042 | 0029 | 1 | F | Update NRM root IOCs to support slice priority | 15.2.0 |
| 2019-06 | SA#84 | SP-190371 | 0031 | 2 | B | Add IOCs for threshold monitoring control | 16.0.0 |
| 2019-06 | SA#84 | SP-190373 | 0033 | 2 | B | Update generic NRM Information Service to support Managed NF Service Object | 16.0.0 |
| 2019-09 | SA#85 | SP-190744 | 0038 | 2 | A | Update class definition with inheritance information | 16.1.0 |
| 2019-09 | SA#85 | SP-190744 | 0043 | 1 | A | Correct PMControl (Add report period attribute and disambiguate the delivery method attributes) | 16.1.0 |
| 2019-09 | SA#85 | SP-190751 | 0044 | - | A | Correct NR definition to avoid misalignment with RAN2 and add NRM definition | 16.1.0 |
| 2019-09 | SA#85 | SP-190744 | 0046 | 1 | A | Correct definitions of granularity period. | 16.1.0 |
| 2019-09 | SA#85 |  |  |  |  | Correction in implementation of CR0043 | 16.1.1 |
| 2019-12 | SA#86 | SP-191158 | 0057 | 2 | A | Correct definition of network resource | 16.2.0 |
| 2019-12 | SA#86 | SP-191173 | 0059 | - | A | Add measurementsList attribute into related IOCs | 16.2.0 |
| 2019-12 | SA#86 | SP-191166 | 0062 | 2 | B | Add heartbeat control NRM fragment | 16.2.0 |
| 2019-12 | SA#86 | SP-191166 | 0063 | 2 | B | Add notification subscription control fragment | 16.2.0 |