3GPP TSG-SA5 Meeting #129e 	S5-201219

# e-meeting, 24 February – 4 March 2020

# Source:	Huawei

# Title:	Add message content

# Document for:	Approval

Agenda Item:	7.4.3

1	Discussion

This is a pCR to TS 28.201 Adding message flow.

2	References

[1]	3GPP TS 28.201 "Charging management; network slice performance and analytics charging in 5G System; Stage 2".

3	Rationale

This pCR proposes to add high level message flow in network slice performance and analytics charging principles and scenarios clause of TS 28.201 "Charging management; network slice performance and analytics charging in 5G System ; Stage 2".

4	Detailed propsosal

Propose to incorporate the following change into the TS 28.201 [1].

First changes

5.3	Message flows

5.3.x	Message flow of NSI start and modify for network slice performance and analytics charging

The following figure 5.3.x.1 describes abstract message flow of charging when a network slice instance is modified.

Figure 5.3.x.1: Abstract message flow of charging when a network slice instance is modified

Step 1-2. Network slice MnS consumer requests modification of NSI. The requirements of performance and analytics based charging can be changed with this request to modify NSI. The NSI and NSSI are modified by 3GPP management system.

Step 3ch-a. NSIF sends Charging Data Request [Event] to CHF for authorization and a network slice performance and analytics charging event.

Step 3ch-b. The CHF creates network slice CHF CDR for this Charging Data Request [Event].

Step 3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the NSIF.

End of changes