| 3GPP TS 28.535 V1.1.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management Services for Communication Service Assurance; Requirements (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4 	Concepts and background	8

4.1	Lifecycle of a communication service	8

4.2	Management control loops	8

4.2.1 	Overview	8

4.2.2	Control loops	9

4.2.3	Open control loops	10

4.2.4	Closed control loops	10

5 	Business level use cases and requirements	11

5.1	Use cases	11

5.1.1	Communication service assurance	11

5.1.2	Communication service assurance for shared resources	11

5.1.3	Use case for obtaining resource requirements for a communication service	12

5.1.4	Use case for interaction with core network for service assurance	12

6 	Specification level use cases and requirements	13

6.1	Use cases	13

6.1.1	Communication service quality assurance and optimization of communication services	13

6.1.2	 NWDAF assisted CSI SLS Assurance	14

6.2	Requirements	14

Annex A (informative): Management control loop in different layers	15

Annex <X> (informative): Change history	16

.

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# Introduction

The present document describes closed loop assurance solution enabling a service provider or an operator to continuously deliver the requested level of communication service quality to the customer and is part of a TS-family covering the 3rd Generation Partnership Project Technical Specification Group Services and System Aspects Management and orchestration of networks, as identified below:

TS 28.535: Management Services for Communication Service Assurance; Requirements

TS 28.536: Management Services for Communication Service Assurance; Stage 2 and stage 3

The solution described builds upon the management services specifications as identified below:

TS 28.530: Management and orchestration; Concepts, use cases and requirements

TS 28.533: Management and orchestration; Architecture framework.

TS 28.532: Management and orchestration; Management services.

TS 28.540: Management and orchestration; 5G Network Resource Model (NRM); Stage 1

TS 28.541: Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

TS 28.531: Management and orchestration; Provisioning.

TS 28.545: Management and orchestration; Fault Supervision (FS)

TS 28.550: Management and orchestration; Performance assurance

TS 28.552: Management and orchestration; 5G performance measurements

TS 28.554: Management and orchestration; 5G End to end Key Performance Indicators (KPI).

# 1	Scope

The present document describes, concepts and background, and specifies use cases and requirements for closed loop communication service assurance solution that adjusts and optimizes the services provided by NG-RAN and 5GC.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for next generation new services and markets"

[3]	3GPP TS 28.550: "Management and orchestration; Performance assurance"

[4]	3GPP TS 28.531: "Management and orchestration; Provisioning"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

communication services: subset of 3GPP defined services. Examples of 3GPP services (e.g. 5G LAN, URLLC) can be found in TS 22.261 [2].

communication service instance: run-time construct of a communication service for a defined group of users

NOTE: a communication service has two facades a customer facing façade and a resource facing façade, in this specification the CSI represents the resource facing façade,

service level specification: specification of the minimum acceptable standard of service

SLA requirements: service and network requirements derived from SLAs.

NOTE: A provider may add additional requirements not directly derived from SLA's, associated to provider internal [business] goals.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CS	Communication Service

CSA	Communication Service Assurance

CSC	Communication Service Customer

CSI	Communication Service Instance

CSP	Communication Service Provider

SLA	Service Level agreement

SLS	Service Level Specification

# 4 	Concepts and background

Editor’s Note: the following concepts are amongst others to be described in this clause, lifecycle of a communication service, service level specification, management control loop, interface/management services,

## 4.1	Lifecycle of a communication service

Communication service assurance applies to different phases in the life of a communication service instance (CSI), a CSI goes through the following lifecycle phases; preparation, commissioning, operation and decommissioning.

- Preparation phase:

Providing a CSI starts with preparation, which includes communication service design, pre-planning, feasibility check, i.e., checking the attainable communication service quality from both resource and service aspects, negotiation of the communication service attributes, preparing communication service and network requirements derived from SLA.

- CSI commissioning phase:

Once the CSI is prepared, it can be established by converting the communication service requirement for the CSI to network requirements (interaction and use of NF resources including RAN, CN) and creation of the CSI. When the CSI is created, it is deployed on the network resources and ready to be used by the communication service consumers (subscribers, UEs). Before allowing the maximum agreed number of communication service consumers to use the CSI a communication service assurance control loop is deployed to allow the network to converge to a state where the communication service assurance is stable and within the boundaries of the SLS. The assurance control loop learns the communication service behaviour during an initial deployment or trail phase.

- CSI operation phase:

After the commissioning phase, the CSI is activated for use of all communication service consumers (subscribers, UEs) that are allowed to use the communication service. The initial deployment or trail phase for the training of the communication service assurance algorithms has entered the operation phase. An activated CSI allows run-time operation of the communication service, e.g., quality of experience assurance, quality of service assurance. The optimization of CSI utilization may continue during the operation phase of the CSI.

- CSI decommissioning phase:

When the CSI is no longer needed, after being de-activated, the lifecycle of the CSI ends with CSI termination.

Figure 4.1.1 Lifecycle of a communication service instance

## 4.2	Management control loops

### 4.2.1 	Overview

For communication service assurance one can identify two interactions of management control loops:

- between the CSC and the CSP: In this case, the CSC provides the requirements for an assured communication service to the CSP, the CSP provides the corresponding communication service, the CSP also provides feedback to the CSC. The CSP adjusts the CSI or the CSC adjusts the SLS continuously to achieve the assured requirements.

- between the CSP and the NSP: the CSI provided by CSP requires the network capabilities. For example, the CSP requires a certain network latency. The NSP management system adjusts the network or CSP adjusts the latency requirement continuously to satisfy the latency requirement.

Figure 4.2.1.1: Communication service closed loop assurance

Generally, the management control loop for a CSI consists of Monitoring, Analysis, Decision and Execution, and the adjustment of CSI is completed by the continuous iteration of the steps in a management control loop. As described in section 4.1, the management control loop for CSI is deployed in the preparation phrase and takes effect during the preparation phase and operation phase.

Figure 4.2.1.1 shows the overall process of the CSI management control loop:

Figure 4.2.1.1: Management Control Loop

### 4.2.2	Control loops

A control loop is a building block for management of networks and services. The basic principle of any control loop is to adjust the value of a measured or observed variable (expressed as for example an attribute) to equal the value of a desired goal (expressed as for example an attribute). The producer of the measurements or observations, the control service, and the controlled entity are all required to create a control loop.

For the control loop to act on input in the context of the set goal, the control loop provided through following four steps that continuously consume and produce information from each other in a loop in the following sequence monitor, analyse, decide and execute

A control loop can be an open control loop in which case a human operator or other operations system is involved, and the control loop is dependent on availability and reaction time of a human operator or other operations system. A control loop can be closed and operates without human operator or other operations system involvement other than possibly the initial configuration of the measurement producer and configuration of control loop.

### 4.2.3	Open control loops

In an open control loop, the human operator intervenes in one or more of the process steps of the loop, see Figure 4.2.3.1. The human is in full control of all aspects of the detail steps in control loop, including decisions taken in the loop. The management system collects, analyses and presents the data to the operator, but the operator decides which action to take.

Figure 4.2.3.1: Open control loop entities

### 4.2.4	Closed control loops

In a closed control loop, there is no direct involvement of a human operator or other operations system in the control loop, the control loop is fully automated. As shown in Figure 4.2.4.1 the human operator or other operations system is not fully absent but in a different role, not directly controlling the details inside the process steps but using goals that allow the control loop to make autonomous decisions within the boundaries of the set goal. Once the control loop is configured, and the goal is set the controlled entity is adjusted according the input and the set goals.

In a closed loop the input to the control loop provided by human operator or other operations system is the goal which allows the control loop to operate autonomously, whilst providing reports and explanations as output to a human operator or other operations system.

Typically the goal is set within certain parameter boundaries, the control loop can automatically adjust the output based on the input within the parameter boundaries, however once control loop cannot automatically adjust the human operator or other operations system needs to be informed and the control loop becomes an open control loop, any decisions are now made by the human operator or other operations system and not by the control loop.

Figure 4.2.4.1: Closed control loop entities

# 5 	Business level use cases and requirements

## 5.1	Use cases

### 5.1.1	Communication service assurance

The CSP wants to meet the CSC expectations on automation as well as internal goals on CAPEX and OPEX efficiency.

The CSP has access to capabilities, procedures and tools that can address both CAPEX and OPEX in the provisioning and management of communication services to their customers (CSC). The CSC expects the CSP to offer a variety of communication services including business critical communication services that allow the CSC (e.g. Enterprise) to run their applications in a predictable manner [2]. Hence automation of the on-boarding of the CSC application, which will use communication services provided by the CSP, on a 5GS, is a requirement to meet the following needs:

- reduce the complexity for a CSC application to be on-boarded on a 5GS.

- improve the network performance over time, based on predicting communication service behaviour

- reduce the cost ownership through automation

During the operation of the communication service the CSP provides assurance of service quality expectation and CSP meets the CSC expectations on automation as well as internal goals on CAPEX and OPEX efficiency.

REQ-CSA_CSA-FUN-01 The CSP shall be able to provide a statement of CS service requirements to a 5GS and receive capability information about these from the 5GS.REQ-CSA_CSA-FUN-03 The 5GS shall have the capabilities to monitor, and report to CSP the fulfilment of committed CS requirements and actions taken to adjust for deviations.

REQ-CSA_CSA-FUN-03 The 5GS shall have the capabilities to monitor and report to CSP actions taken to adjust deviations on committed CS requirements.

REQ-CSA_CSA-FUN-04 The 5GS shall have the capability to provide in-operation assurance of service quality expectation.

REQ-CSA_CSA-FUN-04 The 5GS shall have the capability to ensure the service quality requirements during the service operations.

Editor’s Note: The specification level requirements on OAM are for FFS.

### 5.1.2	Communication service assurance for shared resources

Editor's Note: The need for a use case of communication service assurance for dedicated resources is FFS.

A CSP network where at least one eMBB CSI is operating providing services to end-users. A CSC requests from the CSP Order Care a new CSI for business-critical application(s), submitting an initial proposed CSI SLA. The management system assists CSP Order Care with analysis of the proposed CSI SLA and, when SLA is committed, works together with NF’s to ensure CSI SLA goals and optimal use of resources for previous as well as the new CSI(s).

In this scenario, it is assumed that the SLA’s for the two CSI’s will allow for them to share resources, for example RAN and TN resources.

The management systems CS-Assurance service receives the request from Order Care and using a MDAS CS Preparation Assistance service, explores and evaluates CSI realisation and impact on other CSIs, if any.

Once the Order Care has committed to an SLA with a CSC, the management system activates the CSI.

As the CSI operates, a management service for CSI assurance, CSI-Assurance, continuously monitors the SLA fulfilment using MDAS, PM Assurance Services [3] including and Core Network NwDAF QoE analytics service, if available.

Based on goals for CSI SLA fulfilment, or other KPI’s, the CSI-Assurance service may initiate an action when SLA goals are not met, be that over- or under fulfilment. The CSI-Assurance service may use an MDAS to assist in selecting proper action and how to best execute the action.

The CSI-Assurance triggers the action by using provisioning service [4] towards RAN, Transport and Core Network and monitors the effect of the change.

Editor’s Note: Whether provisioning services shall be including also management of SON functions when such are available, is FFS.

REQ-CSIA_CON-xx: The 3GPP management system shall have the capability providing a management service for assisting in assessing (evaluating) a proposed SLA for a requested CSI.

REQ-CSIA_CON-xx: The 3GPP management system shall have the capability providing a management service for assisting in asserting an agreed SLA for a requested CSI.

REQ-CSIA_CON-xx: The 3GPP management system shall have the capability to process 5GS data and provide analytics services to its consumers.

Editor’s Note: information exchange with application such QoE reporting, is FFS

### 5.1.3	Use case for obtaining resource requirements for a communication service

Once a request for a communication service is received, in the service provisioning phase of a CSI, the 3GPP management system needs to identify the resources required for the service in order to do service assurance. For example, during the feasibility study, in order to assure the performance, the 3GPP management system should be able to determine the resource availability for that service. This could be done by requesting the MDAS provider about the resource requirements and checking the available resources.

MDAS provider may already have resource requirement for a given service requirement, obtained by the historical analysis using offline or online monitoring of resource usage of similar services. In that case, the 3GPP management system can determine the feasibility and if feasible provision the communication services using those resources to go to the operational phase.

If the resource requirement cannot be determined (e.g. not sufficient prior data), the 3GPP management system may assign certain amount of initial resources and limit the number of users admitted by configuring the appropriate CN functions. The resource usage information and the services using those resources in a given time period with their performance (e.g. delay) is monitored by the 3GPP management system for different number of UEs to learn the resource requirement for different service requirements. This data could be used to determine resource requirements for future service requests during the provisioning phase or to adjust resources to reflect the changing service demands for the already admitted communication services.

It may be a continuous learning process in the run-time phase, since service degradation could happen due to various reasons and resources may need to be adjusted to address such situations.

REQ-CSA_RR-CON-01 The 3GPP management system shall be able to determine the resource requirement for a given communication service requirement.

REQ-CSA_RR-CON-02 The 3GPP management system shall be able to allocate certain amount of resources for a communication service and configure the 5GC functions to limit the number of users of a given communication service instance.

### 5.1.4	Use case for interaction with core network for service assurance

The goal is to enable the 3GPP management system to take early action to prevent service degradation.

The 3GPP management system configures the control plane functions (e.g. NWDAF) so as to report potential service degradation according to the SLS.  If the service degradation occurs or predicted when the resources are scaled down, resources could be scaled up to solve the issue. Therefore, it is necessary for the 3GPP management system to configure the 5GC functions such that in the event that a potential service degradation or overloading is predicted, that is sent to the 3GPP management system. This can be done by properly configuring the service overloading conditions (e.g. triggering parameters) in the 5GC functions. The 3GPP management system could configure the 5GC functions to trigger when the service load is increased or predicted to be increased beyond a certain threshold level. The 3GPP management system could then do resource scaling or use MDAS to find a proper solution.

Similarly, when the resources are underutilized the 3GPP management system could do scaling down or deactivation of resources.

REQ-CSA_RR-CON-01 The 3GPP management system shall be able to configure the 5GC functions to make them report of a potential service load increase beyond a certain threshold so that the 3GPP management system can do scaling up of resources in time without impacting the SLA.

REQ-CSA_RR-CON-02 The 3GPP management system shall be able to determine the service load thresholds that need to be used by the 5GC functions to report, so that a potential resource overprovisioning situation can be ascertained.

REQ-CSA_RR-CON-03 The 3GPP management system shall be able to perform scaling down of resources when a resource overprovisioning is detected.

Editor's Note: the service load needs to be clarified.

# 6 	Specification level use cases and requirements

## 6.1	Use cases

Editor’s Note: use cases are documented as flat list clauses with one use case per clause until a structure appears, then the use cases may be grouped. Each use case is documented using the use case template

### 6.1.1	Communication service quality assurance and optimization of communication services

The goal of the use case is to enable communication service quality assurance and optimization for the set of services provided by the network to certain group (category) of UEs. For example, the set can include the communication services provided via certain NSI(s) or to IoT devices in certain area.

It is assumed that the relevant NFs are deployed and active in NG-RAN and 5GC. The group of NG-RAN and 5GC nodes, which are essential for the set of E2E services, provide provisioning and PM management services. It is also assumed that the providers of the related NSI / NSSI provisioning and PM management services are deployed and active.

The management system is consuming above management services either directly or through proxy nodes that re-expose the management services; the management system is aware of the performance requirements imposed on the set of communication services.

The management system is collecting the service experience information and monitoring the key performance indicators, KPIs, related to the targeted services. Analytics hosted by the MDAF may be utilized for processing of the network data to derive and analyse the KPIs. If the service quality assurance and optimization function detects performance degradation or potential for improvement of the targeted KPIs, the 3GPP management system modifies the configuration parameters in the corresponding NG-RAN and 5GC nodes and NSI(s)/NSSI(s), using the NG-RAN, 5GC, and NSI(s)/NSSI(s) (where relevant) provisioning services, and continues monitoring the performance.

If the network performance does not recover or improve, the management system may adjust the modifications, for example roll back to previous configuration. In any case it continues collecting the network data and monitoring of the performance indicators.

### 6.1.2		NWDAF assisted CSI SLS Assurance

The goal of this use case is to assure the SLS (Service Level Specifications) for a particular CSI (Communication Service Instance) is crucial for the 5G network management. The negotiated SLS for a particular CSI should be assured in an autonomous way.

3GPP management system can be leveraged to enable autonomous SLS assurance for a deployed Communication Service. 3GPP management system can collect QoE data, related to network slice and applications, from NWDAF. Since the data collected will relate to network slice and a single NSI may be serving multiple CSIs, the corresponding QoE data for the target CSI need to be ascertained. Once the QoE data for a CSI is known, the SLS breach can also be ascertained. If the SLS is breached, the root cause analysis is performed to find the cause for SLS breach. Depending on the location of cause (at RAN or at, 5GC), remedial actions will be initiated to mitigate the SLS breach and network optimization is done so that the negotiated SLS can be assured.

The QoE analytical data from NWDAF is per Application for an NSI. It is crucial to derive the CSI level QoE data from the data received from NWDAF in order to ascertain the SLS breach.

.

## 6.2	Requirements

REQ-CSM_SAO-CON-01 The 3GPP management system shall have the capability to take actions based on the target communication service KPIs for a set of communication services supplied to a group or category of UEs.

REQ-CSM_SAO-CON-02 The 3GPP management system shall have the capability to collect service experience information.

REQ-CSM_SAO-CON-03The 3GPP management system shall have the capability to analyse the KPIs related to the set of communication services delivered to a group or category of UEs.

REQ-CSM_SAO-CON-04 The 3GPP management system shall have the capability to modify the configuration parameters related to the set of communication services supplied to a group or category of UEs.

REQ-SLS_ASU-XXX-01	The management system for SLS assurance shall have the capability to collect NSI related QoE data from NWDAF.

REQ-SLS_ASU-XXX-02	The management system for SLS assurance shall have the capability to derive CSI related QoE data from the collected NSI related QoE data.

REQ-SLS_ASU-XXX-03	The management system for SLS assurance shall have the capability to ascertain SLS breach.

REQ-SLS_ASU-XXX-04	The management service for SLS assurance shall have the capability to perform the root cause analysis, identifying the underlying reason, for an SLS breach.

REQ-SLS_ASU-XXX-05	The management system for SLS assurance shall have the capability to take corrective actions against the root cause identified.

NOTE: The management system refers to the producer of management service for SLS assurance.

Annex A (informative):
Management control loop in different layers

This example gives a high-level view of management control loops in different layers, which consists of CSI control loop, NSI control loop, NSSI control loop and NF control loop, which described as Figure X.1, where the analytic could be leverage MDAS, and different control loop can provide input (interact with) to other control loops.

Editor’s Note: the management control loop inside the ME is FFS.

Figure A.1 Management control loop in different layers

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-09 | SA5#126 |  |  |  |  | Skeleton | 0.0.0 |
| 2019-10 | SA5#127 | S5-196611 |  |  |  | pCR 28.535 Add scope | 0.1.0 |
| 2019-10 | SA5#127 | S5-196612 |  |  |  | pCR 28.535 Add introduction | 0.1.0 |
| 2019-10 | SA5#127 | S5-196483 |  |  |  | pCR 28.535 Introduce table of content | 0.1.0 |
| 2019-10 | SA5#127 | S5-196615 |  |  |  | pCR 28.535 Add lifecycle of a communication service | 0.1.0 |
| 2019-10 | SA5#127 | S5-196478 |  |  |  | pCR 28.535 Add management control loops | 0.1.0 |
| 2019-11 | SA5#128 | S5-197582 |  |  |  | pCR 28.535 Communication service assurance scenario | 0.2.0 |
| 2019-11 | SA5#128 | S5-197585 |  |  |  | pCR 28.535 use case for obtaining resource requirements for a communication service | 0.2.0 |
| 2019-11 | SA5#128 | S5-197787 |  |  |  | pCR 28.535 Service quality assurance and optimization | 0.2.0 |
| 2019-11 | SA5#128 | S5-197833 |  |  |  | pCR 28.535 Interaction with core network for service assurance | 0.2.0 |
| 2019-12 | SA#86 | SP-191186 |  |  |  | Presented for information | 1.0.0 |
| 2020-03 | SA5#129 | S5-201488 |  |  |  | pCR 28.535 Add text for clause 4.2 Management control loops | 1.1.0 |
| 2020-03 | SA5#129 | S5-201437 |  |  |  | pCR 28.535 Add UC on NWDAF assisted SLS Assurance | 1.1.0 |