3GPP TSG-SA5 Meeting #129-e	S5-201412

# Online, , 24th Feb 2020 - 4th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 0320 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add MDT trace recording session start and stop mechanism for 5G |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oy LM Ericsson AB |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMDT |  |  |  |  |  | Date: |  |  | 2020-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add MDT trace recording session start and stop mechanism for 5G |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add trace recording session start and stop mechanism for 5G |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MDT trace recording session start and stop mechanism for 5G would be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.X, 4.2.2.Y, 4.2.2.Z, 4.2.2.Q, 4.2.2.U, 4.2.2.V |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 4.2.2.X	NG-RAN starting mechanisms for management based MDT

A trace recording session of immediate MDT or logged MDT shall be started in the gNodeB for each selected UE that satisfy the MDT UE selection criteria (i.e. capability condition), provided that a cell trace session for immediate MDT or logged MDT has been activated in the gNodeB from management system for the given cell(s) before.

The gNodeB shall configure the corresponding MDT RRC measurements at the selected UE.

When several PLMNs are supported in the RAN for management based MDT, possibly combined with Trace, the gNB shall only select UEs where the pLMNTarget = selectedPLMN-Identity that the UE includes in RRCConnectionSetup message 3GPP TS 36.331 [32].

NOTE:	The NG-RAN starting mechanism for split architecture is not included in this release.

Next change

#### 4.2.3.Y	5GC starting mechanisms for signalling based MDT

In the AMF, no trace recording sessions are started for MDT trace sessions. The AMF sends the trace session activation to the gNodeB with parameters as specified in 4.1.2.12.

Next change

#### 4.2.3.Z	NG-RAN starting mechanisms for signalling based MDT

A trace recording session of either immediate or logged MDT shall be started in the gNodeB for a given UE when a trace session activation request is received from the AMF for the UE and the MDT UE selection conditions are satisfied for the UE. The gNodeB shall configure the corresponding MDT RRC measurements at the UE. If selection conditions are not satisfied, the gNodeB shall store the trace control and configuration parameters and forward these parameters when the UE handovers to other gNBs over Xn or N2.

If the gNodeB receives a signalling based MDT activation request when the UE is served by a cell that is in the gNodeB but not in the MDT area scope then the gNodeB shall store the MDT configuration and configure the UE when the UE moves to a cell in the gNodeB (intra gNodeB handover) that satisfies the area scope in the request.

NOTE:	The NG-RAN starting mechanism for split architecture is not included in this release.

Next change

#### 4.2.4.Q	NG-RAN stopping mechanisms for management based MDT

In case of immediate MDT, the gNodeB shall stop a trace recording session for a given UE when the UE changes cell or goes to idle mode or inactive state or when the cell trace session is deactivated at the gNodeB from its NG-RAN management system The gNodeB shall deactivate the corresponding MDT RRC measurements in the UE.

In case of logged MDT, there is no stopping mechanism in the gNodeB. The gNodeB does not need to maintain a logged MDT, trace recording session once it has been configured in the UE.

NOTE:	The NG-RAN stoping mechanism for split architecture is not included in this release.

Next change

#### 4.2.5.U	5GC stopping mechanisms for signalling based MDT

There is no stopping mechanism in the 5GC for MDT trace recording sessions, as there are no starting mechanisms either (see also clause 4.2.3.Y).

Next change

#### 4.2.5.V	NG-RAN stopping mechanisms for signalling based MDT

In case of immediate MDT, the gNodeB shall stop an ongoing trace recording session for a given UE when a trace session deactivation is received from the AMF. The gNodeB shall deactivate the corresponding MDT measurements in the UE.

If the configured area scope is not satifisfied in the target cell after a handover, the gNB may deactivate the Immediate MDT configured to the UE like explained in clause 4. 4.

In case of logged MDT, there is no stopping mechanism in the gNodeB. The gNodeB does not need to maintain a logged MDT trace recording session once it has been configured in the UE.

NOTE:	The NG-RAN stoping mechanism for split architecture is not included in this release.

End of changes