3GPP TSG-SA5 Meeting #130-e	S5-202174

# Online, , 20th Apr 2020 - 28th Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0225 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PDU Address in for IPv6 multi-homing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI 16 |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PDU Addresses per PSA for IPv6 multi-homing is not specified in TS 32.291. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution is to add PDU Address per PSA in PDU container information for IPv6 multi-homing, including in the charging data request/response message and CDR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In IPv6 multi-homing scenario, only one PDU address is reported to CHF, and other PDU addresses are droped by SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.2.9, 7.2, A.2, A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change to TS 32.291

6.1.6.2.2.9	Type PDUContainerInformation

Table 6.1.6.2.2.9-1: Definition of type PDUContainerInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timeofFirstUsage | DateTime | OC | 0..1 | the time stamp for the first IP packet to be transmitted and mapped to the reporting used unit. |  |
| timeofLastUsage | DateTime | OC | 0..1 | the time stamp for the last IP packet to be transmitted and mapped to the reporting used unit. |  |
| qoSInformation | QoSData | OC | 0..1 | the QoS applied for the reporting used unit. In case gbrUl or gbrDl are present for GBR flow, the GBR targets are "GUARANTEED", otherwise, are " NOT_GUARANTEED". |  |
| qoSCharacteristics | QosCharacteristics | OC | 0..1 | Map of QoS characteristics for non standard 5QIs and non-preconfigured 5QIs. |  |
| afChargingIdentifier | ChargingId | OC | 0..1 | An identifier, provided from the AF, may be used to correlate the measurement for the Charging key/Service identifier values in this PCC rule with application level reports. |  |
| userLocationInformation | UserLocation | OC | 0..1 | provides information on the location |  |
| uetimeZone | TimeZone | OC | 0..1 | the UE Time Zone during the used unit container interval. |  |
| rATType | RatType | OC | 0..1 | the RAT Type of the used unit |  |
| servingNodeID | array(ServingNetworkFunctionID) | OC | 0..N | the list of serving node identifiers during the used unit container interval. |  |
| presenceReportingAreaInformation | map(PresenceInfo) | OC | 0..N | the Presence Reporting Area status of UE during the used unit container interval. |  |
| 3gppPSDataOffStatus | 3GPPPSDataOffStatus | OC | 0..1 | the 3GPP Data off Status during the used unit container interval. |  |
| sponsorIdentity | string | OC | 0..1 | an identifier of the sponsor. |  |
| applicationserviceProviderIdentity | string | OC | 0..1 | an identifier of the application service provider |  |
| chargingRuleBaseName | string | OC | 0..1 | the reference to group of PCC rules predefined at the SMF. |  |
| usedPDUAddress | PDUAddress | OC | 0..1 | user ip address/prefix for IPv6 multi-homing. |  |

2nd change to TS 32.291

7.2	Bindings for 5G data connectivity

Table 7.2-1: Bindings of 5G data connectivity CDR field, Information Element and Resource Attribute

| Information Element |  | CDR Field |  | Resource Attribute |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ChargingDataRequest |  |
| Multiple Unit Usage |  | List of Multiple Unit Usage |  | /multipleUnitUsage |  |
| UPF ID |  | UPF ID |  | /multipleUnitUsage/uPFID |  |
| Used Unit Container |  | Used Unit Container |  | /multipleUnitUsage/usedUnitContainer |  |
| PDU Container Information |  | PDU Container Information |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation |  |
| Time of First Usage |  | Time of First Usage |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/timeofFirstUsage |  |
| Time of Last Usage |  | Time of Last Usage |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/timeofLastUsage |  |
| QoS Information |  | QoS Information |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/qoSInformation |  |
| QoS Characteristics |  | QoS Characteristics |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/qoSCharacteristics |  |
| AF Charging Identifier |  | AF Charging Identifier |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/afChargingIdentifier |  |
| User Location Information |  | User Location Information |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/userLocationInformation |  |
| UE Time Zone |  | UE Time Zone |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/uetimeZone |  |
| RAT Type |  | RAT Type |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/rATType |  |
| Serving Network Function ID |  | Serving Network Function ID |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/servingNodeID |  |
| Presence Reporting Area Information |  | Presence Reporting Area Information |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/ presenceReportingAreaInformation |  |
| 3GPP PS Data Off Status |  | 3GPP PS Data Off Status |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/3gppPSDataOffStatus |  |
| Sponsor Identity |  | Sponsor Identity |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/sponsorIdentity |  |
| Application Service Provider Identity |  | Application Service Provider Identity |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/applicationserviceProviderIdentity |  |
| Charging Rule Base Name |  | Charging Rule Base Name |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/chargingRuleBaseName |  |
| Used PDU Address |  | Used PDU Address |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/usedPDUAddress |  |
| PDU Session Charging Information |  | PDU Session Charging Information |  | /pDUSessionChargingInformation |  |
| Charging Id |  | Charging Id |  | /pDUSessionChargingInformation/chargingId |  |
| Home Provided ChargingId |  | Home Provided ChargingId |  | /pDUSessionChargingInformation/ homeProvidedChargingId |  |
| User Information |  | User Information |  | /pDUSessionChargingInformation/ userInformation |  |
| User Identifier |  | User Identifier |  | /pDUSessionChargingInformation/userInformation/servedGPSI |  |
| User Equipment Info |  | User Equipment Info |  | /pDUSessionChargingInformation/userInformation/ servedPEI |  |
| Unauthenticated Flag |  | Unauthenticated Flag |  | /pDUSessionChargingInformation/userInformation/ unauthenticatedFlag |  |
| Roamer In Out |  | Roamer In Out |  | /pDUSessionChargingInformation/userInformation/ roamerInOut |  |
| User Location Info |  | User Location Info |  | /pDUSessionChargingInformation/ userLocationinfo |  |
| UE Time Zone |  | UE Time Zone |  | /pDUSessionChargingInformation/uEtimeZone |  |
| Presence Reporting Area Information |  | Presence Reporting Area Information |  | /pDUSessionChargingInformation/ presenceReportingAreaInformation |  |
| PDU Session Information |  | PDU Session Information |  | /pDUSessionChargingInformation/pduSessionInformation |  |
| PDU Session ID |  | PDU Session ID |  | /pDUSessionChargingInformation/pduSessionInformation/pduSessionID |  |
| Network Slice Instance Identifier |  | Network Slice Instance Identifier |  | /pDUSessionChargingInformation/pduSessionInformation/networkSlicingInfo |  |
| PDU Type |  | PDU Type |  | /pDUSessionChargingInformation /pduSessionInformation/pduType |  |
| PDU Address |  | PDU Address |  | /pDUSessionChargingInformation /pduSessionInformation/pduAddress |  |
| PDU IP Address |  | PDU IP Address |  | /pDUSessionChargingInformation/pduSessionInformation/pduAddress/pduIPv4Address /pDUSessionChargingInformation/pduSessionInformation/pduAddress/pduIPv6Addresswithprefix |  |
| PDU Address prefix length |  | PDU Address prefix length |  | /pDUSessionChargingInformation /pduSessionInformation/pduAddress/pduAddressprefixlength |  |
| Dynamic Address Flag |  | Dynamic Address Flag |  | /pDUSessionChargingInformation /pduSessionInformation/pduAddress/ iPv4dynamicAddressFlag pDUSessionChargingInformation /pduSessionInformation/pduAddress/ iPv6dynamicAddressFlag |  |
| SSC Mode |  | SSC Mode |  | /pDUSessionChargingInformation /pduSessionInformation/sscMode |  |
| SUPI PLMN ID |  | SUPI PLMN ID |  | /pDUSessionChargingInformation /pduSessionInformation/hPlmnId |  |
| Serving Network Function ID |  | Serving Network Function ID |  | /pDUSessionChargingInformation /pduSessionInformation/ servingNetworkFunctionID |  |
| Serving CN PLMN ID |  | Serving CN PLMN ID |  | /pDUSessionChargingInformation/pduSessionInformation/servingCNPlmnId |  |
| RAT Type |  | RAT Type |  | /pDUSessionChargingInformation /pduSessionInformation/ratType |  |
| Data Network Name Identifier |  | Data Network Name Identifier |  | /pDUSessionChargingInformation /pduSessionInformation/dnnid |  |
| DNN Selection Mode |  | DNN Selection Mode |  | /pDUSessionChargingInformation /pduSessionInformation/dNNselectionMode |  |
| Authorized QoS information |  | Authorized Qos Information |  | /pDUSessionChargingInformation /pduSessionInformation/authorized qoSInformation |  |
| Subscribed QoS Information |  | Subscribed QoS Information |  | /pDUSessionChargingInformation /pduSessionInformation/subscribedQoSInformation |  |
| Authorized Session-AMBR |  | Authorized Session-AMBR |  | /pDUSessionChargingInformation /pduSessionInformation/authorizedSessionAMBR |  |
| Subscribed Session-AMBR |  | Subscribed Session-AMBR |  | /pDUSessionChargingInformation /pduSessionInformation/subscribedSessionAMBR |  |
| Charging Characteristics |  | Charging Characteristics |  | /pDUSessionChargingInformation /pduSessionInformation/ chargingCharacteristics |  |
| Charging Characteristics Selection Mode |  | Charging Characteristics Selection Mode |  | /pDUSessionChargingInformation /pduSessionInformation/chargingCharacteristicsSelectionMode |  |
| PDU session start Time |  | PDU session start Time |  | /pDUSessionChargingInformation /pduSessionInformation/startTime |  |
| PDU session stop Time |  | PDU session stop Time |  | /pDUSessionChargingInformation /pduSessionInformation/stopTime |  |
| Diagnostics |  | Diagnostics |  | /pDUSessionChargingInformation /pduSessionInformation/diagnostics |  |
| 3GPP PS Data Off Status |  | 3GPP PS Data Off Status |  | /pDUSessionChargingInformation /pduSessionInformation/3gppPSDataOffStatus |  |
| Session Stop Indicator |  | Session Stop Indicator |  | /pDUSessionChargingInformation /pduSessionInformation/sessionStopIndicator |  |
| Unit Count Inactivity Timer |  | - |  | /pDUSessionChargingInformation/unitCountInactivityTimer |  |
| RAN Secondary RAT Usage Report |  | RAN Secondary RAT Usage Report |  | /pDUSessionChargingInformation/rANSecondaryRATUsageReport |  |
| NG RAN Secondary RAT Type |  | NG RAN Secondary RAT Type |  | /pDUSessionChargingInformation/rANSecondaryRATUsageReport/rANSecondaryRATType |  |
| Qos Flows Usage Reports |  | Qos Flows Usage Reports |  | /pDUSessionChargingInformation/rANSecondaryRATUsageReport/qosFlowsUsageReports |  |
| Roaming QBC information |  | Roaming QBC information |  | /roamingQBCInformation |  |
| Multiple QFI container |  | Multiple QFI container |  | /roamingQBCInformation/multipleQFIcontainer |  |
| Triggers |  | Triggers |  | /roamingQBCInformation/multipleQFIcontainer/triggers |  |
| Trigger Timestamp |  | Trigger Timestamp |  | /roamingQBCInformation/multipleQFIcontainer/triggerTimestamp |  |
| Time |  | Time |  | /roamingQBCInformation/multipleQFIcontainer/time |  |
| Total Volume |  | Total Volume |  | /roamingQBCInformation/multipleQFIcontainer/totalVolume |  |
| Uplink Volume |  | Uplink Volume |  | /roamingQBCInformation/multipleQFIcontainer/uplinkVolume |  |
| Downlink Volume |  | Downlink Volume |  | /roamingQBCInformation/multipleQFIcontainer/downlinkVolume |  |
| Local Sequence Number |  | Local Sequence Number |  | /roamingQBCInformation/multipleQFIcontainer/localSequenceNumber |  |
| QFI Container information |  | QFI Container information |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation |  |
| QoS Flow Id |  | QoS Flow Id |  | /roamingQBCInformation/multipleQFIcontainer/ qFIContainerInformation/qFI |  |
| Time of First Usage |  | Time of First Usage |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/ timeofFirstUsage |  |
| Time of Last Usage |  | Time of Last Usage |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/timeofLastUsage |  |
| QoS Information |  | QoS Information |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/qoSInformation |  |
| QoS Characteristics |  | QoS Characteristics |  | /roamingQBCInformation/multipleQFIcontainer/ qFIContainerInformation/qoSCharacteristics |  |
| User Location Information |  | User Location Information |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/ userLocationInformation |  |
| UE Time Zone |  | UE Time Zone |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/uetimeZone |  |
| Presence Reporting Area Information |  | Presence Reporting Area Information |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/presenceReportingAreaInformation |  |
| RAT Type |  | RAT Type |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/rATType |  |
| Report Time |  | Report Time |  | /roamingQBCInformation/multipleQFIcontainer/ qFIContainerInformation/reportTime |  |
| Serving Network Function ID |  | Serving Network Function ID |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/ servingNetworkFunctionID |  |
| 3GPP PS Data Off Status |  | 3GPP PS Data Off Status |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/3gppPSDataOffStatus |  |
| UPF ID |  | UPF ID |  | /roamingQBCInformation/uPFID |  |
| Roaming Charging Profile |  | Roaming Charging Profile |  | /roamingQBCInformation/roamingChargingProfile |  |
| Trigger |  | Trigger |  | /roamingQBCInformationroamingChargingProfile/trigger |  |
| Partial record method |  | Partial record method |  | /roamingQBCInformation/roamingChargingProfile/partialRecordMethod |  |
|  |  |  |  | ChargingDataResponse |  |
| Multiple Unit information |  | - |  | /multipleUnitInformation |  |
| UPF ID |  | - |  | /multipleUnitInformation/uPFID |  |

3rd change to TS 32.291

A.2	Nchf_ConvergedCharging API

openapi: 3.0.0

info:

title: Nchf_ConvergedCharging

version: 3.0.0.alpha-3

description: |

ConvergedCharging Service    © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 32.291 V16.3.0: Telecommunication management; Charging management;

5G system, charging service; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/32_series/32.291/'

servers:

- url: '{apiRoot}/nchf-convergedcharging/v2'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501.

paths:

/chargingdata:

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifyUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingNotifyRequest'

responses:

'204':

description: 'No Content, Notification was succesfull'

'400':

description: Bad request

content:

application/json:

schema:

$ref: >-

TS29571_CommonData.yaml#/components/schemas/ProblemDetails

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/chargingdata/{ChargingDataRef}/update':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: ChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'200':

description: OK. Updated Charging Data resource is returned

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/chargingdata/{ChargingDataRef}/release':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: ChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'204':

description: No Content.

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

ChargingDataRequest:

type: object

properties:

subscriberIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

nfConsumerIdentification:

$ref: '#/components/schemas/NFIdentification'

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

oneTimeEvent:

type: boolean

oneTimeEventType:

$ref: '#/components/schemas/oneTimeEventType'

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

serviceSpecificationInfo:

type: string

multipleUnitUsage:

type: array

items:

$ref: '#/components/schemas/MultipleUnitUsage'

minItems: 0

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

sMSChargingInformation:

$ref: '#/components/schemas/SMSChargingInformation'

nEFChargingInformation:

$ref: '#/components/schemas/NEFChargingInformation'

registrationChargingInformation:

$ref: '#/components/schemas/RegistrationChargingInformation'

n2ConnectionChargingInformation:

$ref: '#/components/schemas/N2ConnectionChargingInformation'

locationReportingChargingInformation:

$ref: '#/components/schemas/LocationReportingChargingInformation'

required:

- nfConsumerIdentification

- invocationTimeStamp

- invocationSequenceNumber

ChargingDataResponse:

type: object

properties:

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

invocationResult:

$ref: '#/components/schemas/InvocationResult'

sessionFailover:

$ref: '#/components/schemas/SessionFailover'

multipleUnitInformation:

type: array

items:

$ref: '#/components/schemas/MultipleUnitInformation'

minItems: 0

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

required:

- invocationTimeStamp

- invocationSequenceNumber

ChargingNotifyRequest:

type: object

properties:

notificationType:

$ref: '#/components/schemas/NotificationType'

reauthorizationDetails:

type: array

items:

$ref: '#/components/schemas/ReauthorizationDetails'

minItems: 0

required:

- notificationType

ChargingNotifyResponse:

type: object

properties:

invocationResult:

$ref: '#/components/schemas/InvocationResult'

NFIdentification:

type: object

properties:

nFName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nFIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

nFIPv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

nFPLMNID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

nodeFunctionality:

$ref: '#/components/schemas/NodeFunctionality'

nFFqdn:

type: string

required:

- nodeFunctionality

MultipleUnitUsage:

type: object

properties:

ratingGroup:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatingGroup'

requestedUnit:

$ref: '#/components/schemas/RequestedUnit'

usedUnitContainer:

type: array

items:

$ref: '#/components/schemas/UsedUnitContainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

required:

- ratingGroup

InvocationResult:

type: object

properties:

error:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

failureHandling:

$ref: '#/components/schemas/FailureHandling'

Trigger:

type: object

properties:

triggerType:

$ref: '#/components/schemas/TriggerType'

triggerCategory:

$ref: '#/components/schemas/TriggerCategory'

timeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

volumeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

volumeLimit64:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

maxNumberOfccc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

required:

- triggerType

- triggerCategory

MultipleUnitInformation:

type: object

properties:

resultCode:

$ref: '#/components/schemas/ResultCode'

ratingGroup:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatingGroup'

grantedUnit:

$ref: '#/components/schemas/GrantedUnit'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

quotaHoldingTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

finalUnitIndication:

$ref: '#/components/schemas/FinalUnitIndication'

timeQuotaThreshold:

type: integer

volumeQuotaThreshold:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

unitQuotaThreshold:

type: integer

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

required:

- ratingGroup

RequestedUnit:

type: object

properties:

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

UsedUnitContainer:

type: object

properties:

serviceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceId'

quotaManagementIndicator:

$ref: '#/components/schemas/QuotaManagementIndicator'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

eventTimeStamps:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

localSequenceNumber:

type: integer

pDUContainerInformation:

$ref: '#/components/schemas/PDUContainerInformation'

required:

- localSequenceNumber

GrantedUnit:

type: object

properties:

tariffTimeChange:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

FinalUnitIndication:

type: object

properties:

finalUnitAction:

$ref: '#/components/schemas/FinalUnitAction'

restrictionFilterRule:

$ref: '#/components/schemas/IPFilterRule'

filterId:

type: string

redirectServer:

$ref: '#/components/schemas/RedirectServer'

required:

- finalUnitAction

RedirectServer:

type: object

properties:

redirectAddressType:

$ref: '#/components/schemas/RedirectAddressType'

redirectServerAddress:

type: string

required:

- redirectAddressType

- redirectServerAddress

ReauthorizationDetails:

type: object

properties:

serviceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceId'

ratingGroup:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatingGroup'

quotaManagementIndicator:

$ref: '#/components/schemas/QuotaManagementIndicator'

PDUSessionChargingInformation:

type: object

properties:

chargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

homeProvidedChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

pduSessionInformation:

$ref: '#/components/schemas/PDUSessionInformation'

unitCountInactivityTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'
        rANSecondaryRATUsageReport:

$ref: '#/components/schemas/RANSecondaryRATUsageReport'

required:

- pduSessionInformation

UserInformation:

type: object

properties:

servedGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

servedPEI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

unauthenticatedFlag:

type: boolean

roamerInOut:

$ref: '#/components/schemas/RoamerInOut'

PDUSessionInformation:

type: object

properties:

networkSlicingInfo:

$ref: '#/components/schemas/NetworkSlicingInfo'

pduSessionID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

pduType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

hPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

servingNetworkFunctionID:

$ref: '#/components/schemas/ServingNetworkFunctionID'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

dnnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

dnnSelectionMode:

$ref: '#/components/schemas/dnnSelectionMode'

chargingCharacteristics:

type: string

chargingCharacteristicsSelectionMode:

$ref: '#/components/schemas/ChargingCharacteristicsSelectionMode'

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sessionStopIndicator:

type: boolean

pduAddress:

$ref: '#/components/schemas/PDUAddress'

diagnostics:

$ref: '#/components/schemas/Diagnostics'

authorizedQoSInformation:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/AuthorizedDefaultQos'

subscribedQoSInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SubscribedDefaultQos'

authorizedSessionAMBR:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

subscribedSessionAMBR:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

servingCNPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

required:

- pduSessionID

- dnnId

PDUContainerInformation:

type: object

properties:

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosData'

qoSCharacteristics:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosCharacteristics'

afChargingIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNodeID:

type: array

items:

$ref: '#/components/schemas/ServingNetworkFunctionID'

minItems: 0

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sponsorIdentity:

type: string

applicationserviceProviderIdentity:

type: string

chargingRuleBaseName:

type: string

usedPduAddress:

$ref: '#/components/schemas/PDUAddress'

NetworkSlicingInfo:

type: object

properties:

sNSSAI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- sNSSAI

PDUAddress:

type: object

properties:

pduIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

pduIPv6AddresswithPrefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

pduAddressprefixlength:

type: integer

iPv4dynamicAddressFlag:

type: boolean

iPv6dynamicPrefixFlag:

type: boolean

ServingNetworkFunctionID:

type: object

properties:

servingNetworkFunctionInformation:

$ref: '#/components/schemas/NFIdentification'

aMFId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfId'

required:

- servingNetworkFunctionInformation

RoamingQBCInformation:

type: object

properties:

multipleQFIcontainer:

type: array

items:

$ref: '#/components/schemas/MultipleQFIcontainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

roamingChargingProfile:

$ref: '#/components/schemas/RoamingChargingProfile'

MultipleQFIcontainer:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

localSequenceNumber:

type: integer

qFIContainerInformation:

$ref: '#/components/schemas/QFIContainerInformation'

required:

- localSequenceNumber

QFIContainerInformation:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

reportTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosData'

qoSCharacteristics:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosCharacteristics'

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNetworkFunctionID:

type: array

items:

$ref: '#/components/schemas/ServingNetworkFunctionID'

minItems: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

3gppChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

diagnostics:

$ref: '#/components/schemas/Diagnostics'

enhancedDiagnostics:

type: array

items:

type: string

required:

- reportTime

RoamingChargingProfile:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

partialRecordMethod:

$ref: '#/components/schemas/PartialRecordMethod'

SMSChargingInformation:

type: object

properties:

originatorInfo:

$ref: '#/components/schemas/OriginatorInfo'

recipientInfo:

type: array

items:

$ref: '#/components/schemas/RecipientInfo'

minItems: 0

userEquipmentInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

sMSCAddress:

type: string

sMDataCodingScheme:

type: integer

sMMessageType:

$ref: '#/components/schemas/SMMessageType'

sMReplyPathRequested:

$ref: '#/components/schemas/ReplyPathRequested'

sMUserDataHeader:

type: string

sMStatus:

type: string

sMDischargeTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

numberofMessagesSent:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

sMServiceType:

$ref: '#/components/schemas/SMServiceType'

sMSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

sMSresult:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

submissionTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

sMPriority:

$ref: '#/components/schemas/SMPriority'

messageReference:

type: string

messageSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

messageClass:

$ref: '#/components/schemas/MessageClass'

deliveryReportRequested:

$ref: '#/components/schemas/DeliveryReportRequested'

OriginatorInfo:

type: object

properties:

originatorSUPI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

originatorGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

originatorOtherAddress:

$ref: '#/components/schemas/SMAddressInfo'

originatorReceivedAddress:

$ref: '#/components/schemas/SMAddressInfo'

originatorSCCPAddress:

type: string

sMOriginatorInterface:

$ref: '#/components/schemas/SMInterface'

sMOriginatorProtocolId:

type: string

RecipientInfo:

type: object

properties:

recipientSUPI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

recipientGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

recipientOtherAddress:

$ref: '#/components/schemas/SMAddressInfo'

recipientReceivedAddress:

$ref: '#/components/schemas/SMAddressInfo'

recipientSCCPAddress:

type: string

sMDestinationInterface:

$ref: '#/components/schemas/SMInterface'

sMrecipientProtocolId:

type: string

SMAddressInfo:

type: object

properties:

sMaddressType:

$ref: '#/components/schemas/SMAddressType'

sMaddressData:

type: string

sMaddressDomain:

$ref: '#/components/schemas/SMAddressDomain'

RecipientAddress:

type: object

properties:

recipientAddressInfo:

$ref: '#/components/schemas/SMAddressInfo'

sMaddresseeType:

$ref: '#/components/schemas/SMAddresseeType'

MessageClass:

type: object

properties:

classIdentifier:

$ref: '#/components/schemas/ClassIdentifier'

tokenText:

type: string

SMAddressDomain:

type: object

properties:

domainName:

type: string

3GPPIMSIMCCMNC:

type: string

SMInterface:

type: object

properties:

interfaceId:

type: string

interfaceText:

type: string

interfacePort:

type: string

interfaceType:

$ref: '#/components/schemas/InterfaceType'

RANSecondaryRATUsageReport:

type: object

properties:

rANSecondaryRATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

qosFlowsUsageReports:

type: array

items:

$ref: '#/components/schemas/QosFlowsUsageReport'

Diagnostics:

type: integer

IPFilterRule:

type: string

QosFlowsUsageReport:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

startTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

NEFChargingInformation:

type: object

properties:

groupIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

aPIDirection:

$ref: '#/components/schemas/APIDirection'

aPITargetNetworkFunction:

$ref: '#/components/schemas/NFIdentification'

aPIResultCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

aPIName:

type: string

aPIReference:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

aPIContent:

type: string

required:

- aPIName

RegistrationChargingInformation:

type: object

properties:

registrationMessagetype:

$ref: '#/components/schemas/RegistrationMessageType'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

5GMMCapability:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

mICOModeIndication:

$ref: '#/components/schemas/MICOModeIndication'

smsIndication:

$ref: '#/components/schemas/SmsIndication'

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 0

serviceAreaRestriction:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceAreaRestriction'

minItems: 0

requestedNSSAI:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 0

allowedNSSAI:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 0

rejectedNSSAI:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 0

required:

- registrationMessagetype

N2ConnectionChargingInformation:

type: object

properties:

n2ConnectionMessageType:

$ref: '#/components/schemas/N2ConnectionMessageType'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

amfUeNgapId:

type: integer

ranUeNgapId:

type: integer

ranNodeId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GlobalRanNodeId'

restrictedRatList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 0

forbiddenAreaList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Area'

minItems: 0

serviceAreaRestriction:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceAreaRestriction'

minItems: 0

restrictedCnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CoreNetworkType'

minItems: 0

allowedNSSAI:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 0

rrcEstCause:

type: string

pattern: '^[0-9a-fA-F]+$'

required:

- n2ConnectionMessageType

LocationReportingChargingInformation:

type: object

properties:

locationReportingMessageType:

$ref: '#/components/schemas/LocationReportingMessageType'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

required:

- locationReportingMessageType

N2ConnectionMessageType:

type: integer

LocationReportingMessageType:

type: integer

NotificationType:

anyOf:

- type: string

enum:

- REAUTHORIZATION

- ABORT_CHARGING

- type: string

NodeFunctionality:

anyOf:

- type: string

enum:

- SMF

- SMSF

- SGW

- I_SMF

- type: string

ChargingCharacteristicsSelectionMode:

anyOf:

- type: string

enum:

- HOME_DEFAULT

- ROAMING_DEFAULT

- VISITING_DEFAULT

- type: string

TriggerType:

anyOf:

- type: string

enum:

- QUOTA_THRESHOLD

- QHT

- FINAL

- QUOTA_EXHAUSTED

- VALIDITY_TIME

- OTHER_QUOTA_TYPE

- FORCED_REAUTHORISATION

- UNUSED_QUOTA_TIMER # Included for backwards compatibility, shall not be used

- UNIT_COUNT_INACTIVITY_TIMER

- ABNORMAL_RELEASE

- QOS_CHANGE

- VOLUME_LIMIT

- TIME_LIMIT

- PLMN_CHANGE

- USER_LOCATION_CHANGE

- RAT_CHANGE

- SESSION_AMBR_CHANGE

- UE_TIMEZONE_CHANGE

- TARIFF_TIME_CHANGE

- MAX_NUMBER_OF_CHANGES_IN_CHARGING_CONDITIONS

- MANAGEMENT_INTERVENTION

- CHANGE_OF_UE_PRESENCE_IN_PRESENCE_REPORTING_AREA

- CHANGE_OF_3GPP_PS_DATA_OFF_STATUS

- SERVING_NODE_CHANGE

- REMOVAL_OF_UPF

- ADDITION_OF_UPF

- INSERTION_OF_ISMF

- REMOVAL_OF_ISMF

- CHANGE_OF_ISMF

- START_OF_SERVICE_DATA_FLOW

- ECGI_CHANGE

- TAI_CHANGE

- HANDOVER_CANCEL

- HANDOVER_START

- HANDOVER_COMPLETE

- GFBR_GUARANTEED_STATUS_CHANGE

- type: string

FinalUnitAction:

anyOf:

- type: string

enum:

- TERMINATE

- REDIRECT

- RESTRICT_ACCESS

- type: string

RedirectAddressType:

anyOf:

- type: string

enum:

- IPV4

- IPV6

- URL

- type: string

TriggerCategory:

anyOf:

- type: string

enum:

- IMMEDIATE_REPORT

- DEFERRED_REPORT

- type: string

QuotaManagementIndicator:

anyOf:

- type: string

enum:

- ONLINE_CHARGING

- OFFLINE_CHARGING

- type: string

FailureHandling:

anyOf:

- type: string

enum:

- TERMINATE

- CONTINUE

- RETRY_AND_TERMINATE

- type: string

SessionFailover:

anyOf:

- type: string

enum:

- FAILOVER_NOT_SUPPORTED

- FAILOVER_SUPPORTED

- type: string

3GPPPSDataOffStatus:

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

ResultCode:

anyOf:

- type: string

enum:

- SUCCESS

- END_USER_SERVICE_DENIED

- QUOTA_MANAGEMENT_NOT_APPLICABLE

- QUOTA_LIMIT_REACHED

- END_USER_SERVICE_REJECTED

- USER_UNKNOWN

- RATING_FAILED

- type: string

PartialRecordMethod:

anyOf:

- type: string

enum:

- DEFAULT

- INDIVIDUAL

- type: string

RoamerInOut:

anyOf:

- type: string

enum:

- IN_BOUND

- OUT_BOUND

- type: string

SMMessageType:

anyOf:

- type: string

enum:

- SUBMISSION

- DELIVERY_REPORT

- SM_SERVICE_REQUEST

- type: string

SMPriority:

anyOf:

- type: string

enum:

- LOW

- NORMAL

- HIGH

- type: string

DeliveryReportRequested:

anyOf:

- type: string

enum:

- YES

- NO

- type: string

InterfaceType:

anyOf:

- type: string

enum:

- UNKNOWN

- MOBILE_ORIGINATING

- MOBILE_TERMINATING

- APPLICATION_ORIGINATING

- APPLICATION_TERMINATING

- type: string

ClassIdentifier:

anyOf:

- type: string

enum:

- PERSONAL

- ADVERTISEMENT

- INFORMATIONAL

- AUTO

- type: string

SMAddressType:

anyOf:

- type: string

enum:

- EMAIL_ADDRESS

- MSISDN

- IPV4_ADDRESS

- IPV6_ADDRESS

- NUMERIC_SHORTCODE

- ALPHANUMERIC_SHORTCODE

- OTHER

- IMSI

- type: string

SMAddresseeType:

anyOf:

- type: string

enum:

- TO

- CC

- BCC

- type: string

SMServiceType:

anyOf:

- type: string

enum:

- VAS4SMS_SHORT_MESSAGE_CONTENT_PROCESSING

- VAS4SMS_SHORT_MESSAGE_FORWARDING

- VAS4SMS_SHORT_MESSAGE_FORWARDING_MULTIPLE_SUBSCRIPTIONS

- VAS4SMS_SHORT_MESSAGE_FILTERING

- VAS4SMS_SHORT_MESSAGE_RECEIPT

- VAS4SMS_SHORT_MESSAGE_NETWORK_STORAGE

- VAS4SMS_SHORT_MESSAGE_TO_MULTIPLE_DESTINATIONS

- VAS4SMS_SHORT_MESSAGE_VIRTUAL_PRIVATE_NETWORK(VPN)

- VAS4SMS_SHORT_MESSAGE_AUTO_REPLY

- VAS4SMS_SHORT_MESSAGE_PERSONAL_SIGNATURE

- VAS4SMS_SHORT_MESSAGE_DEFERRED_DELIVERY

- type: string

ReplyPathRequested:

anyOf:

- type: string

enum:

- NO_REPLY_PATH_SET

- REPLY_PATH_SET

- type: string

oneTimeEventType:

anyOf:

- type: string

enum:

- IEC

- PEC

- type: string

dnnSelectionMode:

anyOf:

- type: string

enum:

- VERIFIED

- UE_DNN_NOT_VERIFIED

- NW_DNN_NOT_VERIFIED

- type: string

APIDirection:

anyOf:

- type: string

enum:

- INVOCATION

- NOTIFICATION

- type: string

RegistrationMessageType:

anyOf:

- type: string

enum:

- INITIAL

- MOBILITY

- PERIODIC

- EMERGENCY

- DEREGISTRATION

- type: string

MICOModeIndication:

anyOf:

- type: string

enum:

- MICO_MODE

- NO_MICO_MODE

- type: string

SmsIndication:

anyOf:

- type: string

enum:

- SMS_SUPPORTED

- SMS_NOT_SUPPORTED

- type: string

4th change to TS 32.291

A.3	Nchf_ OfflineOnlyCharging API

openapi: 3.0.0

info:

title: Nchf_OfflineOnlyCharging

version: 1.0.0.alpha-3

description: |

OfflineOnlyCharging Service

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 32.291 V16.3.0: Telecommunication management; Charging management;

5G system, charging service; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/32_series/32.291/'

servers:

- url: '{apiRoot}/nchf-offlineonlycharging/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501.

paths:

/offlinechargingdata:

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/offlinechargingdata/{OfflineChargingDataRef}/update':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: OfflineChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'200':

description: OK. Updated Charging Data resource is returned

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/offlinechargingdata/{OfflineChargingDataRef}/release':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: OfflineChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'204':

description: No Content.

'404':

description: Not Found

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

ChargingDataRequest:

type: object

properties:

subscriberIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

nfConsumerIdentification:

$ref: '#/components/schemas/NFIdentification'

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

oneTimeEvent:

type: boolean

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

multipleUnitUsage:

type: array

items:

$ref: '#/components/schemas/MultipleUnitUsage'

minItems: 0

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

required:

- nfIdentification

- invocationTimeStamp

- invocationSequenceNumber

ChargingDataResponse:

type: object

properties:

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

invocationResult:

$ref: '#/components/schemas/InvocationResult'

sessionFailover:

$ref: '#/components/schemas/SessionFailover'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

required:

- invocationTimeStamp

- invocationSequenceNumber

NFIdentification:

type: object

properties:

nFName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nFIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

nFIPv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

nFPLMNID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

nodeFunctionality:

$ref: '#/components/schemas/NodeFunctionality'

nFFqdn:

type: string

required:

- nodeFunctionality

MultipleUnitUsage:

type: object

properties:

ratingGroup:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatingGroup'

usedUnitContainer:

type: array

items:

$ref: '#/components/schemas/UsedUnitContainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

required:

- ratingGroup

InvocationResult:

type: object

properties:

error:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

failureHandling:

$ref: '#/components/schemas/FailureHandling'

Trigger:

type: object

properties:

triggerType:

$ref: '#/components/schemas/TriggerType'

triggerCategory:

$ref: '#/components/schemas/TriggerCategory'

timeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

volumeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

volumeLimit64:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

maxNumberOfccc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

required:

- triggerType

- triggerCategory

UsedUnitContainer:

type: object

properties:

serviceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceId'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

eventTimeStamps:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

localSequenceNumber:

type: integer

pDUContainerInformation:

$ref: '#/components/schemas/PDUContainerInformation'

required:

- localSequenceNumber

PDUSessionChargingInformation:

type: object

properties:

chargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

userLocationTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

pduSessionInformation:

$ref: '#/components/schemas/PDUSessionInformation'

unitCountInactivityTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'
        rANSecondaryRATUsageReport:

$ref: '#/components/schemas/RANSecondaryRATUsageReport'

required:

- pduSessionInformation

UserInformation:

type: object

properties:

servedGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

servedPEI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

unauthenticatedFlag:

type: boolean

roamerInOut:

$ref: '#/components/schemas/RoamerInOut'

PDUSessionInformation:

type: object

properties:

networkSlicingInfo:

$ref: '#/components/schemas/NetworkSlicingInfo'

pduSessionID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

pduType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

hPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

servingNetworkFunctionID:

$ref: '#/components/schemas/ServingNetworkFunctionID'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

dnnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

chargingCharacteristics:

type: string

chargingCharacteristicsSelectionMode:

$ref: '#/components/schemas/ChargingCharacteristicsSelectionMode'

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sessionStopIndicator:

type: boolean

pduAddress:

$ref: '#/components/schemas/PDUAddress'

diagnostics:

$ref: '#/components/schemas/Diagnostics'

authorizedQoSInformation:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/AuthorizedDefaultQos'

subscribedQoSInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SubscribedDefaultQos'

authorizedSessionAMBR:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

subscribedSessionAMBR:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

servingCNPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

required:

- pduSessionID

- dnnId

PDUContainerInformation:

type: object

properties:

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosData'

qoSCharacteristics:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosCharacteristics'

aFCorrelationInformation:

type: string

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNodeID:

type: array

items:

$ref: '#/components/schemas/ServingNetworkFunctionID'

minItems: 0

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sponsorIdentity:

type: string

applicationserviceProviderIdentity:

type: string

chargingRuleBaseName:

type: string

UsedPduAddress:

$ref: '#/components/schemas/PDUAddress'

NetworkSlicingInfo:

type: object

properties:

sNSSAI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- sNSSAI

PDUAddress:

type: object

properties:

pduIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

pduIPv6AddresswithPrefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

pduAddressprefixlength:

type: integer

iPv4dynamicAddressFlag:

type: boolean

iPv6dynamicPrefixFlag:

type: boolean

ServingNetworkFunctionID:

type: object

properties:

servingNetworkFunctionInformation:

$ref: '#/components/schemas/NFIdentification'

aMFId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfId'

required:

- servingNetworkFunctionInformation

RoamingQBCInformation:

type: object

properties:

multipleQFIcontainer:

type: array

items:

$ref: '#/components/schemas/MultipleQFIcontainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

roamingChargingProfile:

$ref: '#/components/schemas/RoamingChargingProfile'

MultipleQFIcontainer:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

localSequenceNumber:

type: integer

qFIContainerInformation:

$ref: '#/components/schemas/QFIContainerInformation'

required:

- localSequenceNumber

QFIContainerInformation:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosData'

qoSCharacteristics:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosCharacteristics'

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNetworkFunctionID:

type: array

items:

$ref: '#/components/schemas/ServingNetworkFunctionID'

minItems: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

RoamingChargingProfile:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

partialRecordMethod:

$ref: '#/components/schemas/PartialRecordMethod'

RANSecondaryRATUsageReport:

type: object

properties:

rANSecondaryRATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

qosFlowsUsageReports:

type: array

items:

$ref: '#/components/schemas/QosFlowsUsageReport'

Diagnostics:

type: integer

IPFilterRule:

type: string

QosFlowsUsageReport:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

startTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

NodeFunctionality:

anyOf:

- type: string

enum:

- SMF

- SMSF

- type: string

ChargingCharacteristicsSelectionMode:

anyOf:

- type: string

enum:

- HOME_DEFAULT

- ROAMING_DEFAULT

- VISITING_DEFAULT

- type: string

TriggerType:

anyOf:

- type: string

enum:

- FINAL

- ABNORMAL_RELEASE

- QOS_CHANGE

- VOLUME_LIMIT

- TIME_LIMIT

- PLMN_CHANGE

- USER_LOCATION_CHANGE

- RAT_CHANGE

- SESSION_AMBR_CHANGE

- UE_TIMEZONE_CHANGE

- TARIFF_TIME_CHANGE

- MAX_NUMBER_OF_CHANGES_IN_CHARGING_CONDITIONS

- MANAGEMENT_INTERVENTION

- CHANGE_OF_UE_PRESENCE_IN_PRESENCE_REPORTING_AREA

- CHANGE_OF_3GPP_PS_DATA_OFF_STATUS

- SERVING_NODE_CHANGE

- REMOVAL_OF_UPF

- ADDITION_OF_UPF

- INSERTION_OF_ISMF

- REMOVAL_OF_ISMF

- CHANGE_OF_ISMF

- START_OF_SERVICE_DATA_FLOW

- GFBR_GUARANTEED_STATUS_CHANGE

- type: string

TriggerCategory:

anyOf:

- type: string

enum:

- IMMEDIATE_REPORT

- DEFERRED_REPORT

- type: string

FailureHandling:

anyOf:

- type: string

enum:

- TERMINATE

- CONTINUE

- RETRY_AND_TERMINATE

- type: string

SessionFailover:

anyOf:

- type: string

enum:

- FAILOVER_NOT_SUPPORTED

- FAILOVER_SUPPORTED

- type: string

3GPPPSDataOffStatus:

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

ResultCode:

anyOf:

- type: string

enum:

- SUCCESS

- END_USER_SERVICE_DENIED

- type: string

PartialRecordMethod:

anyOf:

- type: string

enum:

- DEFAULT

- INDIVIDUAL

- type: string

RoamerInOut:

anyOf:

- type: string

enum:

- IN_BOUND

- OUT_BOUND

- type: string

End of Change