| 3GPP TS 28.202 V0.1.0 (2020-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Network slice management charging in the 5G System (5GS); Stage 2 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Architecture considerations	7

4.1	High level architecture	7

4.2	Network Slice Management converged charging architecture	7

4.2.1	High level architecture network slice charging	7

4.2.2	Converged charging architecture	8

5	Network Slice Management charging principles and scenarios	8

5.1	Network Slice Management charging principles	8

5.1.1	General	8

5.1.2	Requirements	9

5.1.3	Network Slice Management charging information	9

5.2	Network Slice Management charging scenarios	9

5.2.1	Basic principles	9

5.2.1.1	General	9

5.2.1.2	Applicable triggers in Network Slice Management charging	10

5.2.2	Message flows	10

5.2.2.1	General	10

5.2.2.2	NSI Allocation - PEC	10

5.2.2.3	NSI Modification - PEC	11

5.2.2.4	NSI Deallocation - PEC	12

5.2.3	CDR generation	13

5.2.3.1	Introduction	13

5.2.3.2	Triggers for CHF CDR	13

5.2.3.2.1	General	13

5.2.3.2.2	Triggers for CHF CDR generation	13

5.2.4	Ga record transfer flows	14

5.2.5	Bnsm CDR file transfer	14

6	Definition of charging information	14

6.1	Data description for network slice management charging	14

6.1.1	Message contents	14

6.1.1.1	General	14

6.1.1.2	Charging Data Request message	14

6.1.1.3	Charging data response message	15

6.2	Network Slice management charging specific parameters	15

6.2.1	Definition of Network Slice management charging information	15

6.2.1.1	General	15

6.2.1.2	Definition of NS Management charging information	15

Annex <X> (informative): Change history	16

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the Converged Charging description for network slice management charging in the 5G System (5GS). The document specifies the charging aspects for management of network slicing in mobile networks.

The following management operations are within the scope:

-	Network Slice Instance creation, modification and termination;

-	Network Slice Instance activation and deactivation.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles ".

[2] - [49]	Void

[50]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[51]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[51] - [54]	Void.

[55]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[56]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[57]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[58] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101] - [199]	Void.

[200] - [249]	Void.

[250]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[251]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[252]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[253]	3GPP TS 28.532: "Management and orchestration; Management services".

[254]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[255] - [299]	Void.

[300] - [399]	Void.

[400] - [499]	Void.

[500] - [599]	Void.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bnsm	Reference point for the CDR file transfer from the network slice management CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and the CGF.

Nchf	Service based interface exhibited by CHF.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MnS	Management Service

NSI	Network Slice Instance

NSSAI	Network Slice Selection Assistance Information

S-NSSAI	Single Network Slice Selection Assistance Information

# 4	Architecture considerations

## 4.1	High level architecture

The high level service based management architecture for Network Slicing management is specified in TS 28.533 [250] clause 4, with Management Services (MnS) produced by MnS Producer and consumed by MnS Consumer.

## 4.2	Network Slice Management converged charging architecture

### 4.2.1	High level architecture network slice charging

The scope of the present document is network slice management charging, the details for other types of network slice charging are defined in TS 32.255 [x] and TS 28.201 [x].

### 4.2.2	Converged charging architecture

Network Slice Management converged charging can be achieved under the two alternative options of the architecture depicted in figure 4.2.2.1.

Figure 4.2.2.1: Network Slice Management converged charging architecture

The Charging Trigger Function is specified in TS 32.240 [1].

The enhanced Charging Trigger Function (eCTF) encompasses following functionalities:

-	Charging Trigger Function as specified in TS 32.240 [1].

-	Nchf consumer.

-	MnS consumer.

The Service Producer(s) represent the MnS producer(s) of the management services specified in TS 28.532 [253] which are used for Network Slicing Management.

Details on the interfaces and functions can be found in TS 32.240 [1] for the general architecture components. Ga is described in clause 5.2.4 and Bnsm in clause 5.2.5 of the present document, and Nchf is described in TS 32.290 [50].

Editor's Note: the relationship between the eCTF and CSIF as well as naming are ffs

# 5	Network Slice Management charging principles and scenarios

## 5.1	Network Slice Management charging principles

### 5.1.1	General

The charging functions for the Network Slice Management charging, are based on provisioning management services used for provisioning of network slice.

The management services are specified under TS 28.533 [250], TS 28.532 [253] and TS 28.541 [254]. Management services and procedures for provisioning are described TS 28.531 [252].

How a MnS consumer consumes and is authorized to consume provisioning MnS produced by the MnS producer, are out of the scope of this document.

The subscriber is the MnS consumer of the provisioning MnS.

Editor's Note: depending on the model, whether the subscriber is always the MnS consumer is ffs.

The following provisioning of network slice procedures of clause 7 in TS 28.531 [252] are considered for Network Slice Management charging:

-	Network Slice Instance (NSI) Allocation.

-	Network Slice Instance (NSI) Modification.

-	Network Slice Instance (NSI) Deallocation.

Editor’s note: identification for a network slice instance and relationship with S-NSSAI needs to be clarified.

### 5.1.2	Requirements

The following are high-level charging requirements specific to the Network Slice Management charging:

-	The eCTF shall support converged charging.

-	The eCTF shall collect charging information per NSI Allocation.

-	The eCTF shall collect charging information per NSI Modification.

-	The eCTF shall collect charging information per NSI Deallocation.

### 5.1.3	Network Slice Management charging information

The charging information for Network Slice Management charging are defined under the following list:

-	identity of the MnS consumer of provisioning MnS.

-	operation type invoked by MnS consumer of provisioning MnS.

-	identifier of the Network Slice Instance.

-	network slice related requirements identified by a service profile as defined in TS 28.531 [252] clause 4.5. and detailed in TS 28.541 [254] clause 6.3.3.

Editor's Note: the list of charging information needs to be completed.

## 5.2	Network Slice Management charging scenarios

### 5.2.1	Basic principles

#### 5.2.1.1	General

Network Slice Management charging is performed by the MnS producer of provisioning MnS interacting with CHF using Nchf specified in TS 32.290 [50] and TS 32.291 [51].In order to provide the data required for the charging management activities outlined in TS 32.240 [1], the MnS producer shall be able to perform converged charging for provisioning MnS when consumed by MnS consumer for NSI allocation, modification and deallocation.

The MnS producer of provisioning MnS shall be able to perform converged charging by interacting with CHF, for charging data related to Network Slice Management. The Charging Data Request and Charging Data Response are exchanged between the MnS producer and the CHF, based on PEC scenarios specified in TS 32.290 [50]. The Charging Data Request is issued by the MnS producer towards the CHF when certain conditions (chargeable events) are met.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in clauses 5.2.2.

A detailed formal description of the converged charging parameters to CHF defined in the present document can be found in TS 32.291 [51].

A detailed formal description of the CDR parameters defined in the present document can be found in TS 32.298 [57].

Editor’s Note: the case where there is no internal CTF in the MnS producer needs to be reflected in the text.

#### 5.2.1.2	Applicable triggers in Network Slice Management charging

A set of trigger conditions are defined for the MnS producer to invoke a Charging Data Request [Event] towards the CHF.

Table 5.2.1.2.1 summarizes the set of default trigger conditions and their category which shall be supported by the MnS producer.

Table 5.2.1.2.1: Default Trigger conditions in MnS producer

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| NSI Allocation Response | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| NSI Modification Response | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| NSI Deallocation Response | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event] issued by the MnS producer for these chargeable events, is specified in clause 5.2.3.

Editor’s Note: the case where there is no internal CTF in the MnS producer needs to be considered.

### 5.2.2	Message flows

#### 5.2.2.1	General

The flows in the present document specify the interaction between the MnS producer and the CHF for Network Slice Management charging functionality, in different scenarios, based on TS 28.531 [252] clause 7 procedures and flows.

Editor’s Note: flows for the case where there is no internal CTF in the MnS producer need to be considered.

#### 5.2.2.2	NSI Allocation - PEC

Figure 5.2.2.2.1 describes the scenario where a NSI Allocation request is invoked towards a MnS producer in PEC mode

Figure 5.2.2.2.1: NSI allocation - PEC

1.	MnS producer receives an NSI Allocation Request from an MnS consumer.

2.	MnS producer performs NSI Allocation process by creating a new NSI or using an existing NSI.

3.	The MnS producer sends the NSI Allocation Response with appropriate result.

3ch-a. The MnS producer sends Charging Data Request [Event] to CHF associated to the NSI Allocation transaction.

3ch-b. The CHF creates a CDR for this NSI Allocation transaction.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the MnS producer.

#### 5.2.2.3	NSI Modification - PEC

Figure 5.2.2.3.1 describes the scenario where a NSI Modification request is invoked towards a MnS producer in PEC mode

Figure 5.2.2.3.1: NSI Modification - PEC

1.	MnS producer receives an NSI Modification Request from an MnS consumer.

2.	MnS producer performs NSI Modification process.

3.	The MnS producer sends the NSI Modification Response with appropriate result.

3ch-a. The MnS producer sends Charging Data Request [Event] to CHF associated to the NSI Modification transaction.

3ch-b. The CHF creates a CDR for this NSI Modification transaction.

3ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the MnS producer.

#### 5.2.2.4	NSI Deallocation - PEC

Figure 5.2.2.4.1 describes the scenario where a NSI Deallocation request is invoked towards a MnS producer in PEC mode

Figure 5.2.2.4.1: NSI Deallocation - PEC

1.	MnS producer receives an NSI Deallocation Request from an MnS consumer.

2.	The MnS producer sends the NSI Deallocation Response.

2ch-a. The MnS producer sends Charging Data Request [Event] to CHF associated to the NSI Deallocation transaction.

2ch-b. The CHF creates a CDR for this NSI Deallocation transaction.

2ch-c. The CHF acknowledges by sending Charging Data Response [Event] to the MnS producer.

3.	MnS producer performs the NSI Deallocation process.

### 5.2.3	CDR generation

#### 5.2.3.1	Introduction

The CHF CDRs for Network slice management charging are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The following clauses describe in detail the conditions for generating the CHF CDR, which shall be supported by the CHF.

#### 5.2.3.2	Triggers for CHF CDR

##### 5.2.3.2.1	General

A Network slice management charging CHF CDR is used to capture charging information related to Network slice management chargeable events.

##### 5.2.3.2.2	Triggers for CHF CDR generation

A Network slice management charging CHF CDR is generated by the CHF for each received Charging Data Request [Event].

### 5.2.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [55].

### 5.2.5	Bnsm CDR file transfer

Details of the Bnsm protocol application are specified in TS 32.297 [56].

# 6	Definition of charging information

## 6.1	Data description for network slice management charging

### 6.1.1	Message contents

#### 6.1.1.1	General

The Charging Data Request and Charging Data Response are specified in subclause 5.1.2.2.1 of TS 32.290 [50].

Table 6.1.1.1.1 describes the use of these messages for network slice management charging.

Table 6.1.1.1.1: Network slice management charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | MnS producer | CHF |
| Charging Data Response | CHF | MnS producer |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1].

Editor’s Note: the case where there is no internal CTF in the MnS producer needs to be considered.

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the MnS producer as used for Network slice management charging.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57] and holds the identifier of the MnS Consumer. |
| NF Consumer Identification | M | Described in TS 32.290 [57] and and holds the identifier of the MnS producer. |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| One-time Event | OC | Described in TS 32.290 [57] |
| One-time Event Type | OC | Described in TS 32.290 [57] |
| Notify URI | - | This field is not applicable. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Usage | - | This field is not applicable. |
| NS Management Charging information | OM | This field holds NS Management specific information described in clause 6.2.1.x |

Editor's Note: use of Multiple Unit Usage is ffs

Editor's Note: the different types of MnS consumer needs to be determined.

#### 6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for Network slice management charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | - | This field is not applicable. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Information | - | This field is not applicable. |

## 6.2	Network Slice management charging specific parameters

### 6.2.1	Definition of Network Slice management charging information

#### 6.2.1.1	General

The Charging Information parameter used for Network Slice management charging is provided in the following clauses.

#### 6.2.1.2	Definition of NS Management charging information

Specific charging information used for Network Slice Management charging is provided within the NS Management Charging Information.

The detailed structure of the NS Management Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2.1: Structure of NS Management Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Operation type | M | This field holds the operation type received by the MnS producer associated to the provisioning MnS. |
| Service Profile | OM | This field holds the service profile received by the MnS producer associated to the operation type of the provisioning MnS, as defined in TS 28.541 [254] clause 6.3.3. |

Editor's Note: use of MnS producer is still under discussion

Editor's Note: the information elements in the table are ffs.

# Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-04 | SA5#130e | S5-202426 S5-202441 S5-202442 S5-202443 S5-202444 S5-202445 S5-202446 S5-202447 S5-202448 |  |  |  | Create initial text for the Scope clause Introduce symbols abbreviations and references Introduce Architecture Introduce Charging Principles Introduce NS management Charging scenarios principles Introduce NS management charging triggers and flows Introduce CDR generation Introduce Message content Introduce NS management charging information | 0.1.0 |