- 3GPP TSG SA WG5 Meeting #130-e	S5-202042

- April 10-24, 2020	revision of S5-2abcde

- Source:	Intel

- Title:	pCR 28.313 changes to RACH optimization

- Document for:	Approval

Agenda Item:	6.4.9

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TS 28.313 v0.3.0 “Management and orchestration; Self-Organizing Networks (SON) for 5G networks”

[2] 	R3-201610 “Addition of SON features”

# 3	Rationale

In the baseline CR R3-201610 [2], RAN3 has added the following SON features to support RACH optimization:

- 15.X.3	Support for RACH Optimization

Editor’s note: This section captures the stage 2 descriptions for RACH Optimization

RACH optimization is supported by UE reported information made available at the NG RAN node and by PRACH parameters exchange between NG RAN nodes.

The contents of the RACH information report comprise of the following:

  - Contention detection indication per RACH attempt

  - Indexes of the SSBs and number of RACH preambles sent on each tried SSB listed in chronological order of attempts.

  - Indication whether the selected SSB is above or below the rsrp-ThresholdSSB threshold per RACH attempt.

This contribution proposes changes to RACH use case, requirements, information, and procedure in TS 28.313 [1] to support RAN3’s SON features.

# 4	Detailed proposal

1st modified section

## 6.1	Requirements

### 6.1.1	Distributed SON management

#### 6.1.1.1	RACH Optimization (Random Access Optimisation)

REQ-RACH-FUN-1 MnS producer should have a capability allowing the authorized consumer to set and update the targets for RACH optimization function.

REQ-RACH-FUN-2 MnS producer should have a capability allowing an authorized consumer to enable or disable the RACH optimization function.

REQ-RACH-FUN-3 MnS producer should have a capability allowing the authorized consumer to collect performance measurements that are used to evaluate the RACH performance.

Next modified section

## 6.4	Use cases

### 6.4.1	Distributed SON management

#### 6.4.1.1	RACH Optimization (Random Access Optimisation)

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To automatically configure the RACH parameters in a cell in order to achieve the optimal network performance by reducing the network access time, and minimize the failures. |  |
| Actors and Roles | D-SON management function to support RACH Optimization function. |  |
| Telecom resources | - gNB; - The producer of provisioning MnS |  |
| Assumptions | N/A |  |
| Pre-conditions | - 5G NR cells are in operation. - RACH Optimization function is in operation. |  |
| Begins when | The D-SON management function decides to enable the RACH Optimization function. |  |
| Step 1 (M) | The D-SON management function requests the producer of provisioning MnS to set the targets for the RACH optimization function. |  |
| Step 2 (M) | The D-SON management function requests the producer of provisioning MnS to enable the RACH optimization function. |  |
| Step 3 (M) | The D-SON management function collects the RACH related measurements, and analyze them to evaluate the RACH performance. |  |
| Step 4 (O) | If the D-SON management function determines that the RACH performance does not meet the target, it updates the targets for RACH optimization function; |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The RACH performance has been optimized. |  |
| Traceability | REQ-RACH-FUN-1, REQ-RACH-FUN-2, REQ-RACH-FUN-3 |  |

Next modified section

# 7	Management services for SON

## 7.1	Management services for D-SON management

### 7.1.1	RACH Optimization (Random Access Optimisation)

#### 7.1.1.1	MnS component type A

| MnS Component Type A | Note |
| --- | --- |
| Operations defined in clause 5 of TS 28.532 [3]: - getMOIAttributes operation - modifyMOIAttributes operation - notifyMOIAttributeValueChange operation | It is supported by Provisioning MnS for NF, as defined in 28.531 [11]. |
| Operations defined in clause 11.3.1.1.1 in TS 28.532 [3] and clause 6.2.3 of TS 28.550 [12]: - notifyFileReady operation - reportStreamData operation | It is supported by Performance Assurance MnS for NFs, as defined in 28.550 [12]. |

#### 7.1.2.1	MnS Component Type B definition

##### 7.1.2.1.1	Targets information

The targets of RACH optimization are shown in Table 7.1.2.1.1-1.

Table 7.1.2.1.1-1.  RACH optimization targets

| Targets | Definition | Legal Values |
| --- | --- | --- |
| UE access delay probability per SSB | The probability distribution of UE access delay that is used to minimize the access delays for the UEs under the SSBs. | CDF of access delay |
| Number of preambles send per SSB probability | The probability of the number of preambles sent per SSB. | CDF of access delay |

##### 7.1.2.1.2	Control information

The parameter is used to control the RACH optimization function.

| Control parameter | Definition | Legal Values |
| --- | --- | --- |
| RACH optimization control | This attribute allows authorized consumer to enable/disable the RACH optimization functionality. See attribute rachOptimizationControl in TS 28.541 [13]. | Boolean On, off |

##### 7.1.2.1.3	Parameters to be updated

#### 7.1.1.3	MnS Component Type C definition

##### 7.1.1.3.1	Performance measurements

Performance measurements related to the RACH optimization are captured in Table 7.1.1.3.1-1:

Table 7.1.1.3.1-1.  RACH optimization related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Distribution of RACH preambles sent | Distribution of the number of preambles UEs sent to achieve synchronization per SSB, where the number of preambles sent corresponds to PREAMBLE_TRANSMISSION_COUNTER (see clause 5.1.1 in TS 38.321 [4]) in UE. | UE access delay probability per SSB |
| Distribution of UEs access delay per SSB | Distribution of the time needed for UEs to successfully attach to the network per SSB. | Number of preambles send per SSB probability |

End of modified section