3GPP TSG-SA5 Meeting #130e	S5-202137rev

- e-meeting, 20 April – 24 April 2020												Revision of S5-20xxxx

- Source:	Huawei Technologies

- Title:	pCR TS 28.201 Add the charging data request and response

- Document for:	Approval

Agenda Item:	7.4.2

# 1	Discussion

This is a pCR to TS 28.201 adding the charging data request and response.

# 2	References

[1]	3GPP TS 28.201 "Charging management; network slice performance and analytics charging in 5G System ; Stage 2".

# 3	Rationale

This pCR proposes to introduce the charging data request and response in TS 28.201 "Charging management; network slice performance and analytics charging in 5G System ; Stage 2".

# 4	Detailed propsosal

Propose to incorporate the following change into the TS 28.201 [1].

First changes

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the CSIF as used for performance and analytics based charging.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57]. |
| Subscriber Identifier | OC | This field contains the identification of the subscriber of the network slice. |
| NF Consumer Identification | M | Described in TS 32.290 [57]. |
| NF Functionality | M | Described in TS 32.290 [57]. |
| NF Name | OC | Described in TS 32.290 [57]. |
| NF Address | OC | Described in TS 32.290 [57]. |
| NF PLMN ID | OC | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | Described in TS 32.290 [57]. |
| Invocation Sequence Number | - | This field is not applicable. |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in TS 32.290 [57]. |
| One-time Event Type | OC | Described in TS 32.290 [57]. |
| Notify URI | - | This field is not applicable. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Usage | - | This field is not applicable. |
| Network Slice Charging Information | OC | This field holds the network slice information, which is reported to the CHF. |

Editor's Note: whether the Multiple Unit Usage is applicable or not is ffs.

#### 6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CSIF as used for performance and analytics based charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57]. |
| Invocation Timestamp | M | Described in TS 32.290 [57]. |
| Invocation Result | OC | Described in TS 32.290 [57]. |
| Invocation Sequence Number | - | This field is not applicable. |
| Session Failover | OC | Described in TS 32.290 [57]. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Information | - | This field is not applicable. |

Editor's Note: the table of charging data response message content is ffs.

Next changes

## 6.2	Network slice performance and analytics charging specific parameters

### 6.2.1	Definition of network slice performance and analytics charging information

#### 6.2.1.1	General

The Charging Information parameter used for network slice performance and analytics charging is provided in the following clauses.

#### 6.2.1.2	Definition of Performance and Analytics Charging Information

Specific charging information used for network slice performance and analytics charging is provided within the Performance and Analytics Charging Information.

The detailed structure of the Performance and Analytics Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2.1: Structure of Network Slice Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Customer Identifier | OC | This field holds CSC (Communiation Service Customer) identifier if available. |
| Trigger | OC | This field holds the reason for reporting the Performance and analytics information to CHF. |
| Trigger Timestamp | OC | This field holds the timestamp of the trigger. |
| Report Start Time | OC | This field holds the reporting record beginning time. |
| Report Stop Time | OC | This field holds the reporting record endding time. |
| Network Slice Instance Assistance Identifier | OC | This field holds network slice information the performance and analytics inforamtion belongs to. |
| Performance Information | OC | This field holds the performance and analytics information. |
| Performance and Analytics Content | OC | This field holds the content of performance and analytics measurement information (e.g.latency, throughput). It may occur multiple times. |
| Start Time | OC | This field holds the begin time from which the measurement of KPI is active. |
| Stop Time | OC | This field holds the end time after which the measurement of KPI is stopped. |
| Notification Sequence | OC | This field holds the received notification sequence number. |

Editor's Note: the table of Network Slice Charging Information is ffs.

Editor’s Note: the Performance and analytics content is ffs.

End of changes