3GPP TSG-SA5 Meeting #131e	S5-203054

25 May to 03 June 2020, E-meeting

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0047 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add KPIs on integrated uplink delay in NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | KPIs on integrated UL packet delay in NG-RAN is missing in TS 28.554. The integrated UL packet delay contributes the significant part of the e2e delay for 5G network, therefore the performance of the integrated UL packet delay needs to be monitored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the KPIs on integrated UL packet delay in NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UL packet delay performance in NG-RAN cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 6.3.1.x	Integrated uplink delay in NG-RAN

##### 6.3.1.x.1	Integrated uplink delay in NG-RAN for a sub-network

a)	DelayUlIntNgranSNw.

b)	This KPI describes the average UL packet delay through the NG-RAN from the UE for a sub-network. It is the average integrated packet delay from the time when an UL RLC SDU was scheduled until time when the corresponding PDCP SDU was sent to the core network from gNB-CU-UP. The KPI type is MEAN in unit of 0.1 ms. This KPI can optionally be split into KPIs per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

c)	This KPI is the sum of

1) 	Average UL packet delay weighted by data volume, including UL delay on air interface and UL delay within NR Cell DU, for all NRCellDU MOIs contained by the SubNetwork MOI, and

2)	Average of UL packet delay weighted by data volume, including UL delay on F1 interface and UL delay within GNB-CU-UP, for all GNBCUUPFunction MOIs contained by the SubNetwork MOI.

This KPI is calculated in the following formula:

DelayUlIntNgranSNw = NRCellDU((DRB.RlcDelayUl + DRB.AirIfDelayUl)*QosFlow.PdcpPduVolumeUl_Filter(NOTE 1)) NRCellDUQosFlow.PdcpPduVolumeUl_Filter(NOTE 1) +  GNBCUUPFunction((DRB.PdcpReordDelayUl + DRB.PdcpF1DelayUl)*QosFlow.PdcpPduVolumeUl_Filter) GNBCUUPFunctionQosFlow.PdcpPduVolumeUl_Filter

For the optional KPI per QoS level (mapped 5QI or QCI in NR option 3), the KPI is calculated by the measurements (subcounters) per QoS level. The KPI formula is as following where the QoS identifies the QoS level:

DelayUlIntNgranSNw.QoS = NRCellDU((DRB.RlcDelayUl.QoS + DRB.AirIfDelayUl.QoS)*QosFlow.PdcpPduVolumeUl_Filter(QoS,   NOTE 1)) NRCellDUQosFlow.PdcpPduVolumeUl_Filter(QoS,   NOTE 1)  + GNBCUUPFunction((DRB.PdcpReordDelayUl.QoS + DRB.PdcpF1DelayUl.QoS)*QosFlow.PdcpPduVolumeUl_Filter(QoS)) GNBCUUPFunctionQosFlow.PdcpPduVolumeUl_Filter(QoS)

For the optional KPI per S-NSSAI, the KPI is calculated by the measurements (subcounters) per S-NSSAI. The KPI formula is as following where the SNSSAI identifies the S-NSSAI:

DelayUlIntNgranSNw.SNSSAI = NRCellDU((DRB.RlcDelayUl.SNSSAI + DRB.AirIfDelayUl.SNSSAI)*QosFlow.PdcpPduVolumeUl_Filter(S-NSSAI,   NOTE 1)) NRCellDUQosFlow.PdcpPduVolumeUl_Filter(S-NSSAI,   NOTE 1)  + GNBCUUPFunction((DRB.PdcpReordDelayUl.SNSSAI + DRB.PdcpF1DelayUl.SNSSAI)*QosFlow.PdcpPduVolumeUl_Filter(S-NSSAI)) GNBCUUPFunctionQosFlow.PdcpPduVolumeUl_Filter(S-NSSAI)

NOTE 1:	The weight QosFlow.PdcpPduVolumeUl_Filter for NRCellDU is the measurement of UL PDCP PDU Data Volume of the corresponding NRCellCU composing the same NR cell.

d)	SubNetwork

e)	It is assumed that the F1 uplink delay is the same as the F1 downlink delay, so the DRB.PdcpF1DelayUl equals to the DRB.PdcpF1DelayDl. For the non-split gNBs, the value of DRB.PdcpF1DelayUl (optionally DRB.PdcpF1DelayUl.QOS, and optionally DRB.PdcpF1DelayUl.SNSSAI) is set to zero for the corresponding GNBCUUPFunction because there are no F1-interfaces in this scenario.

##### 6.3.1.x.2	Integrated uplink delay in NG-RAN for a network slice subnet

a)	DelayUlIntNgranNss.

b)	This KPI describes the average UL packet delay through the NG-RAN from the UE for a network slice subnet. It is the average integrated packet delay from the time when an UL RLC SDU was scheduled until time when the corresponding PDCP SDU was sent to the core network from gNB-CU-UP. The KPI type is MEAN in unit of 0.1 ms.

c)	This KPI is the sum of

1) 	Average of UL packet delay weighted by data volume, including UL delay on air interface and UL delay within NR Cell DU, for all NRCellDU MOIs associated with the NetworkSliceSubnet MOI, and

2)	Average of UL packet delay weighted by data volume, including UL delay on F1 interface and UL delay within GNB-CU-UP, for all GNBCUUPFunction MOIs associated with the NetworkSliceSubnet MOI.

This KPI is calculated in the following formula:

DelayUlIntNgranNss = NRCellDU((DRB.RlcDelayUl.SNSSAI + DRB.AirIfDelayUl.SNSSAI)*QosFlow.PdcpPduVolumeUl_Filter(S-NSSAI,   NOTE 1)) NRCellDUQosFlow.PdcpPduVolumeUl_Filter(S-NSSAI,   NOTE 1)  + GNBCUUPFunction((DRB.PdcpReordDelayUl.SNSSAI + DRB.PdcpF1DelayUl.SNSSAI)*QosFlow.PdcpPduVolumeUl_Filter(S-NSSAI)) GNBCUUPFunctionQosFlow.PdcpPduVolumeUl_Filter(S-NSSAI) Where the SNSSAI identifies the S-NSSAI that the network slice subnet supports.

NOTE 1:	The weight QosFlow.PdcpPduVolumeUl_Filter for NRCellDU is the measurement of UL PDCP PDU Data Volume of the corresponding NRCellCU composing the same NR cell.

d)	NetworkSliceSubnet

e)	It is assumed that the F1 uplink delay is the same as the F1 downlink delay, so the DRB.PdcpF1DelayUl equals to the DRB.PdcpF1DelayDl. For the non-split gNBs, the value of DRB.PdcpF1DelayUl.SNSSAI is set to zero for the corresponding GNBCUUPFunction because there are no F1-interfaces in this scenario.

End of Modified Sections