3GPP TSG-SA5 Meeting #131e 	S5-203080

# e-meeting 25th May-3rd June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0231 | rev | <Rev#> | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of authentication measurements for AMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile,ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The authentication is essential part of all in AMF, so this addition of authentication measurements is necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “Number of authentication requests”, “Number of failed authentications due to parameter error” and “Number of authentication rejection” in clause 5.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The authentication situation is literally unknown. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

5.2.a	Number of authentication requests

a)	This measurement provides the number of authentication requests.

b)	SI

c)	AMF sends authentication request message to UE.

d)	A single integer value

e)	AMF.AuthReq

f)	AMFFunction

g)	Valid for packet switching

h)	5GS

5.2.b	Number of failed authentications due to parameter error

a)	This measurement provides the number of UE sends the authentication failure cause value to AMF when detects that the authentication parameter contained in the authentication request is wrong. The cause value may be 20, 21, 26

b)	SI

c)	AMF receives the authentication failure message sent by UE

d)	A single integer value

e)	AMF.AuthFail
AMF.AuthFail.20
AMF.AuthFail.21
AMF.AuthFail.26

f)	AMFFunction

g)	Valid for packet switching

h)	5GS

5.2.c	Number of authentication rejection

a)	This measurement provides the number of authentication rejection.

b)	SI

c)	AMF sends authentication reject message to UE

d)	A single integer value

e)	AMF.AuthReject

f)	AMFFunction

g)	Valid for packet switching

h)	5GS

End of Modified Sections