3GPP TSG-SA5 Meeting #131e 	S5-203093

# e-meeting 25th May-3rd June 2020

Title:	LS for SON management

Response to:

Release:	Rel-16

Work Item:	SON_5G

Source:	SA5

To:	RAN3

Cc:

Contact Person:

#### Name:	Shi Xiaoli

Tel. Number:

E-mail Address:	shixiaoli@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA5 has captured SON concept and Self-establishment of new RAN NE in network in clause 4.1 and clause 4.2 in TS 28.313 including the definition of centralized SON, Distributed SON and Hybrid SON. The overview of SON concept is shown as below:

4.1.1	Overview

Based on the location of the SON algorithm, SON is categorized into four different solutions that are possible for implementing various SON use cases, the solution is selected depending on the needs of the SON use cases.

a)	Centralized SON:

1)	Cross Domain-Centralized SON

2)	Domain-Centralized SON

b)	Distributed SON

c)	Hybrid SON.

The SON algorithm is not standardized by 3GPP.

The following figure illustrates the overview of SON Framework.

Figure 4.1.1-1 Overview of SON Framework

The introduction of Self-establishment of new RAN NE in network in clause 4.2 is shown as below:

4.2.1	Introduction

Self-establishment of new RAN NE in network describes the procedure of a new NG-RAN NE can automatically establish when it is powered up and connect to the IP network in multi-vendor scenario, which includes:

-	Network Configuration data handling

-	Plug and connect to management system

-	Self-Configuration

SA5 considers that the use case of SON discussed in RAN3 is Distributed SON (D-SON). SA5 provides management services for D-SON management and management services for C-SON including RACH optimization, MRO (Mobility Robustness Optimisation), PCI configuration and ANR management in clause 7.1 and clause 7.2 in TS 28.313, where D-SON use case including RACH optimization, MRO (Mobility Robustness Optimisation), PCI configuration and ANR management, and C-SON use case including PCI configuration. For D-SON, the NFs monitors the network events, analyses the network data, makes decisions on the SON actions and executes the SON actions in the network nodes as described in clause 4.1.3 in TS 28.313.

In addition, SA5 considers that the Self-configuration discussed in RAN3 is related with Self-establishment of new RAN NE in network. SA5 provides network configuration data to support self-configuration process including network configuration data preparation, network configuration data transfer and network configuration data validation as described in clause 4.2.2 in TS 28.313.

SA5 respectfully ask RAN3 to confirm that whether there is a consensus on the SON concept and self-establishment of new RAN NE defined by SA5.

2. Actions:

To RAN3.

ACTION:

SA5 would like to respectfully request RAN3 to confirm that whether there is a consensus on the SON concept and self-establishment of new RAN NE defined by SA5 and provide feedback to SA5 if needed.

3. Date of Next TSG-SA WG5 Meetings:

SA5#132e	24 – 28 August 2020	e-meeting

SA5#133	12 – 16 October 2020	US