3GPP TSG-SA5 Meeting #131e	S5-203285

25 May to 03 June 2020, E-meeting

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0058 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Convert performance measurement job control API to YAML |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE-ADPM5G |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The YAML definition for performance measurement job control API is not available yet, so this API cannot be published on the ETSI Forge. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Convert the performance measurement job control API to YAML. Updated the resource structure and URLs aligning with the generic URL structure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance measurement job control API cannot be published on the ETSI Forge. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2, E.1, E.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 8.2	Resources

### 8.2.0	Resource structure

Figure 8.2.0-1 shows the resource structure of the performance measurement job control service.

Figure 8.2.0-1: Resource URI structure of the performance measurement job control service

Table 8.2.0-1 provides an overview of the resources and applicable HTTP methods.

Table 8.2.0-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| measJobs | /measJobs | GET | Retrieve all or a list of measurement jobs |
|  |  | POST | Create a measurement job |
| measJob | /measJobs /{jobId} | GET | Retrieve a measurement job |
|  |  | DELETE | Stop a measurement job |

### 8.2.1	Resource definitions

#### 8.2.1.1	Void

#### 8.2.1.2	Resource “/measJobs”

##### 8.2.1.2.1	Description

This resource represents a collection of measurement jobs.

##### 8.2.1.2.2	URI

Resource URI = http://{URI-DN-prefix}/{root}/PerfMeasJobCtrlMnS/v1650/{LDN-first-part}

The resource URI variables a defined in the following table.

Table 8.2.1.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| URI-DN-prefix | See subclause 4.4 of TS 32.158 [14] |
| root | See subclause 4.4 of TS 32.158 [14] |
| LDN-first-part | See subclause 4.4 of TS 32.158 [14] |

##### 8.2.1.2.3	HTTP methods

8.2.1.2.3.1	HTTP POST

This method shall support the URI query parameters specified in the following table.

Table 8.2.1.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 8.2.1.2.3.1-2: Data structures supported by the POST request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| measJobCreation-RequestType | The resource representation of the measurement job to be created | M |

Table 8.2.1.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| measJobCreation-ResponseType | 201 Created | In case of success the representation of the created measurement job is returned. | M |
|  | 202 Partially created | In case of partial success the representation of the created measurement job with unsupported list is returned. |  |
| error-Type | 4xx/5xx | Returned in case of an error | M |

8.2.1.2.3.2	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 8.2.1.2.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| jobIdList | Array (string) | This parameter extends the set of targeted resources beyond the base resource identified with the path component of the URI. No scoping mechanism is specified in the present release. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 8.2.1.2.3.2-2: Data structures supported by the GET request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |

Table 8.2.1.2.3.2-3: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| measJobsRetrieval-ResponseType | 200 OK | The resource representations of the measurement job list retrieved. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

#### 8.2.1.3	Resource “/measJobs/{jobId}”

##### 8.2.1.3.1	Description

This resource represents a measurement job.

##### 8.2.1.3.2	URI

Resource URI = http://{URI-DN-prefix}/{root}/PerfMeasJobCtrlMnS/v1650/{LDN-first-part}/measJobs/{jobId}

The resource URI variables a defined in the following table.

Table 8.2.1.3.2-1: URI variables

| Name | Definition |
| --- | --- |
| URI-DN-prefix | See subclause 4.4 of TS 32.158 [14] |
| root | See subclause 4.4 of TS 32.158 [14] |
| LDN-first-part | See subclause 4.4 of TS 32.158 [14] |
| jobId | The id of the measurement job |

##### 8.2.1.3.3	HTTP methods

8.2.1.3.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 8.2.1.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 8.2.1.3.3.1-2: Data structures supported by the GET request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 8.2.1.3.3.1-3: Data structures supported by the GET response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| measJobsRetrieval-ResponseType | 200 OK | The resource representations of the measurement job retrieved. | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

8.2.1.3.3.2	HTTP DELETE

This method shall support the URI query parameters specified in the following table.

Table 8.2.1.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |

This method shall support the request data structures, the response data structures and response codes specified in the following tables.

Table 8.2.1.3.3.2-2: Data structures supported by the DELETE request body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 8.2.1.3.3.2-3: Data structures supported by the DELETE response body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| error-ResponseType | 4xx/5xx | Returned in case of an error | M |

Next Modified Section

## E.1	Introduction

This clause describes the capabilities of the Management Services in the structure of the OpenAPI Specification Version 3.0.1. The OpenAPI document is represented in the YAML format option.

Next Modified Section

## E.2	Performance measurement job control service

openapi: 3.0.1

info:

title: TS 28.550 Performance Measurement Job Control Service

version: 16.5.0

description: >-

OAS 3.0.1 specification of the Performance Measurement Job Control Service

@ 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.550 V16.5.0; Performance assurance

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.550/

servers:

- url: 'http://{URI-DN-prefix}/{root}/PerfMeasJobCtrlMnS/v1650/{LDN-first-part}'

variables:

URI-DN-prefix:

description: See subclause 4.4 of TS 32.158

default: example.com

root:

description: See subclause 4.4 of TS 32.158

default: 3GPPManagement

LDN-first-part:

description: See subclause 4.4 of TS 32.158

default: ''

paths:

/measJobs:

post:

summary: Create a measurement job

description: To create a measurement job the representation of the measurement job is POSTed on the /measJobs collection resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/measJobCreation-RequestType'

responses:

'201':

description: Success case ("201 Created"). The representation of the newly created measurement job resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/measJobCreation-ResponseType'

'202':

description: Partial success case ("202 Partically created"). The representation of the newly created measurement job resource with unsupported list shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/measJobCreation-ResponseType'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

get:

summary: Read resources of measurement jobs

description: 'With HTTP GET, resources of measurement jobs are read. The resources to be read are identified with the path component (base resource) and the query component (jobIdList) of the URI. The fields query component allows to select the resource properties to be returned.'

parameters:

- name: jobIdList

in: query

description: This parameter identifies the list of jobId to select the resources from the collection resources identified with the path component of the URI.

required: true

schema:

type: array

items:

type: string

responses:

'200':

description: 'Success case ("200 OK"). The resources identified in the request for retrieval are returned in the response message body. In case the fields query parameter is used, the selected resources are returned.'

content:

application/json:

schema:

$ref: '#/components/schemas/measJobsRetrieval-ResponseType'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

'/measJobs/{jobId}':

get:

summary: Read resource of a single measurement job

description: 'With HTTP GET, resource of a measurement job is read. The resource to be read is identified with the path component of the URI.'

parameters:

- name: jobId

in: path

description: Identifies the measurement job to be read.

required: true

schema:

$ref: '#/components/schemas/uri-Type'

responses:

'200':

description: 'Success case ("200 OK"). The resource identified in the path for retrieval is returned in the response message body. '

content:

application/json:

schema:

$ref: '#/components/schemas/measJobsRetrieval-ResponseType'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

delete:

summary: Delete a single measurement job

description: The measurement job is deleted by deleting the corresponding measurement job resource. The resource to be deleted is identified with the path component of the URI.

parameters:

- name: jobId

in: path

description: Identifies the measurement job to be deleted.

required: true

schema:

$ref: '#/components/schemas/uri-Type'

responses:

'204':

description: Success case ("204 No Content"). The measurement job resource has been deleted. The response message body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

components:

schemas:

dateTime-Type:

type: string

format: date-Time

uri-Type:

type: string

measJobCreation-RequestType:

type: object

properties:

iOCName:

type: string

iOCInstanceList:

type: array

items:

$ref: '#/components/schemas/uri-Type'

measurementCategoryList:

type: array

items:

type: string

reportingMethod:

$ref: '#/components/schemas/reportingMethod-Type'

granularityPeriod:

type: integer

reportingPeriod:

type: integer

startTime:

$ref: '#/components/schemas/dateTime-Type'

stopTime:

$ref: '#/components/schemas/dateTime-Type'

schedule:

$ref: '#/components/schemas/schedule-Type'

streamTarget:

type: string

priority:

$ref: '#/components/schemas/priority-Type'

reliability:

type: string

measJobCreation-ResponseType:

type: object

properties:

unsupportedList:

type: array

items:

$ref: '#/components/schemas/unsupportedMeas-Type'

measJobsRetrieval-ResponseType:

type: object

properties:

jobInfoList:

type: array

items:

$ref: '#/components/schemas/measJobInfo-ResourceType'

error-ResponseType:

type: object

properties:

error:

type: object

properties:

errorInfo:

type: string

measJobInfo-ResourceType:

type: object

properties:

href:

$ref: '#/components/schemas/uri-Type'

iOCName:

type: string

iOCInstanceList:

type: array

items:

$ref: '#/components/schemas/uri-Type'

measurementCategoryList:

type: array

items:

type: string

reportingMethod:

$ref: '#/components/schemas/reportingMethod-Type'

granularityPeriod:

type: integer

reportingPeriod:

type: integer

startTime:

$ref: '#/components/schemas/dateTime-Type'

stopTime:

$ref: '#/components/schemas/dateTime-Type'

schedule:

$ref: '#/components/schemas/schedule-Type'

streamTarget:

type: string

priority:

$ref: '#/components/schemas/priority-Type'

reliability:

type: string

schedule-Type:

type: object

properties:

scheduleOption:

$ref: '#/components/schemas/scheduleOption-Type'

dailySchedule:

type: array

items:

$ref: '#/components/schemas/timeInterval-Type'

weeklySchedule:

type: array

items:

$ref: '#/components/schemas/scheduleOfDay-Type'

timeInterval-Type:

type: object

properties:

intervalStart:

type: string

format: Time

intervalEnd:

type: string

format: Time

scheduleOfDay-Type:

type: object

properties:

dayOfWeek:

$ref: '#/components/schemas/dayOfWeek-Type'

intervalsOfDay:

type: array

items:

$ref: '#/components/schemas/timeInterval-Type'

unsupportedMeas-Type:

type: object

properties:

iOCInstance:

$ref: '#/components/schemas/uri-Type'

measurementTypeName:

type: string

reason:

type: string

reportingMethod-Type:

type: string

enum:

- file

- streaming

priority-Type:

type: string

enum:

- low

- medium

- high

scheduleOption-Type:

type: string

enum:

- daily

- weekly

dayOfWeek-Type:

type: string

enum:

- Monday

- Tuesday

- Wednesday

- Thursday

- Friday

- Saturday

- Sunday

End of Modified Sections