- 3GPP TSG-SA5 Meeting #131e	S5-203061rev1

- 25 May to 03 June 2020, E-meeting

- Source:	Intel

- Title:	pCR on MDA process

- Document for:	Discussion and endorsement

- Agenda Item:	6.6.4

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.809 “Study on enhancement of Management Data Analytics (MDA)”

# 3	Rationale

The MDA process relies on the Machine Learning technologies. To optimize the accuracy of MDA result, the MDA process may need the consumer to be involved, such as for ML training and MDA result validation.

The MDA process, in connection with the service consumer, is missing in the draft TR [1].

This pCR is to illustrate the MDA process in connection with the service consumer.

# 4	Detailed proposal

1st Modified Section

## 5.x	MDA process

The MDA may rely on ML technologies, which need the consumer to be involved to optimize the accuracy of the MDA results.

The MDA process in terms of the interaction with the consumer, when utilizing ML technologies, is described in the figure below.

Figure 5.x-1: MDA process

ML model training: The consumer may need to train the ML model for MDA. The consumer provides the training data including training input and the desired output to the MDAS producer. The MDAS producer classifies the training data and uses the training input and the desired output to train the ML model, i.e., to train the algorithm of the ML model to be able to provide the desired output by analysis of the training input. The MDAS producer provides an ML model training report as one kind of output data.

Data analysis: The MDAS producer classifies the raw data received and pass along to the ML model for analysis. After analysis, the MDAS producer generates an analytics report as output data of the analysis and provides to the consumer.

Validation: The consumer may validate the output data provided by the MDAS producer. The output data to be validated may be the analytics report or the ML model training report as described above. The consumer may provide the validation data as feedback to the MDAS producer, and the MDAS producer will use the validation data for further ML model training.

End of Modified Sections