- 3GPP TSG SA WG5 Meeting #131-e	S5-203072

- May 25 – June 3, 2020	revision of S5-20abcd

- Source:	Intel

- Title:	pCR 28.313 handover related parameters

- Document for:	Approval

Agenda Item:	6.4.9

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TS 28.313 v0.3.0 “Management and orchestration; Self-Organizing Networks (SON) for 5G networks”

[2] 	R3-201610 “Addition of SON features”

# 3	Rationale

RAN3 has O&M requirements, as shown below (see CR R3-201610 [2]), to support MRO SON feature:

15.X.2.6 	O&M Requirements

All automatic changes of the HO and/or reselection parameters for mobility robustness optimisation shall be within the range allowed by OAM.

This contribution proposes to add the parameters for the ranges of HO parameters to TS 28.313 [1] to support RAN3’s SON features.

# 4	Detailed proposal

1st modified section

### 7.1.2	MRO (Mobility Robustness Optimisation)

#### 7.1.2.2	MnS Component Type B definition

##### 7.1.2.2.3	Parameters to be updated

The ranges of handover parameters to be updated by the MRO are shown in the Table 7.1.2.2.3-1:

Table 7.1.2.2.3-1.  Ranges of handover parameters

| Control parameters | Definition | Legal Values |
| --- | --- | --- |
| A3 offset RSRP range | - The range of RSRP offset as defined in reportConfigNR in TS 38.331 [9] that is used as handover triggering condition for event A3. Allowed value -30..30 times 0.5 dB to get the actual value. | Min value: -30..30 Max value: -30..30 |
| A3 offset RSRQ range | - The range of RSRQ offset as defined in reportConfigNR in TS 38.331 [9] that is used as handover triggering condition for event A3. Allowed value -30..30 times 0.5 dB to get the actual value. | Min value: -30..30 Max value: -30..30 |
| A6 offset RSRP range | - The range of RSRP offset as defined in reportConfigNR in TS 38.331 [9] that is used as handover triggering condition for event A6. Allowed value -30..30 times 0.5 dB to get the actual value. | Min value: -30..30 Max value: -30..30 |
| A6 offset RSRQ range | - The range of RSRQ offset as defined in reportConfigNR in TS 38.331 [9] that is used as handover triggering condition for event A6. Allowed value -30..30 times 0.5 dB to get the actual value. | Min value: -30..30 Max value: -30..30 |

End of modified section