3GPP TSG-SA5 Meeting #131e 	S5-203082

- e-meeting, 25 May – 3 June 2020

- Source:	ZTE

- Title:	Use case and potential solutions of MDA assisted health maintenance

- Document for:	Approval

Agenda Item:	6.6.4

# 1	Decision/action requested

The group is asked to approve the proposal.

# 2	References

[1]	3GPP TR 28.809 Study on enhancement of Management Data Analytics (MDA)

# 3	Rationale

In 5G network, millions of alarms are generated every day. The causes of these alarms are usually the faults or abnormal states of the network. The current treatment method is generally based on the alarm information analysis, to find out the cause of the faults or abnormal states, and then determine the fault repair method and solve the problems, so as to eliminate the alarms. Because the amount of alarms is so large, it is a big challenge to deal with these alarms in a timely and efficient manner.

One the other hand, when we look at it the other way, if the network can be maintained very well so that it has fewer faults and abnormal states, then there will be fewer alarms. This requires the system to be able to predict the potential faults or abnormal states before they occur, and to recommend appropriate handling actions to prevent the fault or abnormal state really occur. We can call this procedure as the health maintenance of the network.

This contribution proposes the use case, requirements and potential solution for network health maintenance based on MDAS.

# 4	Detailed proposal

It proposed to make the following changes in TR 28.809 [1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP draft TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[x]	3GPP TS 28.532: "Management and orchestration; Generic management services".

1st Change

### 6.4.x	MDA assisted health maintenance

#### 6.4.x.1	Use case

In 5G network, millions of alarms are generated every day. The causes of these alarms are usually the faults or abnormal states of the network. The current treatment method is generally based on the alarm information analysis, to find out the cause of the faults or abnormal states, and then determine the fault repair method and solve the problems, so as to eliminate the alarms. Because the amount of alarms is so large, it is a big challenge to deal with these alarms in a timely and efficient manner.

One the other hand, when we look at it the other way, if the network can be maintained very well so that it has fewer faults and abnormal states, then there will be fewer alarms. This requires the system to be able to predict the potential faults or abnormal states before they occur, and to recommend appropriate handling actions to prevent the fault or abnormal state really occur. We can call this procedure as the health maintenance of the network.In 5G network, MDAS is adopted, which is in conjunction with AI and ML techniques. The MDAS producer may be trained train the ML model of the MDAS by using the historical alarms, performance measurements, configuration data and maintenance log data, and network topology information to obtain the basic health maintenance knowledges (e.g. the relationship between the faults or potential faults and the related maintenance actions).

The MDAS producer monitors and analyses the alarms, performance measurements and KPIs continuously, and p analytics report including the predictive information of potential faults or abnormal states and correspondtenance actions to prevent the fault or abnormal state really occur, so that the MDAS consumer can execute the recommended actions accordingly or by taking the recommended actions into account.

The MDAS producer is informed when the recommended actions are taken by the MDAS consumer to maintain the network health, so that the MDAS producer can evaluate the result of the executed actions and update its basic health maintenance knowledges.

The MDAS producer also periodically does the ML training based on the new collected alarms, performance measurements and KPIs, configuration data, and updates its basic health maintenance knowledges.

#### 6.4.x.2	Potential requirements

REQ-HEALTH_MDA-01:	The MDAS producer shall have a capability to provide the analytics report describing the network health maintenance analysis.

REQ-HEALTH_MDA-02:	The analytics report describing the results from the network health maintenance analysis should include the following information:

- Health maintenance report Identifier

- List of potential faults and severity levels

- Affected objects

- Recommended actions

#### 6.4.x.3	Possible solutions

##### 6.4.x.3.1	Solution description

The MDAS producer analyses the management data described in the following subclauses to identify the potential faults or abnormal states and corresponding recommended actions. The required data can be from RAN domain or CN domain or both.

As the table in 6.4.x.3.4 shows, the analytics report is able to be provided by the MDAS producer to describe the analytics result and recommendations of network health maintenance. It can be a domain specific or cross domain analytics report. This procedure may be triggered by the request or periodically.

##### 6.4.x.3.2	Data required for MDA assisted health maintenance for RAN domain

Following table shows the potential data required to analyse the network health maintencance issue for RAN domaim.

| Data Category | Required Data |
| --- | --- |
| Alarm Data | The alarm information collected from RAN domain, see TS 28.532[x]. The types of alarms are FFS. |
| Performance Measurements | RAN related performance measurements and KPIs, see TS 28.552 [8] and TS 28.554[7]; The detailed types of performance measurements and KPIs are FFS |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. |

Note: The above parameters may not be the complete list.

##### 6.4.x.3.3	Data required for MDA assisted health maintenance for CN domain

Following table shows the potential data required to analyse the network health maintencance issue for CN domaim.

| Data Category | Required Data |
| --- | --- |
| Alarm Data | The alarm information collected from CN domain, see TS 28.532[x]. The types of alarms are FFS. |
| Performance Measurements | CN related performance measurements and KPIs, see TS 28.552 [8] and TS 28.554[7]; The detailed types of performance measurements and KPIs are FFS |
| Configuration Data | The execution data including the changes or the configuration of the MOIs. |
| Network topology | The topology of the network deployment. |

Note: The above parameters may not be the complete list.

##### 6.4.x.3.4	Analytics report for MDA assist health maintenance

Following table shows the potential information carried in the analytics report of MDA assist health maintenance.

| Analytics Report of MDA assisted health maintenance | Attribute Name | Description |
| --- | --- | --- |
|  | Health maintenance report identifier | The identifier of the MDA assisted health maintenance analytics report |
|  | List of potential faults | List of potential faults, including: - Fault type - Location - Severity level - Affected objects |
|  | Recommended actions | The recommend actions to eliminate the causes of the faults |

End of changes