3GPP TSG-SA5 Meeting #131e 	S5-203117

- e-meeting 25th May-3rd June 2020

- Source:	Huawei

- Title:	CSIF description addition in subclause 5.2.1.1

- Document for:	Approval

Agenda Item:	7.4.2

# 1	Decision/action requested

This is a pCR to TS 28.202 to add CSIF description in subclause 5.2.1.1, for the CSIF based architecture.

# 2	References

[1]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

# 3	Rationale

This pCR proposes to add CSIF description in subclause 5.2.1.1, for the CSIF based architecture.

# 4	Detailed proposal

First change

#### 5.2.1.1	General

Network Slice Management charging is performed by

- the MnS producer of provisioning MnS, or

- the eCTF, which subscribes to network slice management information from provisioning MnS producer.

interacting with CHF using Nchf specified in TS 32.290 [50] and TS 32.291 [51].In order to provide the data required for the charging management activities outlined in TS 32.240 [1], the MnS producer shall be able to perform converged charging for provisioning MnS when consumed by MnS consumer for NSI allocation, modification and deallocation.

The MnS producer of provisioning MnS/eCTF shall be able to perform converged charging by interacting with CHF, for charging data related to Network Slice Management. The Charging Data Request and Charging Data Response are exchanged between the MnS producer/eCTF and the CHF, based on PEC scenarios specified in TS 32.290 [50]. The Charging Data Request is issued by the MnS producer/eCTF towards the CHF when certain conditions (chargeable events) are met.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in clauses 5.2.2.

A detailed formal description of the converged charging parameters to CHF defined in the present document can be found in TS 32.291 [51].

A detailed formal description of the CDR parameters defined in the present document can be found in TS 32.298 [57].

Whether network slice management charging is active is configued in MnS producer or eCTF. The configuration in MnS producer/eCTF may be updated by the CHF.

End of change