- 3GPP TSG SA WG5 Meeting #131e	S5-203139rev1

- May 25-June 3, 2020, e-meeting	revision of S5-203139

- Source:	ETRI

- Title:	pCR 28.809 addition of network slice load analysis requirements and solution

- Document for:	Approval

Agenda Item:	6.6.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	3GPP TR 28.809 v0.2.0 “Study on enhancement of management data analytics”

[2] 	3GPP TS 23.501 v16.3.0 “System architecture for the 5G System (5GS); Stage 2 (Release 16)”

[3] 	3GPP TS 28.552 v16.4.0 “5G performance measurement (Release 16)”

[4] 	3GPP TS 28.554 v16.3.0 “5G end to end Key Performance Indicators (KPI) (Release 16)”

[5] 	3GPP TS 28.530 v16.1.0 “Concepts, use cases and requirements (Release 16)”

[6] 	3GPP TS 28.531 v16.4.0 “Management and orchestration; Provisioning (Release 16)”

# 3	Rationale

This contribution proposes to add network slice load analysis requirements and solution to TR 28.809 [1].

# 4	Detailed proposal

1st modified section

#### 6.3.2.2	Potential requirements

REQ-NS_LOAD_ANA-1: MDAS producer shall have the capability to provide analytics report describing the network slice load problem.

REQ-NS_LOAD_ANA-2: The analytics report describing the network slide load problem should include the following information:

-	The identifier of the network slice load problem;

-	Indication of the network slice load problem;

-	The start time and end time of the network slice load problem;

-	The type of network slice load problem (overloaded or underutilized);

-	The geographical area, location(UE, gNB, or UPF), domain(RAN, CN) where the network slice load problem exists;

-	The cause(es) of network slice problem (e.g., number of UEs accessing the network, number of QoS flows, service types and the end users distribution);

-	 Root cause of the network slice load problem;

-	The applications affected by the network slide load problem

-	The severity level of the network slice load problem;

-	The recommended actions to solve the network slice load problem.

#### 6.3.2.3	Possible solutions

##### 6.3.2.3.1	Solution description

The MDAS producer correlates and analyse the ongoing and/or potential network slice load problem based on the the current and historical performance data related to resource usage and network traffic for the target network slice.  The slice load is a comprehensive result.  Various factors may impact the network slice load, e.g. number of UEs accessing the network, number of QoS flows, the resource utilizations of different NFs which is related with the network slice instance.  The analysis of network slice load should consider the load of services with different characteristics (e.g., QoS information, service priority), load distribution to derive the corresponding resource requirements.   To identify degradation of the KPIs documented in an SLS due to load issues, traffics and resources related performance measurements and UE measurements can be utilized.  Note that this solution covers both domain specific and cross-domain load problem analysis scenarios.  Based on the anlysis above, the MDAS producer is able to provide the analytics report as defined in 6.3.2.3.3 related with the network slice load analytics triggered by event or periodically.

##### 6.3.2.3.2	Required data for network slice load analysis for cross domain

For cross domain analysis, the RAN and CN domain required data as described in 6.3.x.3.3 and 6.3.x.3.4 may be also needed, as well as the potential data described in the following table..

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | “S-NSSAI” as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for network slice load analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | RAN domain measurement data: RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [4]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552[3]; Performance measurement data for gNB as defined in clause 5.1 of TS 28.552 [3]; CN domain measurement data Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [4]; Other measurement data: End-to-end Latency of 5G Network: “End-to-end Latency of 5G Network” as defined in clause 6.3.1, TS 28.554 [4]; UE throughput: The IP throughput of end users, see clause 5.1.1.3 of TS 28.552 [3]; Throughput for network slice instance: Upstream/Downstream throughput for network and Network Slice Instance, see clause 6.3.2 and clause 6.3.3 of TS 28.554 [4]; Virtualised resource utilization of Network Slice Instance, see clause 6.4.2 of TS 28.554 [4] |
| MDT Data | Measurement data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12]. |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS28.530 [5] and clause 7.15 of TS28.531[6]. |
| Network topology | Network topology data | The topology of the network for network slice load analytics. |

Note: The above parameters may not be the complete list.

##### 6.3.2.3.3	Required data for network slice load for RAN domain

The management data required to analyze the network slice load for RAN domain are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | “S-NSSAI” as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for network slice load analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | RAN domain measurement data: RAN UE throughput: A KPI that shows how NG-RAN impacts the service quality provided to an end-user, see clause 6.3.6 of TS 28.554 [4]; Radio resource utilization: The usage of physical radio resource utilization of the network, see clause 5.1.1.2 of TS 28.552[3]; Performance measurement data for gNB as defined in clause 5.1 of TS 28.552 [3]; |
| MDT Data | Measurement data | UE measurements related to RSRP, RSRQ, SINR and UE location information, see TS 37.320 [12]. |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS28.530 [5] and clause 7.15 of TS28.531[6]. |
| Network topology | Network topology data | The topology of the network for network slice load analytics. |

Note: The above parameters may not be the complete list.

##### 6.3.2.3.4	Required data for network slice load for CN domain

The management data required to analyze the network slice load for CN domain are defined as the following table.

| Input data | Data type | Description |
| --- | --- | --- |
| S-NSSAI | Service data | “S-NSSAI” as defined in clause 5.15.2, TS 23.501 [2]. MDAS uses this information to identify target network slices for network slice load analytics and may derive network topology information according to S-NSSAI. |
| Performance measurement | Measurement data | CN domain measurement data User subscription data by performance measurement for AMF as defined in clause 5.2 of TS 28.552 [3]; Performance measurement data for SMF as defined in caluse 5.3 of TS 28.552 [3]; Performance measurement data for UPF as defined in clause 5.4 of TS 28.552 [3]; Performance measurement data for NF as defined in clause 5.7 of TS 28.552 [3]; Throughput at N3 interface: Upstream/Downstream GTP data throughput at N3 interface, see clause 6.3.4 and clause 6.3.5 of TS 28.554 [4]; |
| Capacity planning data | Use case and procedures | Capacity management use case and procedure, see clause 5.4.15 of TS28.530 [5] and clause 7.15 of TS28.531[6]. |
| Network topology | Network topology data | The topology of the network for network slice load analytics. |

Note: The above parameters may not be the complete list.

##### 6.3.2.3.3	Analytics report for network slice load analysis

Following table provides the potential information of the domain specific or cross domain analytics report for network slice load analysis based on the required data received as described in 6.3.2.3.2, 6.3.2.3.3 and 6.3.2.3.4..

| Analytics Report of resource utilization analysis | Attribute Name | Description |
| --- | --- | --- |
|  | Network slice load problem identifier | The identifier indicates the network slice load problem |
|  | Type of network slice load problem | Indicates the type of the network slice load problem, e.g., ongoing/potential or overload/underutilized network slice load problem |
|  | Time period | Describes the time period(s) during which the network slice load problem has happened or is going to happen |
|  | Slice load level and distribution | Describes the detailed load distribution e.g., load distribution for different applications (probably coming from respective AFs), different frequency layers, different RATs, different locations and/or time periods, different NFs etc. |
|  | Slice load information | Describes the load of the following resources in the network slice: DRBs and SRBs, radio resource utilization and availability, virtual resources of RAN CU, etc |
|  | Slice load statistics | Describes the load statistics of the above resources and their correlated results in terms of coverage and time periods |
|  | Slice load predictive results | Describes the load predictive results of the above resources and their correlations in terms of coverage and time periods |
|  | Indication of network slice load demand | “Indicates, for each time period, that the network slice is overutilized (load is too high) or underutilized (load is too low) |
|  | Percentage of network slice load demand | Describes how much of the slice load capacity is either overloaded or underutilizedt in each time period |
|  | List of network entities | Lists the network entities involved in the network slice load problem |
|  | Recommended actions | Describes the recommended actions to solve the network slice load problem |

End of modified section