3GPP TSG-SA5 Meeting #132e 	S5-204417

# e-meeting 17th 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0010 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove Editors Notes from clause 5.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | COSLA |  |  |  |  |  | Date: |  |  | 2020-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This clause contains the following Editor’s notes: Editor's Note: The need for a use case of communication service assurance for dedicated resources is FFS. No use case has been proposed for this release therefore the Editor’s note can be removed. Editor's Note: Whether provisioning services shall be including also management of SON functions when such are available, is FFS. This topic is not specific to this specification and can therefore be removed Editor's Note: information exchange with application such QoE reporting, is FFS The sources for QoE information can be NWDAF or directly from an UE using MDT/Trace or using QoE reporting. The use of MDT/Trace and QoE reporting are captured in specifications for those topics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The Editor’s notes have been removed. The requirements tags have been put in bold typeface |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential for confusion on the requirements, causing misunderstanding and possible incorrect implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.1.2	Communication service assurance for shared resources

A CSP network where at least one eMBB service is operating, providing services to end-users. A CSC requests from the CSP Order Care a new eMBB service (or any other communication service) for business-critical application(s), submitting an initial proposed communication service SLA. The management system assists CSP Order Care with analysis of the proposed SLA and, when SLA is committed, works together with NF's to ensure communication service SLA goals and optimal use of resources for previous as well as the new communication service(s).

In this scenario, it is assumed that the SLA's for the two communication services will allow for them to share resources, for example RAN and TN resources.

The management systems CS-Assurance service receives the request from Order Care and using a MDAS CS Preparation Assistance service, explores and evaluates communication service realisation and impact on other communication services, if any.

Once the Order Care has committed to an SLA with a CSC, the management system activates the communication service.

As the communication service operates, a management service for communication service assurance, CSA, continuously monitors the SLA fulfilment using MDAS, PM Assurance Services [3] including and Core Network NwDAF QoE analytics service, if available.

Based on goals for SLA fulfilment, or other KPI's, the CSA service may initiate an action when SLA goals are not met, be that over- or under fulfilment. The CSA service may use an MDAS to assist in selecting proper action and how to best execute the action.

The CSA service triggers the action by using provisioning service [4] towards RAN, Transport and Core Network and monitors the effect of the change.

REQ-CSIA_CON-01: The 3GPP management system shall have the capability providing a management service for assisting in assessing (evaluating) a proposed SLA for a requested communication service.

REQ-CSIA_CON-02: The 3GPP management system shall have the capability providing a management service for assisting in asserting an agreed SLA for a requested communication service.

REQ-CSIA_CON-03: The 3GPP management system shall have the capability to process 5GS data and provide analytics services to its consumers.