3GPP TSG-SA5 Meeting #132e 	S5-204435

# e-meeting 17th - 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0262 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements for RACH optimization management for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SON_5G |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Measurements for RACH optimization mangagement are needed to support the RACH optimization SON function, specified in 28.313. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add measurements for RACH optimization management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RACH optimizatoin functions's performance cannot be observed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.20.a, 5.1.1.20.b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.1.1.20.a	Distribution ofnumber of RACH preambles per cell

a)	This measurement provides the distribution of the number of RACH preambles sent by the UE, as reported by the UEs inside the RA-ReportList-r16 IE in the UEInformationResponse-r16 message. The measurement is incremented each time a UEInformationResponse-r16 message containing a RA-ReportList-r16 IE (see TS 38.331 [20]) is received.

b)	CC.

c)	Each of the RA-Report-r16 IEs in the RA-ReportList-r16 increments the measurement bin that is identified by Bin, where Bin corresponds to the number of RACH preambles sent to the cell denoted by cellId-r16 before a successful connection establishment. The number of RACH preambles is obtained by adding all numberOfPreamblesSentOnSSB-r16 IEs in the RA-Report-r16. See TS 38.331 [20] clause 6.2.2.

d)	Each measurement is an integer value.

e)	RACH.PreambleDist.Bin

where Bin is to identify the bins associated with the number of preambles sent.

NOTE:	The number of Bins and the range for each bin is left to implementation.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization management, see TS 28.313 [30].

##### 5.1.1.20.b	Distribution of RACH access delay

a)	This measurement provides the distribution of the RACH access delay, that is the interval from the time a UE sends its first RACH preamble until the UE is connected to the network. The measurement is incremented each time a UEInformationResponse-r16 message containing a RA-ReportList-r16 IE (see TS 38.331 [20]) is received.

b)	CC.

c)	Each of the RA-Report-r16 IEs in the RA-ReportList-r16 increments the measurement bin that is identified by Bin, where Bin corresponds to the UE RACH access delay for that particular RA-Report-r16 received from UE. The access delay is calculated based on the value of numberOfPreamblesSentOnSSB-r16 IE and contentionDetected-r16 IE in PerRAAttemptInfo-r16, where numberOfPreamblesSentOnSSB-r16 IE and PerRAAttemptInfo-r16 IE are contained in PerRASSBInfo-r16 IE. See TS 38.331 [20] clause 6.2.2.

d)	Each measurement is an integer value.

e)	RACH.AccessDelayDist.Bin

where Bin is to identify the bins associated with the RACH access delay.

NOTE:	Bin and the range for each bin is left to implementation.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support RACH optimization management, see TS 28.313 [30].