3GPP TSG-SA5 Meeting #132e 	S5-204078

- e-meeting 17th 28th August 2020

- Source:	China Mobile

- Title:	pCR 28.555 Add concepts and background

- Document for:	Approval

Agenda Item:	6.5.1

# 1	Decision/action requested

To approve the concepts and background.

# 2	References

# 3	Rationale

To provide the concepts and background for TS 28.555.

# 4	Detailed proposal

It proposes to make the following changes to TS 28.555

1st Change

# X	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[a]	ETSI GR NFV-IFA 023 (V3.1.1): "Network Function Virtualisation (NFV); Management and Orchestration; Report on Policy Management in MANO; Release 3".

Next change

# Y	Concepts and Background

## Y.1 	Concepts

Policy Administration Function (PAF) controls and manages the network policy.

Policy Storage Function (PSF) stores the defined network policy,.

Policy Function (PF) makes the policy execution decisions, which means it take the decision to trigger or perform an anction.

Policy Verification Function (PVF) evaluates the effectiveness of the network policy execution.

Note: 	The above concepts are just for everyone to better understand, and these functions are logical functions.

## Y.2 	Policy conflict detection and resolution

There may be policy conflict problem, which is a newly created policy may be conflict with an existing policy.

There are two potential policy conflict detection approaches, one is policy conflict detection in the PF, and the other one is policy conflict detection in the PAF (As described in clauses 5.8.8 and 5.8.9 of GR NFV-IFA 023 [a]). And GR NFV-IFA 023 [a] also descripts policy conflict detection UCs and how to resolve them.

There are multiple scenarios for policy conflict, for example:

1)	If the events are the same and the same condition are triggered (e.g. scaling policy on a VNF), but two policies have different actions, the conflict should be detected and resolved.

2)	If the event and condition of two policies are not be the same, but the different actions (e.g. scaling policy vs. termination policy on a VNF) are towards the same entity. The conflict should be detected and resolved.

3)	If scenario 1) or 2) is enforced by different PFs. The conflict should be detected and resolved.

It is not easier for one PF to resolve all of scenarios of the policy conflict. Hence, it is reasonable to enable PAF to resolve policy conflict, while the conflict can be detected by PAF or PF based on different specific scenarios, it gives rise to the need to define policy management service in 5G service.

End of changes