- 3GPP TSG SA WG5 Meeting #132-e	S5-204111

- August 17 – 28, 2020	revision of S5-20abcd

- Source:	Intel

- Title:	pCR 28.313 add references for MRO measurements

- Document for:	Approval

Agenda Item:	6.4.4

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TS 28.313 v0.4.0 “Management and orchestration; Self-Organizing Networks (SON) for 5G networks”

# 3	Rationale

This contribution proposes to add references to the MRO related measurments.

# 4	Detailed proposal

1st modified section

#### 7.1.2.3	MnS Component Type C definition

##### 7.1.2.3.1	Performance measurements

Performance measurements related MRO are captured in Table 7.1.2.3.1.-1:

Table 7.1.2.3.1-1.  MRO related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Number of handover events | Includes all successful and unsuccessful handover events (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of handover failures | Includes unsuccessful handover events with failure causes (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of intra-RAT handover events | Includes all successful and unsuccessful intra-RAT handover events (see clauses 5.1.1.6.1 and 5.1.1.6.2 in TS 28.552 [5]). | Total intra-RAT handover failure rate |
| Number of intra-RAT handover failures | Includes unsuccessful intra-RAT handover events with failure causes (see clauses 5.1.1.6.1 and 5.1.1.6.2 in TS 28.552 [5]). | Total intra-RAT handover failure rate |
| Number of inter-RAT handover events | Includes all successful and unsuccessful inter-RAT handover events (see clause 5.1.1.6.3 in TS 28.552 [5]). | Total inter-RAT handover failure rate |
| Number of inter-RAT handover failures | Includes unsuccessful inter-RAT handover events with failure causes (see clause 5.1.1.6.3 in TS 28.552 [5]). | Total inter-RAT handover failure rate |
| Number of intra-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed for a long period of time in the cell (see clause 5.1.1.25.1 in TS 28.552 [5]). |  |
| Number of intra-RAT too late handover failures | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.1 in TS 28.552 [5]). |  |
| Number of intra-RAT handover failures to wrong cell | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.1 in TS 28.552 [5]). |  |
| Number of inter-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed in an E-UTRAN cell which connects with 5GC for a long period of time (see clause 5.1.1.25.2 in TS 28.552 [5]). |  |
| Number of inter-RAT too late handover failures | Deteccted when an RLF occurs shortly after a successful handover from an E-UTRAN cell which connects with EPC to a target cell in a E-UTRAN cell which connects with 5GC (see clause 5.1.1.25.2 in TS 28.552 [5]). |  |
| Number of unnecessary handover to another RAT | Detected when a UE is handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient for the service used by the UE (see clause 5.1.1.25.3 in TS 28.552 [5]). |  |
| Number of inter-RAT handover ping pong | Detected when an UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE (see clause 5.1.1.25.4 in TS 28.552 [5]). |  |

End of modified section