3GPP TSG-SA5 Meeting #132e 	S5-204291

- e-meeting 17th 28th August 2020

- Source:	China Mobile

- Title:	Discussion on updating slice profile according to different network slice subnets

- Document for:	Discussion and endorsement

Agenda Item:	6.5.5

# 1	Decision/action requested

This document is for discussion.

# 2	References

[1]	3GPP TS 28.541 Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

# 3	Rationale

In ServiceProfile, a list of end-to-end attributes have been defined, which represent the requirements from service. In the procedure of managing SLA, these requirements should be breaking down to slice profile which can be used by network slice subnet.

To implement these requirements to network, these attributes should be decomposed to each domain. So firstly, the relevant domains of each attribute are listed as below to show which domain these attributes will influence.

serviceProfile:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable | Potential related domains (5GC is 5G Core Network, NG-RAN is defined in TS 23.501, TN is Transport Network) |
| --- | --- | --- | --- | --- | --- | --- |
| serviceProfileId | M | T | F | T | T |  |
| sNSSAIList | M | T | T | F | T |  |
| pLMNIdList | M | T | T | F | T |  |
| maxNumberofUEs | O | T | T | F | T | OAM 5GC |
| coverageArea | O | T | T | F | T | OAM NG-RAN 5GC TN |
| latency | O | T | T | F | T | OAM NG-RAN 5GC TN |
| uEMobilityLevel | O | T | T | F | T | OAM 5GC NG-RAN |
| resourceSharingLevel | O | T | T | F | T | OAM NG-RAN 5GC TN |
| sST | M | T | T | F | T |  |
| availability | O | T | T | F | T | OAM NG-RAN 5GC TN |
| delayTolerance | O | T | T | F | T | OAM NG-RAN 5GC TN |
| deterministicComm | O | T | T | F | T | OAM NG-RAN 5GC TN |
| dLThptPerSlice | O | T | T | F | T | OAM 5GC |
| dLThptPerUE | O | T | T | F | T | OAM NG-RAN 5GC TN |
| uLThptPerSlic | O | T | T | F | T | OAM 5GC |
| uLThptPerUE | O | T | T | F | T | OAM NG-RAN 5GC TN |
| maxPktSize | O | T | T | F | T | OAM 5GC NG-RAN |
| maxNumberofConns | O | T | T | F | T | OAM 5GC |
| kPIMonitoring | O | T | T | F | T | OAM 5GC NG-RAN |
| supportedAccessTech | O | T | T | F | T | OAM NG-RAN 5GC TN |
| userMgmtOpen | O | T | T | F | T | OAM |
| v2XCommModels | O | T | T | F | T | OAM NG-RAN 5GC TN |
| termDensity | O | T | T | F | T | OAM NG-RAN |
| activityFactor | O | T | T | F | T | OAM 5GC NG-RAN |
| uESpeed | O | T | T | F | T | OAM 5GC NG-RAN |
| jitter | O | T | T | F | T | OAM 5GC NG-RAN |
| survivalTime | O | T | T | F | T | OAM 5GC NG-RAN |
| reliability | O | T | T | F | T | OAM 5GC NG-RAN |

Slice profile:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| sliceProfileId | M | T | F | T | T |
| sNSSAIList | M | T | T | F | T |
| pLMNIdList | M | T | T | F | T |
| perfReq | M | T | T | F | T |
| maxNumberofUEs | O | T | T | F | T |
| coverageAreaTAList | O | T | T | F | T |
| latency | O | T | T | F | T |
| uEMobilityLevel | O | T | T | F | T |
| resourceSharingLevel | O | T | T | F | T |

In the R16 slice profile, it’s clear that several attributes in service profile have not been considered in slice profile, and in the perfReq attribute of slice profile, the requirements from SA1 are end-to-end, which also need to be further breaking down. Considering the related domains of each attribute, slice profile of each network subnet can be updated with related attributes.

# 4	Detailed proposal

It is proposed to consider updating slice profile with differnets attributes used in different domians. There are two proposals to do this:

- Split the slice profile into sliceProfile-AN and sliceProfile-CN, which is shown in S5-204293 Update sliceProfile considering different domains proposal 1

- Keep one slice profile and add description to explain which attributes are used in AN or CN, and which attributes are for both domains. This proposal is shown in S5-204293 Update sliceProfile considering different domains proposal 2