3GPP TSG-SA5 Meeting #132e 	S5-204295rev02

- e-meeting 17th 28th August 2020

- Source:	China Telecom

- Title:	Solutions on EE KPI for 5GC

- Document for:	Approval

Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	TR 28.813 0.1.0: Study on new aspects of Energy Efficiency (EE) for 5G

[2] 3GPP TS 28.554: Management and orchestration; 5G end to end Key Performance Indicators (KPI)

[3] ETSI ES 203 539: "Environmental Engineering (EE); Measurement method for energy efficiency of Network Functions Virtualisation (NFV) in laboratory environment"

# 3	Rationale

TS 28.554 [2] – clause 6.7 (Energy Efficiency (EE) KPI) contains so far one single KPI definition: NG-RAN data Energy Efficiency (EE) (cf. clause 6.7.1). A corresponding EE KPI is missing for 5GC.

The power consumption in 5GC is complicated. Taking the NWDAF in 5GC as an example, the services provided by NWDAF includes data collection and analysing. For data collection, there may be network data transferred to and delivered from NWDAF; for data analysing, the large amount of calculation tasks is expected. Both of the two different kinds of the services provided by NWDAF have impact on its power consumption. The EE KPI for 5GC should be universal and be able to apply to all these different cases.

Network Functions Virtualisation (NFV) changes the traditional telecom network architecture to layered model by replacing physical equipment with network functions running on standard server platform. Since the 5GC can be fully virtualized, the energy efficiency metrics and measurement methods for NFV components defined in ETSI ES 203 539 [3] can be introduced as the candidate for the EE KPI(s) for 5GC.

There are three main domains are identified in high-level NFV architecture: Virtualised Network Functions (VNFs), NFV Infrastructure (NFVI) and NFV Management and Orchestration (MANO).

VNF is the software implementation of a network functions which is capable of running over the NFVI. NFVI include diversity of physical resources and virtualised resources to support the execution of the VNFs. The VNFs and NFVI are the dominant parts from energy consumption point of view [3].

The energy efficiency of VNF is evaluated according to hardware energy consumption, resource consumption and utilization related with VNF. The energy efficiency of NFVI is evaluated as resource provision capability which is expressed as service capacity of reference VNFs running on it with amount of energy consumption. Since the NFs within 5GC can be taken as different VNFs, for the purpose of discussion the EE KPI for 5GC, we can focus on the energy efficiency of VNF only.

Proposal 1: the energy efficiency of VNF is evaluated according to hardware energy consumption, resource consumption and utilization related with VNF defined in [3] and this can be used for 5GC EE KPI definition.

Since VNF is software application, its energy consumption cannot be directly and hardware-independently measured with respect to the NFVI impact. Hardware has an idle power consumption regardless of the system activity and a dynamic power consumption caused by the software interactions with hardware components. Software can trigger hardware resources (e.g. CPU, memory, storage, network resources) to switch among different power states to influence the hardware resource utilization and energy consumption [3].

There are two methods defined in [3] for energy consumption measurement of VNF indirectly, and consequently two definitions for the VNF energy efficiency metric are provided:

- measuring the energy consumption of NFVI which only deploy VNF under test, and the VNF Energy Efficiency Ratio is defined as:

Ui is the useful output of VNF under service capacity level i. Depending on the different type of VNFs, it may be

throughput (e.g. bps, pps) for data plane VNF, or capacity (e.g. subscribers, sessions) for control plan VNF.

Pi is power consumption of NFVI platform introduced by VNF deployed under service capacity level i.

VNF_EERi is energy efficiency of VNF under service capacity level i.

VNF_EER is weighted energy efficiency of all service capacity levels.

N is the total number of service capacity levels, and wi is the weight coefficient of level i.

- measuring the resource consumption of VNF under test which solely running on NFVI platform, and the Resource Efficiency Ratio is defined as

where:

Ui is the useful output of VNF under service capacity level i. Depending on the different type of VNFs, it may be

throughput (e.g. bps, pps) for data plane VNF, or capacity (e.g. subscribers, sessions) for control plan VNF.

VNF_RER is resource efficiency of VNF under service capacity level i.

VNF_RER is weighted resource efficiency of all service capacity levels.

N is the total number of service capacity levels, and wi is the weight coefficient of level i.

Resource consumption of VMs allocated to VNF SUT under service capacity level i as following:

Rcpu is CPU resource consumption, defined as used CPU capacity of the underlying VMs related to VNF.

Rmemory is memory resource consumption, defined as total memory used of the underlying VMs related to VNF.

Rstorage is storage resource consumption, defined as total storage used of the underlying VMs related to VNF.

Rnetwork is network resource consumption, defined as the sum of average network throughput of bytes transmitted and

received per second.

Both metrics defined above require that the VNF under test is solely deployed on NFVI.  In other words, if the 5GC NF is the only VNF deployed on the NFVI, both metrics define in [3] can be applied directly.

Observation 1: if a 5GC NF is the only VNF deployed on the NFVI, energy efficiency ratio and resource efficiency ratio define in [3] can be applied as VNF energy efficiency metric.

In practical, it is possible that multiple VNFs are deployed or running on the same NFVI platform, i.e. multiple VNFs contribute to the total power consumption of the NFVI. In that case, it is difficult to get energy efficiency ratio of each VNF based on the power consumption of the entire NFVI platform. However, since the resource consumption used in 2nd method is defined with respect to the VMs related to VNF only, the resource efficiency ratio can be measured for each VNF.

Observation 2: if multiple VNFs are deployed on the NFVI, the resource efficiency ratio can be measured for each VNF.

Since the 5GC NF is composed of 1 to many VNFs, we have the following proposal:

Proposal 2: Based on the resource consumption of VNF the VNF resource efficiency ratio can be used in energy efficiency metric for 5GC.

# 4	Detailed proposal

This document proposes the following changes in TR 28.813 [1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "5G; System Architecture for the 5G System".

[3]	3GPP TS 32.130: "Telecommunication management; Network sharing; Concepts and requirements".

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[6]	ETSI ES 203 228 v1.2.1: "Environmental Engineering (EE); Assessment of mobile network energy efficiency".

[7]	S5-201169/S2-1912770: LS on analytics support for energy saving

[y]	ETSI GS NFV-TST 008 V3.3.1: "Network Functions Virtualisation (NFV) Release 3; Testing; NFVI Compute and Network Metrics Specification"

[z] 	ETSI NFV-IFA 027 V3.3.1: " Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Performance Measurements Specification"

2nd Change

## 4.2	Key Issue #2: EE KPI for 5GC

### 4.2.1	Description

TS 28.554 [5] – clause 6.7 (Energy Efficiency (EE) KPI) contains so far one single KPI definition: NG-RAN data Energy Efficiency (EE) (cf. clause 6.7.1). A corresponding EE KPI is missing for 5GC.

Multiple potential solutions are proposed for this KI:

- Potential solution applying the generic EE KPI.

This potential solution proposes to apply the generic EE KPI (see clause 5.3 of ETSI ES 203 228 [6]):

to the 5GC, leading to the definition of a new EE KPI:

- Potential solution based on the VNF resource efficiency ratio.

With Network Functions Virtualisation (NFV), physical equipment in the telecom network architecture can be replaced by virtual network functions running on standard server platform. Since the 5GC can be fully virtualized, and the 5GC NF can be composed of VNF(s), in this potential solution, the 5GC EE KPI based on Resource Efficiency Ratio (RER) is introduced as:

Depending on the solution, new measurements may have to be defined in TS 28.552 [4].

3rd Change

- 4.2.2.X	Potential solution #X for UsefulOutput5GC

- 4.2.2.X.1	Introduction

This potential solution focuses on the definition of UsefulOuput5GC appeared in:

For this proposed solution, it can be applied to both User Plane Function(UPF) and Control Plane network functions that constitute the 5GC. And it is assumed that the definition of the useful output for UPF and control plane network function are different.

- 4.2.2.1.2	Description

- The UsefulOuput5GC (which is alse denoted as U5GC) is the useful output of the 5GC NF. The useful output of a 5GC NF is defined as the capacity of the 5GC NF, and depending on the different type of 5GC NFs, it may be throughput (e.g. bps, pps) for data plane 5GC NF, or capacity (e.g. subscribers, sessions) for control plan 5GC NF.

In this proposed solution, the following network functions may be considered (see clause 4.2.2 of TS 23.501 [2]):

-	Authentication Server Function (AUSF):

-	Access and Mobility Management Function (AMF)

-	Unstructured Data Storage Function (UDSF)

-	Network Exposure Function (NEF)

-	Intermediate NEF (I-NEF)

-	Network Repository Function (NRF)

-	Network Slice Selection Function (NSSF)

-	Policy Control Function (PCF)

-	Session Management Function (SMF)

-	Unified Data Management (UDM)

-	Unified Data Repository (UDR)

-	User Plane Function (UPF)

-	UE radio Capability Management Function (UCMF)

-	Application Function (AF)

-	5G-Equipment Identity Register (5G-EIR)

-	Network Data Analytics Function (NWDAF)

-	CHarging Function (CHF).

Editor's Note:	The measurement of the UsefulOutput5GC for the 5GC NF is FFS.

4th Change

- 4.2.2.Y	Potential solution #Y for ResourceConsumption5GC

- 4.2.2.Y.1	Introduction

This potential solution focuses on the definition of ResourceConsumption5GC appeared in:

- 4.2.2.1.2	Description

The ResourceConsumption5GC is defined as the resource allocated to 5GC NF. Since the 5GC NF is composed of 1 to many VNFs, the resource allocated to 5GC NF is the sum of the resource allocated to the VNFs composed of the 5GC NF.

where R represents ResourceConsumption.

The resource considered are VM resources and VN resources. Therefore, R5GC can be further denoted as{ RNF, cpu,  RNF, memory, RNF, storage, RNF, network}. Moreover, as 5GC NF is composed of 1 to many VNFs, we have the following definitions:

RNF, cpu is CPU resource consumption, defined as the sum of the used CPU capacity of the underlying VMs allocated to each of the VNFs composing the 5GC NF.

RNF, memory is memory resource consumption, defined as the sum of the total memory used of the underlying VMs allocated to each of the VNFs composing the 5GC NF.

RNF, storage is storage resource consumption, defined as the sum of the total storage used of the underlying VMs allocated to each of the VNFs composing the 5GC NF.

RNF, network is network resource consumption, defined as the sum of the sum of average network throughput of bytes transmitted and received per second on the VN corresponding to each of the VNFs composing the 5GC NF.

NOTE1: Virtual resource consumed by VNF is the sum of the resources of all VMs and VNs allocated to VNF.

NOTE2: Since each type of the resource is one dimension of the 5GC NF or VNF resource consumption, when one or multiple types of resource are used for calculation, such as the calculation of resource efficiency ratio, the calculation is made with respect to aforesaid type(s) of resource.

Since each of the VNFs composing the 5GC NF may not be the only VNF running on a NFVI, the virtual resource consumption of the each VNF needs to be measured separately.

NOTE3: it is assumed that the resource consumption of the a NFVI without VNF deployed is negligible comparing to the resource consumption of the VNF.

Resource consumption can be measured through performance monitoring, which could be external monitoring tools or monitoring functions provided by NFV infrastructure.

The RNF, cpu can be calculated as average CPU utilization (as clause 6.6 in [y]) multiplied by clock speed in megahertz (MHz) of CPU and number of cores.

The measurement of the RNF, memory can be derived from other memory metrics (as clause 8.6 in [y])

The measurement of the RNF, network is defined in [z].

Editor's Note:	It is FFS that whether it is necessary and in what condition the measurement of the data volume can be reused as the measurement of the network resource consumption.

As the methods of measurement for storage systems vary widely and depend on the implementation, the measurement of the RNF, storage may not be defined separately.

Editor's Note:	It is FFS whether the measurement of the storage resource consumption should be defined.

End of change