3GPP TSG-SA5 Meeting #132e 	S5-204353

- e-meeting 17th 28th August 2020

- Source:	Nokia

- Title:	Security riak assessment analytics use case

- Document for:	Approval

Agenda Item:	6.6.3

# 1	Decision/action requested

Propose to introduce a new MDAS use case and solution related to security risk assessment.

# 2	References

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 28.550: "Management and orchestration; Performance assurance".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP draft TR 28.861: "Study on the Self-Organizing Networks (SON) for 5G networks".

[6] 	3GPP TR 28.805: "Study on management aspects of communication services".

[7]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 22.101: "service aspects; service principles".

[10]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[11]	3GPP TS 37.816: "Study on RAN-centric data collection and utilization for LTE and NR".

[12]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description".

[13]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[14]	3GPP TS 28.310: "Energy efficiency of 5G".

[15]	3GPP TR 21.866: "Study on Energy Efficiency Aspects of 3GPP Standards".

[16]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[17]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[18]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

# 3	Rationale

This contribution introduces a new use case, requirements and solution for analysing security related issues.

# 4	Detailed proposal

First Change

## 6.X Security related issues

### 6.X.1 Security risk assessment

#### 6.X.1.1	Use case

Security risk assessement can detect anomalies in managed objects, e.g. NF, identify potential security risks and propose countermeasures to mitigate them. Security risks may originate from a variety of different sources and target distict network objects resulting in different abnormal behaviours, e.g. sudden increase in computing and storage in a virtualization environment, sudden increase of load in network links, abnormal communication patterns between NFs, excessive latency in accessing a NF, relocating a NF in an unexpected location or interrupting the relocation of a NF.

To assess security risks, the management plane can leverage the benfits of MDAS. An MDAS producer can identify a security risk issue by correlating different performance measurements and alarms, i.e. performing a root casue analysis to locate the malicious source and the network objects affected. Such activity can be triggered when certain performance measurements and KPIs indicate an abnormal network behaviour, which is not related to another known faults.

The MDAS producer should correlate the usage, e.g. considering the NF procedures for supporting a number of UEs or other events, with the corresponding resource usage, e.g. in terms of CPU, storage and disk. If no other fault alarms indicate another reason, an unexpected resource usage should trigger a security risk analysis to identify the issue based on a private security risk database that resides on the MDAS producer. The MDAS producer can notify the MDAS consumer, e.g. MnF or NE, and provide recommendations to mitigate the identified security risk. The MDAS consumer based on the recommendations can isolate the malicious network objects, e.g. NF, and can also provide recommendations to harden the network in order to avoid similar security risks in the future.

#### 6.X.1.2	Potential requirements

REQ-SEC_CON-1 The MDAS producer should have a capability to provide the analytics report describing the security risk to authorized consumers based on the correlation of current and predicted performance measurements and alarms.

REQ-SEC_CON-2 The analytics report describing the security issue should contain the following information describing the current and future security risk issue:

-	Identify the type of security risk.

-	Location and network objects affected by the security risk.

-	Root cause analysis of the security risk issue.

-	   Recommended action to isolate and/or restrict the security risk issue and harden the network security.

#### 6. X .1.3	Possible solutions

6. X.1.3.1 		Solution Description

The solution considers security risk assessment related to network. The MDAS producer correlates and analyzes performance mreasurements and alarm data considering the network topology and configuration data related to statistics or predictions to identify the type of security risk.

6. X.1.3.2		Data required

The following data is required to performe the corresponding security risk assessment analysis.

| Data category | Required data |
| --- | --- |
| Alarm Data | Alarm information – types of alarms |
| Service Data | S-NSSAI as defined in clause 5.15.2, TS 23.501 [13]. MDAS may derive network topology information |
| Performance Measurements | Failures and disruptions: - Number of abnormal releases: DRB, QoS flows, PDU sessions, UE context in serving cells as per TS 28.552 [8] - Disruption measurements: CQI/MCS as per TS 28.552 [8] - Excessive delay in accessing NFs (e.g. AMF or SMF) - NF abnormal and excessive communication, e.g. AMF uses a different SMF without performing the expected selection process or it overloads an SMF unexpectedly. - Intra/inter-gNB handover: failures – long handover time as per TS 28.552 [8] Virtualized resources/behaviour: - Virtual resource usage of NF: The resource usage of virtual network functions, see clause 5.7.1 of TS 28.552 [8] - Virtual NF Re-location: Timing/duration and success rate - Frequency of virtual NF Re-location: Rate of relocation - Virtual NF location: NF location with respect to a data network NF context information as per TS 28.552 [8]: - Number of UEs/periodic registration updates (AMF) - Number of PDU sessions/modifications, QoS flows (SMF) - N4/N6/N9 measurements or packet delay, traffic volume, link usage, packet loss (UPF) - Number of application trigger requests rejected (NEF) - Number of failed NF service registration/update, discoveries due to unauthorized NF/error (NRF) |
| Configuration Data | NRM attributes affecting the location and virtual NF resource allocation and configuration NRM update reports (notification and log) containing the creation or changes of the MOIs affecting the virtual NFs |
| Network Topology | Topology of the network |

6. X.1.3.3 		Analytics report

The MDAS producer offers a new Security Analytics service to the MDAS consumer, which supports security risk assessment related providing the following analytics results:

| Information | Description |
| --- | --- |
| Security Incident Identifier | Identifier that indicates the security risk (e.g. DDoS, malicious NF, etc.) |
| Type of Analytics | Statistics or Prediction of security risks |
| Location | Geographical location that the security risk affects |
| Affected Objects | NF, PDU session, QoS Flow, Slice |
| Start/Stop Time | Starts/stop time of the security risk issue |
| Root Cause | The originator of security issue to isolate fast the problem. |
| Severity Level | The severity level (e.g. critical, medium, not important) of the security risk issue |
| Recommended Actions | Recommendation actions to resolve the security risk issue: - Isolate/terminate NF, terminate PDU session, throttle signaling from NF or UE, block UE, etc. - Harden security on specific NF, firewall update, scaling resources, load balancing, admission control, etc. |

End of Change