3GPP TSG-SA5 Meeting #132e 	S5-204391

- Online, , 17 Aug- 28 Aug 2020

- Revision of S5-19xxxx

- Source:	Samsung

- Title:	MDAS assisted network slice traffic projections

- Document for:	Approval

- Agenda Item:	6.6.3

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.809 “Study on enhancement of management data analytics”.

# 3	Rationale

This contribution provide use case and solution for MDAS assisted 5GS traffic projections.

# 4	Detailed proposal

1st Modified Section

### 6.x	Network Slice Traffic Projections

#### 6.8.1.1		Use case

Some of the requirements captured in SliceProfile need to be translated into configurable parameter for various network entities including entities in 5G Core Network (5GC), Radio Access Network (RAN) and Transport Network. One of the example would be: the GST attribute Downlink throughput per slice (dLThptPerSlice) can be translated into maximum downlink throughput per slice (maxDlThptPerSlice) as a configuration parameter, for UPF. However, as one slice may have multiple UPF instances, dividing the total quota available among each UPF instance is critical. The given requirements  can be divided among all the targeted NF instance based on the slice traffic analytics. The value for a particular configurable parameter (translated from a particular ServiceProfile attribute) for a particular NF is crucial, especially if multiple instance of that NFs is available in the network slice instance.

It is desirable to use uses MDAS to get the network slice traffic projections including individual traffic projections on each of the constituent network functions instances present in the slice. The individual traffic projections can be used to divide total available quota among the constituent network functions instances which can then be configured for network function(s), as required. For example, MDAS can provide total number of projected terminal or subscription for each AMF instance in the slice. Based on the projections the total available quota can be divided among the multiple AMF instances in the slice. The AMF instance serving more users or require to serve more users in future will have more quota then other AMF instances in the slice.

#### 6.8.1.2		Potential Requirements

REQ-TRA_CON_CON-1: The MDAS producer should have a capability allowing the authorized consumer to request the slice traffic analytics report describing traffic projection of the slice including its constituent network functions.

REQ-TRA_CON_CON -2: The MDAS producer should have a capability to provide the slice traffic analytics report describing the traffic projections for each constituent network function instance in the slice.

REQ-TRA_CON_CON -3: The slice traffic analytics report providing traffic projection for the slice may include the following information:

- Projected uplink and downlink throughput requirement on each User Plane Function instance (UPF) present in the slice.

- Projected number of Packet Data Unit (PDU) session for each Session Management Function (SMF) instance present in the slice.

- Projected number of UE or Registered subscriptions for each AMF instance present in the slice.

- Projected maximum packet size for each UPF instance present in the slice.

- Projected UE uplink and downlink throughput requirement per slice on each gNodeB (gNB) instance present in the slice.

#### 6.8.1.3		Possible Solutions

The solutions requires MDAS producer to produce slice traffic analytics report providng traffic projections on NFs involved in the slice. Based on the report configuration for the NFs instances can be decided.

6.8.1.3.1 	Data Required

The following table shows the data reqired to generate the traffic analytics report.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements/KPIs |  |
|  | From each UPF instance in the slice - Current Uplink throughput. See 5.4.1.3 in TS 28.552. - Current Downlink throughput. See 5.4.1.4 TS 28.552 - Current Maximum packet size |
|  | From each gNB instance in the slice - Current Uplink UE throughput. See 5.1.1.3 in TS 28.552. - Current Downlink UE throughput. See 5.1.1.3 in TS 28.552. |
|  | From each SMF instance in the slice - Current Number of PDU session. See 5.2.1 in TS 28.552. |
|  | From each AMF instance in the slice - Current Number of registered subscriptions. See 5.2.1 in TS 28.552. |

6.8.1.3.2 	Analytics Report

The slice traffic analytics report shall contain the following

| Slice Traffic Analytics Report | Attribute Name | Description |
| --- | --- | --- |
|  | Slice Identifier | Identifier of the slice for which the report is provided |
|  | Projection Timestamp | Provide a particular time stamp for which the projections are provided |
|  | Projection Duration | Provides a time duration during which the average projections are provided |
|  | For each UPF in the slice | Projected Uplink throughput |
|  |  | Projected Downlink throughput |
|  |  | Projected Maximum packet size |
|  | For each gNB in the slice | Projected Uplink UE throughput. |
|  |  | Projected Downlink UE throughput |
|  | For each SMF in the slice | Projected Number of PDU session |
|  | For each AMF in the slice | Projected Number of registered subscriptions |

End of Modified Sections