3GPP TSG-SA5 Meeting #131e 	S5-204240

# e-meeting 25th May-3rd June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0256 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add incoming and outgoing GTP data packet loss TEI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020-08-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Performance measurement for UPF includes data volume of ougoing GTP packets per QoS level on the N3 interface from UPF to (R)AN. In this measurement scenario, performance measurements on incoming GTP data packet loss and outgoing GTP data packet loss are defined that is split into subcounters per QoS level (5QI). The measurement is obtained by a counter that use the number of missing incoming GTP sequence numbers (TS 29.281) among all GTP packets delivered by a gNB to an UPF interface. However, the sequence number from 1 to 216 to count all GTP packets between gNB and UPF for a subcounter of QoS level is not sufficient for wide band traffics. This contribution is to add TEI information in GTP header as additional information to counter packet loss between gNB and UPF. The subcounter for TEI level would be more specific for packet loss measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new measurement that uses TEI to set subcounter of incoming and outgoing packet loss on N3 interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current definition on measurements of incoming GTP data packet loss and outgoing GTP data packet loss on the N3 interface is not applicable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.x, 5.4.1.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.4.1.x	Incoming GTP Data Packet Loss per TEI

a)	This measurement provides the number of GTP data packets which are not successfully received at UPF. It is a measure of the incoming GTP data packet loss per N3 on an UPF interface. The measurement is split into subcounters per TEI of GTP header (TS 29.281).

b)	CC.

c)	This measurement is obtained by a counter: Number of missing incoming GTP sequence numbers (TS 29.281) among all GTP packets delivered by a gNB to an UPF interface. Separate subcounter is maintained for a GTP tunnel identified by a TEI.

d)	Each measurement is an integer value representing the number of the lost GTP packets. If a GTP tunnel is selected, the measurement within this GTP tunnel is performed by counting the number of the missing incoming GTP sequence number identified by the TEI of GTP packets.

e)	The measurement name has the form

GTP.InDataPktPacketLossN3UPF.TEI where TEI identifies the target GTP tunnel for this measurement.

f)	EP_N3.

g)	Valid for packet switched traffic.

h)	5GS.

5.4.1.y	Outgoing GTP Data Packet Loss per TEI

a)	This measurement provides the number of GTP data packets which are not successfully received at gNB over N3. It is a measure of the outgoing GTP data packet loss per N3 on an UPF interface.  The measurement is split into subcounters per TEI of GTP header (TS 29.281).

b)	CC.

c)	This measurement is obtained by a counter: Number of missing outgoing GTP sequence numbers (TS 29.281) among all GTP packets delivered by an UPF interface to a gNB in a GTP tunnel.

d)	Each measurement is an integer value representing the lost GTP packets.

e)	The measurement name has the form  GTP.OutDataPktPacketLossN3UPF.TEI where TEI identifies the target GTP tunnel for this measurement.

f)	EP_N3.

g)	Valid for packet switched traffic.

h)	5GS.

End of Change