3GPP TSG-SA5 Meeting #133e 	S5-205012

# e-meeting 12th - 21st October 2020

# Source:	3GPP CT3

# Title:	LS on support of stateless NFs

# Document for:	Information, Discussion

Agenda Item: 7.1

3GPP TSG-CT WG3 Meeting #111e	C3-204386

# E-Meeting, 19th – 28th August 2020

Title:	LS on support of stateless NFs

Response to:

Release:	Release 17

Work Item:

Source:	CT3#111e

To:	CT WG4

Cc:	SA WG5

Contact person:	Xiaoyun Zhou

zhouxiaoyun8@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

CT3 is studying the following requirement as defined in subclause 6.5.3.2 and 6.5.3.3 of TS 29.500:

Subclause 6.5.3.2:

6.	When the NF service consumer is changed, and the new NF service consumer does not support handling the notifications as specified in bullet 5, the new NF service consumer should update NF service producers with the new Notification URI. For explicit subscriptions, the NF service consumer should update the subscription or create a new subscription with the new callback URI, depending on the NF service producer's API. For implicit subscriptions, the new Notification URI is carried in a service update request message.

Subclause 6.5.3.3:

6.	When the NF service producer changes, the new NF service producer may update the Subscription Correlation ID by sending a notification to the NF service consumer. The new NF service producer may generate a new resource URI and return it to the NF service consumer upon reception of a service request related to the resource from that NF service consumer, e.g. the new NF service producer may reply with an HTTP 3xx redirect status code pointing to the new location of the resource.

In relation to subclause 6.5.3.2, it seems that this description allows two optional solutions if an NF service consumer changes:

- The NF service consumer, as specified in step 1 of clause 6.5.3.2, provides to NF service producers, via binding indication, the information required to find an alternative NF service consumer within the NF set.

- The NF service consumer, alternatively, as specified in step 6 of clause 6.5.3.2, updates NF service producers with the new notification URI after new NF take over. The notification of the new notification URI is performed for every resource context.

CT3 would like to ask to CT4:

Q1: Is the understanding above correct?

Q2: If it is correct, does it imply that a massive update of the notification URI is happening after new NF take over?

Q3: If it is not a massive update, and the update is individual per resource context, based on e.g. specified service procedures, how could the NF service consumer ensure that the update of the notification URI occurs before an event is matched in the NF service producer and the notification is triggered?

In relation to subclause 6.5.3.3, it seems that this description allows two optional solutions if an NF service producer changes:

- The NF service provider, as specified in step 1 and 3 of clause 6.5.3.3, provides to NF service producers, via binding indication, the information required to find an alternative NF service producer.

- The NF service producer, alternatively, as specified in step 6 of clause 6.5.3.3, may behave according to two optional solutions:

  - The new NF service producer will optionally update the Subscription Correlation ID, if it becomes the role of new service producer.

  - The new service producer generates resource URI, if the NF service consumer request a service and a status code 3xx is used to inform the NF service consumer about the redirection.

CT3 would like to ask CT4:

Q1: Is the understanding above correct?

Q2: If it is correct, does solution “a.” above imply a massive update of the resource URI (Subscription Correlation Id) by the new service producer?

Q3: If it is not a massive update, and the update is performed individually per resource, as per service request, wouldn’t it be enough with solution “b.”, i.e., with the indication of the new location of the resource within the 3xx response?

Q4: Subclause 6.5.3.3, step 3 and step 6 specify the 3xxx redirect response by the NF service producer. Is it required to document per API the different 3xx status codes an API may use? Or is the requirement given in clause 6.5.3.3 of TS 29.500 enough for an interworking of stateless service consumer and producer?

# 2	Actions

To SA WG4

ACTION:	CT3 asks CT4 group to answer above questions.

# 3	Dates of next TSG CT WG 3 meetings

3GPP TSG CT3#112e	04th – 13th November 2020	E-Meeting