3GPP TSG-SA5 Meeting #133e 	S5-205162

# e-meeting 12th - 21st October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0837 | rev | - | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of mandatory SMS message reference in CHF CDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15, 5GS_Ph1-SMSCH |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The messageReference in SMSChargingInformation in the CHF CDR is mandatory, but in in TS 32.291 the corresponding parameter is optional. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the messageReference in SMSChargingInformation in the CHF CDR to optional. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Having a mandatory parameter that cannot be mapped in all cases may case interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.5.2	CHF CDRs

This subclause contains the abstract syntax definitions that are specific to the CHF CDR types defined in this document.

.$CHFChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) chfChargingDataTypes (15) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

CallDuration,

CauseForRecClosing,

ChargingID,

DataVolumeOctets,

Diagnostics,

DynamicAddressFlag,

InvolvedParty,

IPAddress,

LocalSequenceNumber,

ManagementExtensions,

MessageClass,

MessageReference,

MSTimeZone,

NodeAddress,

PLMN-Id,

PriorityType,

RATType,

RecordType,

ServiceSpecificInfo,

Session-Id,

SubscriberEquipmentNumber,

SubscriptionID,

ThreeGPPPSDataOffStatus,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version18 (18) }

ChargingCharacteristics,

ChargingRuleBaseName,

ChChSelectionMode,

EventBasedChargingInformation,

PresenceReportingAreaInfo,

RatingGroupId,

ServiceIdentifier

FROM GPRSChargingDataTypes {itu-t (0) identified-organization (4) etsi (0) mobileDomain (0) charging (5) gprsChargingDataTypes (2) asn1Module (0) version2 (1)}

OriginatorInfo,

RecipientInfo,

SMMessageType,

SMSResult,

SMSStatus

FROM SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  smsChargingDataTypes (10) asn1Module (0) version2 (1)}

;

--

--  CHF RECORDS

--

CHFRecord	::= CHOICE

--

-- Record values 200..201 are specific

--

{

chargingFunctionRecord			[200] ChargingRecord

}

ChargingRecord 	::= SET

{

recordType						[0] RecordType,

recordingNetworkFunctionID		[1] NetworkFunctionName,

subscriberIdentifier			[2] SubscriptionID OPTIONAL,

nFunctionConsumerInformation	[3] NetworkFunctionInformation,

triggers						[4] SEQUENCE OF Trigger OPTIONAL,

listOfMultipleUnitUsage			[5] SEQUENCE OF MultipleUnitUsage OPTIONAL,

recordOpeningTime				[6] TimeStamp,

duration						[7] CallDuration,

recordSequenceNumber			[8] INTEGER OPTIONAL,

causeForRecClosing				[9] CauseForRecClosing,

diagnostics						[10] Diagnostics OPTIONAL,

localRecordSequenceNumber		[11] LocalSequenceNumber OPTIONAL,

recordExtensions				[12] ManagementExtensions OPTIONAL,

pDUSessionChargingInformation	[13] PDUSessionChargingInformation OPTIONAL,

roamingQBCInformation			[14] RoamingQBCInformation OPTIONAL,

sMSChargingInformation			[15] SMSChargingInformation OPTIONAL,

chargingSessionIdentifier		[16] ChargingSessionIdentifier OPTIONAL,

serviceSpecificationInformation	[17] OCTET STRING OPTIONAL

}

--

#### -- PDU Session Charging Information

--

PDUSessionChargingInformation 	::= SET

{

pDUSessionChargingID			[0] ChargingID,

userIdentifier					[1] InvolvedParty OPTIONAL,

userEquipmentInfo				[2] SubscriberEquipmentNumber OPTIONAL,

userLocationInformation			[3] UserLocationInformation OPTIONAL,

userRoamerInOut					[4] RoamerInOut OPTIONAL,

presenceReportingAreaInfo		[5]	PresenceReportingAreaInfo OPTIONAL,

pDUSessionId					[6] PDUSessionId,

networkSliceInstanceID			[7] NetworkSliceInstanceID OPTIONAL,

pDUType							[8] PDUSessionType OPTIONAL,

sSCMode							[9] SSCMode OPTIONAL,

sUPIPLMNIdentifier				[10] PLMN-Id OPTIONAL,

servingNetworkFunctionID		[11] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

rATType							[12] RATType OPTIONAL,

dataNetworkNameIdentifier		[13] DataNetworkNameIdentifier OPTIONAL,

pDUAddress						[14] PDUAddress OPTIONAL,

authorizedQoSInformation		[15] AuthorizedQoSInformation OPTIONAL,

uETimeZone 						[16] MSTimeZone OPTIONAL,

pDUSessionstartTime				[17] TimeStamp OPTIONAL,

pDUSessionstopTime				[18] TimeStamp OPTIONAL,

diagnostics						[19] Diagnostics OPTIONAL,

chargingCharacteristics			[20] ChargingCharacteristics,

-- if chargingCharacteristics is not available a CHF configured value shall be used.

chChSelectionMode				[21] ChChSelectionMode OPTIONAL,

threeGPPPSDataOffStatus			[22] ThreeGPPPSDataOffStatus OPTIONAL,

rANSecondaryRATUsageReport 		[23] SEQUENCE OF NGRANSecondaryRATUsageReport OPTIONAL,

subscribedQoSInformation 		[24] SubscribedQoSInformation OPTIONAL,

authorizedSessionAMBR 			[25] SessionAMBR OPTIONAL,

subscribedSessionAMBR 			[26] SessionAMBR OPTIONAL,

servingCNPLMNID					[27] PLMN-Id OPTIONAL,

sUPIunauthenticatedFlag 		[28] NULL OPTIONAL,

dNNSelectionMode				[29] DNNSelectionMode OPTIONAL

}

--

-- Roaming QBC Information

--

RoamingQBCInformation 	::= SET

{

multipleQFIcontainer			[0] SEQUENCE OF MultipleQFIContainer OPTIONAL,

uPFID							[1] NetworkFunctionName OPTIONAL,

roamingChargingProfile			[2] RoamingChargingProfile OPTIONAL

}

--

#### -- SMS Charging Information

--

SMSChargingInformation	::= SET

{

sMSNodeAddress				[0] AddressString,

originatorInfo				[1] OriginatorInfo OPTIONAL,

recipientInfos				[2] SEQUENCE OF RecipientInfo OPTIONAL,

userEquipmentInfo			[3] SubscriberEquipmentNumber OPTIONAL,

userLocationInformation		[4] UserLocationInformation OPTIONAL,

uETimeZone 					[5] MSTimeZone OPTIONAL,

rATType						[6] RATType OPTIONAL,

sMSCAddress					[7] AddressString OPTIONAL,

eventtimestamp				[8] TimeStamp,

-- 9 to 19 is for future use

sMDataCodingScheme			[20] INTEGER OPTIONAL,

sMMessageType				[21] SMMessageType OPTIONAL,

sMReplyPathRequested		[22] SMReplyPathRequested OPTIONAL,

sMUserDataHeader			[23] OCTET STRING OPTIONAL,

sMSStatus					[24] SMSStatus OPTIONAL,

sMDischargeTime				[25] TimeStamp OPTIONAL,

sMTotalNumber 				[26] INTEGER OPTIONAL,

sMServiceType				[27] SMServiceType OPTIONAL,

sMSequenceNumber 			[28] INTEGER OPTIONAL,

sMSResult					[29] SMSResult OPTIONAL,

submissionTime				[30] TimeStamp OPTIONAL,

sMPriority					[31] PriorityType OPTIONAL,

messageReference			[32] MessageReference OPTIONAL,

messageSize					[33] INTEGER OPTIONAL,

messageClass				[34] MessageClass OPTIONAL,

sMdeliveryReportRequested	[35] SMdeliveryReportRequested OPTIONAL

}

--

-- PDU Container Information

--

PDUContainerInformation 		::= SEQUENCE

{

chargingRuleBaseName				[0] ChargingRuleBaseName OPTIONAL,

aFCorrelationInformation			[1] OCTET STRING OPTIONAL,

timeOfFirstUsage					[2] TimeStamp OPTIONAL,

timeOfLastUsage						[3] TimeStamp OPTIONAL,

qoSInformation						[4] FiveGQoSInformation OPTIONAL,

userLocationInformation				[5] UserLocationInformation OPTIONAL,

presenceReportingAreaInfo			[6] PresenceReportingAreaInfo OPTIONAL,

rATType								[7] RATType OPTIONAL,

sponsorIdentity						[8] OCTET STRING OPTIONAL,

applicationServiceProviderIdentity	[9] OCTET STRING OPTIONAL,

servingNetworkFunctionID			[10] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

uETimeZone 							[11] MSTimeZone OPTIONAL,

threeGPPPSDataOffStatus				[12] ThreeGPPPSDataOffStatus OPTIONAL

}

--

-- QFI Container Information

--

MultipleQFIContainer 		::= SEQUENCE

{

qosFlowId							[0] QoSFlowId OPTIONAL,

triggers							[1] SEQUENCE OF Trigger,

triggerTimeStamp					[2] TimeStamp OPTIONAL,

dataTotalVolume						[3] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[4] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[5] DataVolumeOctets OPTIONAL,

localSequenceNumber					[6] LocalSequenceNumber OPTIONAL,

timeOfFirstUsage					[8] TimeStamp OPTIONAL,

timeOfLastUsage						[9] TimeStamp OPTIONAL,

qoSInformation						[10] FiveGQoSInformation OPTIONAL,

userLocationInformation				[11] UserLocationInformation OPTIONAL,

uETimeZone	 						[12] MSTimeZone OPTIONAL,

presenceReportingAreaInfo			[13] PresenceReportingAreaInfo OPTIONAL,

rATType								[14] RATType OPTIONAL,

reportTime							[15] TimeStamp,

-- if reportTime is not available a CHF configured value shall be used.

servingNetworkFunctionID			[16] SEQUENCE OF ServingNetworkFunctionID OPTIONAL,

threeGPPPSDataOffStatus				[17] ThreeGPPPSDataOffStatus OPTIONAL

}

--

--  CHF CHARGING TYPES

--

--

#### -- A

--

AllocationRetentionPriority	::= SEQUENCE

{

priorityLevel 			[1] INTEGER,

preemptionCapability	[2] PreemptionCapability,

preemptionVulnerability	[3] PreemptionVulnerability

}

AMFID	::= OCTET STRING (SIZE(6))

-- See subclause 2.10.1 of 3GPP TS 23.003 [7] for encoding.

-- AMFID is defined as an OCTET STRING with 3 bytes length, and is presented in first 3 bytes of this form, the last 3 bytes shall be padded with “FFF”

AuthorizedQoSInformation	::= SEQUENCE

--

-- See TS 32.291 [58] for more information

--

{

fiveQi				[1] INTEGER,

-- if fiveQi is not available a CHF configured value shall be used.

aRP					[2] AllocationRetentionPriority,

-- if aRPs not available a CHF configured value shall be used.

priorityLevel 		[3] INTEGER OPTIONAL,

averWindow			[4] INTEGER OPTIONAL,

maxDataBurstVol		[5] INTEGER OPTIONAL

}

--

#### -- B

--

Bitrate	::= OCTET STRING

--

--  See 3GPP TS 29.571 [249] Bitrate data type.

--

--

#### -- C

--

ChargingSessionIdentifier	::= OCTET STRING

-- See 3GPP TS 32.290 [57] for details.

--

#### -- D

--

DataNetworkNameIdentifier	::= IA5String (SIZE(1..63))

--

-- Network Identifier part of DNN in dot representation.

-- For example, if the complete DNN is 'apn1a.apn1b.apn1c.mnc022.mcc111.gprs'

-- The Identifier is 'apn1a.apn1b.apn1c' and is presented in this form in the CDR.

--

DNNSelectionMode	::= ENUMERATED

--

-- See Information Elements TS 29.502 [250] for more information

--

{

uEorNetworkProvidedSubscriptionVerified				(0),

uEProvidedSubscriptionNotVerified					(1),

networkProvidedSubscriptionNotVerified				(2)

}

--

#### -- F

--

FiveGQoSInformation	::= SEQUENCE

--

-- See TS 32.291 [58] for more information

--

{

fiveQi					[1] INTEGER,

-- if fiveQi is not available a CHF configured value shall be used.

aRP						[2] AllocationRetentionPriority,

-- if aRP is not available a CHF configured value shall be used.

qoSNotificationControl	[3] BOOLEAN OPTIONAL,

reflectiveQos			[4] BOOLEAN OPTIONAL,

maxbitrateUL			[5] Bitrate OPTIONAL,

maxbitrateDL			[6] Bitrate OPTIONAL,

guaranteedbitrateUL		[7] Bitrate OPTIONAL,

guaranteedbitrateDL		[8] Bitrate OPTIONAL,

priorityLevel 			[9] INTEGER OPTIONAL,

averWindow				[10] INTEGER OPTIONAL,

maxDataBurstVol			[11] INTEGER OPTIONAL,

maxPacketLossRateDL 	[12] INTEGER OPTIONAL,

maxPacketLossRateUL 	[13] INTEGER OPTIONAL

}

--

#### -- M

--

MultipleUnitUsage 		::= SEQUENCE

{

ratingGroup							[0] RatingGroupId,

usedUnitContainers					[1] SEQUENCE OF UsedUnitContainer OPTIONAL,

uPFID								[2] NetworkFunctionName OPTIONAL

}

--

#### -- N

--

NetworkFunctionality	::= ENUMERATED

{

cHF			(0),	-- this value is not used

sMF			(1),

aMF			(2),

sMSF		(3)

}

NetworkFunctionInformation	::= SEQUENCE

{

networkFunctionality				[0] NetworkFunctionality,

networkFunctionName					[1] NetworkFunctionName OPTIONAL,

networkFunctionIPv4Address			[2] IPAddress OPTIONAL,

networkFunctionPLMNIdentifier		[3] PLMN-Id OPTIONAL,

networkFunctionIPv6Address			[4] IPAddress,

-- if networkFunctionIPv6Address is not available a CHF configured value shall be used.

networkFunctionFQDN					[5] NodeAddress

-- if networkFunctionFQDN is not available a CHF configured value shall be used.

}

NetworkFunctionName	::= IA5String (SIZE(1.. 36))

-- Shall be a Universally Unique Identifier (UUID) version 4, as described in IETF RFC 4122 [410]

NetworkSliceInstanceID	::= SEQUENCE

-- See S-NSSAI subclause 28.4.2 of TS 23.003 [200] for encoding.

{

sST			[0] SliceServiceType,

sD 			[1] SliceDifferentiator OPTIONAL

}

NGRANSecondaryRATType	::= OCTET STRING

--

-- "NR" or "EUTRA"

--

NGRANSecondaryRATUsageReport	::= SEQUENCE

{

nGRANSecondaryRATType			[0] NGRANSecondaryRATType OPTIONAL,

qosFlowsUsageReports			[1] SEQUENCE OF QosFlowsUsageReport OPTIONAL

}

--

#### -- Q

--

QoSFlowId		::= INTEGER

QosFlowsUsageReport		::= SEQUENCE

{

qosFlowId						[0] QoSFlowId OPTIONAL,

startTime						[1] TimeStamp,

endTime							[2] TimeStamp,

dataVolumeDownlink				[3] DataVolumeOctets,

dataVolumeUplink				[4] DataVolumeOctets

}

--

#### -- P

--

PartialRecordMethod	::= ENUMERATED

{

default			(0),

individual		(1)

}

PDUAddress 	::= SEQUENCE

{

pDUIPv4Address				[0] IPAddress OPTIONAL,

pDUIPv6AddresswithPrefix	[1] IPAddress OPTIONAL,

iPV4dynamicAddressFlag		[2] DynamicAddressFlag OPTIONAL,

iPV6dynamicPrefixFlag		[3] DynamicAddressFlag OPTIONAL

}

PDUSessionId 		::= INTEGER (0..255)

--

-- See 3GPP TS 29.571 [249] for details

--

PDUSessionType		::= ENUMERATED

{

iPv4v6			(0),

iPv4			(1),

iPv6			(2),

unstructured	(3),

ethernet		(4)

}

-- See 3GPP TS 29.571 [249] for details.

PreemptionCapability		::= ENUMERATED

{

nOT-PREEMPT			(0),

mAY-PREEMPT			(1)

}

PreemptionVulnerability		::= ENUMERATED

{

nOT-PREEMPTABLE		(0),

pREEMPTABLE			(1)

}

--

#### -- R

--

RatingIndicator	::= BOOLEAN

-- Included if the units have been rated.

RoamingChargingProfile 		::= SEQUENCE

{

roamingTriggers			[0] SEQUENCE OF RoamingTrigger OPTIONAL,

partialRecordMethod		[1] PartialRecordMethod OPTIONAL

}

RoamerInOut	::= ENUMERATED

{

roamerInBound		(0),

roamerOutBound		(1)

}

RoamingTrigger 		::= SEQUENCE

{

trigger					[0] SMFTrigger OPTIONAL,

triggerCategory			[1] TriggerCategory	 OPTIONAL,

timeLimit				[2] CallDuration OPTIONAL,

volumeLimit				[3] DataVolumeOctets OPTIONAL,

maxNbChargingConditions	[4] INTEGER OPTIONAL

}

--

#### -- S

--

ServingNetworkFunctionID	::= SEQUENCE

{

servingNetworkFunctionInformation	[0] NetworkFunctionInformation,

aMFIdentifier						[1] AMFID OPTIONAL

}

SessionAMBR	::= SEQUENCE

{

ambrUL				[1] Bitrate,

ambrDL				[2] Bitrate

}

SliceDifferentiator		::= OCTET STRING (SIZE(3))

--

-- See subclause 28.4.2 TS 23.003 [200]

--

SliceServiceType ::= INTEGER (0..255)

--

-- See subclause 28.4.2 TS 23.003 [200]

--

SMdeliveryReportRequested ::= ENUMERATED

{

yes		(0),

no		(1)

}

SMFTrigger				::= INTEGER

{

startOfPDUSession							(1),

startOfServiceDataFlowNoSession				(2),

-- Change of Charging conditions

qoSChange									(100),

userLocationChange							(101),

servingNodeChange							(102),

presenceReportingAreaChange					(103),

threeGPPPSDataOffStatusChange				(104),

tariffTimeChange							(105),

uETimeZoneChange							(106),

pLMNChange									(107),

rATTypeChange								(108),

sessionAMBRChange							(109),

additionOfUPF								(110),

removalOfUPF 								(111),

gFBRGuaranteedStatusChange					(115),

-- Limit per PDU session

pDUSessionExpiryDataTimeLimit				(200),

pDUSessionExpiryDataVolumeLimit				(201),

pDUSessionExpiryDataEventLimit				(202),

pDUSessionExpiryChargingConditionChanges	(203),

-- Limit per Rating group

ratingGroupDataTimeLimit					(300),

ratingGroupDataVolumeLimit					(301),

ratingGroupDataEventLimit					(302),

-- Quota management

timeThresholdReached						(400),

volumeThresholdReached						(401),

unitThresholdReached						(402),

timeQuotaExhausted							(403),

volumeQuotaExhausted						(404),

unitQuotaExhausted							(405),

expiryOfQuotaValidityTime					(406),

reAuthorizationRequest						(407),

startOfServiceDataFlowNoValidQuota			(408),

otherQuotaType								(409),

-- Others

terminationOfServiceDataFlow				(500),

managementIntervention						(501),

unitCountInactivityTimer					(502),

endOfPDUSession								(503),

cHFResponseWithSessionTermination			(504),

cHFAbortRequest								(505),

abnormalRelease								(506),

-- Limit per QoS Flow

qoSFlowExpiryDataTimeLimit					(600),

qoSFlowExpiryDataVolumeLimit				(601)

}

-- See TS 32.255 [15] for details.

SMReplyPathRequested	::= ENUMERATED

{

noReplyPathSet 			(0),

replyPathSet			(1)

}

SMServiceType 	::= INTEGER

{

-- 0 to 10 VAS4SMS Short Message, see TS TS 22.142 [x] for details

contentProcessing					(0),

forwarding							(1),

forwardingMultipleSubscriptions		(2),

filtering 							(3),

receipt								(4),

networkStorage						(5),

toMultipleDestinations				(6),

virtualPrivateNetwork				(7),

autoreply							(8),

personalSignature					(9),

deferredDelivery					(10)

-- 11 to 99	Reserved for 3GPP defined SM services

-- 100 to 199 Vendor specific SM services

}

SSCMode	::= INTEGER

{

sSCMode1				(1),

sSCMode2				(2),

sSCMode3				(3)

}

-- See 3GPP TS 29.501 [248] for details.

SubscribedQoSInformation	::= SEQUENCE

--

-- See TS 32.291 [58] for more information

--

{

fiveQi				[1] INTEGER,

-- if fiveQi is not available a CHF configured value shall be used.

aRP					[2] AllocationRetentionPriority OPTIONAL,

priorityLevel 		[3] INTEGER OPTIONAL

}

--

#### -- T

--

Trigger	::= CHOICE

{

sMFTrigger		[0] SMFTrigger

}

TriggerCategory	::= ENUMERATED

{

immediateReport		(0),

deferredReport		(1)

}

--

#### -- U

--

UsedUnitContainer 		::= SEQUENCE

{

serviceIdentifier					[0] ServiceIdentifier OPTIONAL,

time								[1] CallDuration OPTIONAL,

triggers							[2] SEQUENCE OF Trigger,

triggerTimeStamp					[3] TimeStamp OPTIONAL,

dataTotalVolume						[4] DataVolumeOctets OPTIONAL,

dataVolumeUplink					[5] DataVolumeOctets OPTIONAL,

dataVolumeDownlink					[6] DataVolumeOctets OPTIONAL,

serviceSpecificUnits				[7] INTEGER OPTIONAL,

eventTimeStamp						[8] TimeStamp OPTIONAL,

localSequenceNumber					[9] LocalSequenceNumber OPTIONAL,

ratingIndicator						[10] RatingIndicator OPTIONAL,

pDUContainerInformation				[11] PDUContainerInformation OPTIONAL,

quotaManagementIndicator			[12] BOOLEAN OPTIONAL

}

UserLocationInformation	::= OCTET STRING

--

-- This data is converted from JSON format of the User Location as described in TS 29.571 [249].

--

.#END

End of changes