3GPP TSG-SA5 Meeting #133e 	S5-205209

# e-meeting 12th - 21st October 2020

# Source:	Huawei, Orange

# Title:	Key Issue EE KPI for network slice types - Potential solution #3 enhancement

# Document for:	Approval

Agenda Item:	6.5.2

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	Draft TR 28.813 0.2.0: Study on new aspects of Energy Efficiency (EE) for 5G

3	Rationale

This pCR proposes enhancements to the already existing potential solution #3 Network Slice Performance definition for MIoT type of network slice for the draft TR 28.813 [1].

4	Detailed proposal

This document proposes the following changes in TR 28.813 [1].

1st Change

4.4.2.3.2	Solution based on the number of active UEs

4.4.2.3.2.1	Introduction

This potential solution focuses on the number of active UEs to the MIoT type of network slice.

In this potential solution, the Network Slice Performance (NSP) for MIoT type of network slice is obtained by summing up the number of active UEs in the DL per cell and the number of active UEs in the UL per cell of the considered network elements over the measurement period, on a per S-NSSAI basis. The samples are aggregated at the NG-RAN node level.

4.4.2.3.2.2	Description

The Network Slice Performance (NSP) for MIoT type of network slice, based on the number of active UEs in the DL per cell and the number of active UEs in the UL per cell, on a per S-NSSAI basis, is defined as:

The following measurements, defined in TS 28.552 [4], are used:

- DRB.MeanActiveUeDl.SNSSAI, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [4] clause 5.1.1.23.1.

- DRB.MeanActiveUeUl.SNSSAI, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [4] clause 5.1.1.23.3.

The final Network Slice EE KPI definition, based on summing up the number of active UEs in the DL per cell and the number of active UEs in the UL per cell of the considered network elements over the measurement period, for MIoT type of slice, would be defined as follows:

End of change