3GPP TSG-SA5 Meeting #133e 	S5-205270

# e-meeting 12th - 21st October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0012 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR TS28.535 Corrections to clause 4.1 and 4.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | <R2/10/2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Figure 4.1.1 is not referred to within the relevant clause 4.1. - Clause 4.2.1, there is some misalignement between the text and figure 4.2.1.1, for example it is not clear where is CSP and NSP fit in the figure, i.e., which steps or part of the diagram relate to CSP and which one belong to NSP. Typically readers of the spec in clause 4.2.1 will keep looking at the figure while reading items 1, 2 and so on and it is not directly corresponding specially to step#1 & 2. Besides the Figure is not referred to within the body of the text in clause 4.2.1. - Minor editorials. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To address (1) add text at end of clause 4.1 to refer to the relevant figure. To address (2) add clarification text to figure 4.2.1.1 to align it with relevant text, and refer to the figure within clause 4.2.1. To address (3) minor corection are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of clarity and refrence to the figures in the relevant text in clause 4.1 and 4.2.1 leading to potential confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modification

# 4	Concepts and background

## 4.1	Lifecycle of a communication service

Communication Service Assurance (CSA) applies to different phases in the life of communication services these lifecycle phases are; preparation, commissioning, operation and decommissioning.

-	Preparation phase:

Providing a communication service starts with preparation, which includes communication service design, pre-planning, feasibility check, i.e., checking the attainable communication service quality from both resource and service aspects, negotiation of the communication service attributes, preparing communication service and network requirements derived from SLA.

-	Commissioning phase:

Once a communication service is prepared, it can be established by converting the communication service requirement to network requirements (interaction and use of NF resources including RAN, CN) to be deployed on the network resources and ready to be used by the communication service consumers (subscribers, UEs). Before allowing the maximum agreed number of communication service consumers to use this communication service a communication service assurance (CSA) control loop is deployed to allow the network to converge to a state where the communication service assurance is stable and within the boundaries of the SLS. The assurance control loop learns the communication service behaviour during an initial deployment or trail phase.

-	Operation phase:

After the commissioning phase, the communication service is activated for use by all communication service consumers (subscribers, UEs) that are allowed to use the communication service. The initial deployment or trail phase for the training of the communication service assurance algorithms has entered the operation phase. A communication service that is activated allows run-time operations e.g., quality of experience assurance, quality of service assurance. The optimization of the utilization by communication services may continue during the operation phase of the communication service.

-	Decommissioning phase:

When the communication service is no longer needed, after being de-activated, the lifecycle of the communication service ends with termination.

Figure 4.1.1 highlights the lifecycle phase sequence involved in the CSA.

Preparation

Commissioning

Operation

Decommissioning

Figure 4.1.1: Lifecycle of a communication service

Start of 2nd modification

## 4.2	Management control loops

### 4.2.1	Overview

For communication service assurance one can identify two interactions of management control loops:

1)	Between the CSC and the CSP: In this case, the CSC provides the requirements for an assured communication service to the CSP, the CSP provides the corresponding communication service, the CSP also provides feedback to the CSC. The CSP adjusts the resources used by a communication service or the CSC adjusts the SLS continuously to achieve the assured requirements.

2)	Between the CSP and the NSP: the communication service provided by CSP requires the network capabilities. For example, the CSP requires a certain network latency. The NSP management system adjusts the network or CSP adjusts the latency requirement continuously to satisfy the latency requirement.

Figure 4.2.1.1: Communication service closed loop assurance

Figure 4.2.1.1 gives a high level description of interaction process involved in the management closed control loop.

Generally, the management control loop for  CSA consists of the steps Monitoring, Analysis, Decision and Execution. The adjustment of the resources used for the communication service is completed by the continuous iteration of the steps in a management control loop. As described in clause 4.1, the management closed control loop for the resources used for the communication service is deployed in the preparation phase and takes effect during the preparation phase and operation phase.

Figure 4.2.1.2 shows the overall process of communication service assurance using a management control loop.

Figure 4.2.1.2: Management Control Loop

End of modifications