3GPP TSG-SA5 Meeting #133e 	S5-205455

# e-meeting 12th - 21st October 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0841 | rev | 1 | Current version: | 16.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SMS originator and recipient info in CHF CDR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1-SMSCH |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Both the Originator Other Address and the Recipient Other Address needs to allow for multiple occurrences to make it possible to send e.g. a NAI and email address at the same time. The TS 32.274 also states that these may have multiple occurrences. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changing the originatorOtherAddress in the originatorInfo to a sequence, the recipientOtherAddress in the recipientInfo and adding the NAI and External Id to the types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The information cannot be recorded and by that not used in rating or billing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Note that this change is not backwards compatible. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-205167. |  |  |  |  |  |  |  |  |

First change

#### 5.2.4.6	SMS CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.274 [34].

.$SMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5)  smsChargingDataTypes (10) asn1Module (0) version2 (1)}

DEFINITIONS IMPLICIT TAGS	::=

BEGIN

-- EXPORTS everything

IMPORTS

DataVolume

FROM CSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) csChargingDataTypes (1) asn1Module (0) version2 (1)}

DiameterIdentity,

LocalSequenceNumber,

ManagementExtensions,

MessageClass,

MessageReference,

MSISDN,

MSTimeZone,

NodeAddress,

PDPAddress,

PLMN-Id,

PriorityType,

RATType,

RecordType,

Session-Id,

SMSResult,

SubscriberEquipmentNumber,

SubscriptionID,

TimeStamp

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version2 (1)}

AddressString,

IMEI,

IMSI,

ISDN-AddressString

FROM MAP-CommonDataTypes {itu-t identified-organization (4) etsi (0) mobileDomain (0) gsm-Network (1) modules (3) map-CommonDataTypes (18)  version18 (18) }

-- from TS 29.002 [214]

CarrierSelectRouting,

NumberPortabilityRouting

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version2 (1)}

;

--

--  SMS RECORDS

--

SMSRecordType	::= CHOICE

--

-- Record values 93,94, 110 to 113 are SMS specific.

--

{

sCSMORecord				 [93] SCSMORecord,

sCSMTRecord				 [94] SCSMTRecord,

sCDVTT4Record			[110] SCDVTT4Record,

sCSMOT4Record			[111] SCSMTRecord,

iSMSMORecord			[112] ISMSMORecord,

iSMSMTRecord			[113] ISMSMTRecord

}

SCSMORecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

originatorInfo				[2] OriginatorInfo OPTIONAL,

recipientInfo				[3] SEQUENCE OF RecipientInfo OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

eventtimestamp				[5]	TimeStamp,

messageReference			[6] MessageReference,

sMTotalNumber 				[7] INTEGER OPTIONAL,

sMSequenceNumber 			[8] INTEGER OPTIONAL,

messageSize					[9] DataVolume OPTIONAL,

messageClass				[10] MessageClass OPTIONAL,

sMdeliveryReportRequested	[11] BOOLEAN OPTIONAL,

sMDataCodingScheme			[12] INTEGER OPTIONAL,

sMMessageType				[13] SMMessageType OPTIONAL,

sMReplyPathRequested		[14] NULL OPTIONAL,

sMUserDataHeader			[15] OCTET STRING OPTIONAL,

userLocationInfo			[16] OCTET STRING OPTIONAL,

rATType						[17] RATType OPTIONAL,

uETimeZone					[18] MSTimeZone OPTIONAL,

sMSResult					[19] SMSResult OPTIONAL,

localSequenceNumber			[22] LocalSequenceNumber OPTIONAL,

recordExtensions			[23] ManagementExtensions OPTIONAL

}

SCSMOT4Record	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

originatorInfo				[2] OriginatorInfo OPTIONAL,

recipientInfo				[3] RecipientInfo OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

eventtimestamp				[5]	TimeStamp,

messageReference			[6] MessageReference,

messageSize					[9] DataVolume OPTIONAL,

messageClass				[10] MessageClass OPTIONAL,

sMdeliveryReportRequested	[11] BOOLEAN OPTIONAL,

sMDataCodingScheme			[12] INTEGER OPTIONAL,

sMMessageType				[13] SMMessageType OPTIONAL,

sMReplyPathRequested		[14] NULL OPTIONAL,

sMUserDataHeader			[15] OCTET STRING OPTIONAL,

userLocationInfo			[16] OCTET STRING OPTIONAL,

rATType						[17] RATType OPTIONAL,

uETimeZone					[18] MSTimeZone OPTIONAL,

sMSResult					[19] SMSResult OPTIONAL,

mTCIWFAddress				[20] NodeAddress OPTIONAL,

sMSApplicationPortID		[21] INTEGER OPTIONAL,

externalIdentifier			[22] SubscriptionID OPTIONAL,

localSequenceNumber			[23] LocalSequenceNumber OPTIONAL,

recordExtensions			[24] ManagementExtensions OPTIONAL

}

SCSMTRecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

recipientInfo				[2] RecipientInfo OPTIONAL,

originatorInfo				[3] OriginatorInfo OPTIONAL,

servedIMEI					[4] IMEI OPTIONAL,

submissionTime				[5]	TimeStamp OPTIONAL,

eventtimestamp				[6]	TimeStamp,

sMPriority					[7]	PriorityType OPTIONAL,

messageReference			[8] MessageReference OPTIONAL,

sMTotalNumber 				[9] INTEGER OPTIONAL,

sMSequenceNumber			[10] INTEGER OPTIONAL,

messageSize					[11] DataVolume OPTIONAL,

messageClass				[12] MessageClass OPTIONAL,

sMdeliveryReportRequested	[13] BOOLEAN OPTIONAL,

sMDataCodingScheme			[14] INTEGER OPTIONAL,

sMMessageType				[15] SMMessageType OPTIONAL,

sMReplyPathRequested		[16] NULL OPTIONAL,

sMUserDataHeader			[17] OCTET STRING OPTIONAL,

sMSStatus					[18] SMSStatus OPTIONAL,

sMDischargeTime				[19] TimeStamp OPTIONAL,

userLocationInfo			[20] OCTET STRING OPTIONAL,

rATType						[21] RATType OPTIONAL,

uETimeZone					[22] MSTimeZone OPTIONAL,

sMSResult					[23] SMSResult OPTIONAL,

sMDeviceTriggerInformation	[25] SMDeviceTriggerInformation OPTIONAL,

localSequenceNumber			[26] LocalSequenceNumber OPTIONAL,

recordExtensions			[27] ManagementExtensions OPTIONAL

}

SCDVTT4Record	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] AddressString,

eventtimestamp				[2]	TimeStamp,

originatorInfo				[3] OriginatorInfo OPTIONAL,

recipientInfo				[4] RecipientInfo OPTIONAL,

sMDeviceTriggerIndicator	[5] SMDeviceTriggerIndicator OPTIONAL,

sMDeviceTriggerInformation	[6] SMDeviceTriggerInformation OPTIONAL,

sMSResult					[7] SMSResult OPTIONAL,

localSequenceNumber			[9] LocalSequenceNumber OPTIONAL,

recordExtensions			[10] ManagementExtensions OPTIONAL

}

ISMSMORecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] NodeAddress,

originatorInfo				[2] OriginatorInfo OPTIONAL,

recipientInfo				[3] SEQUENCE OF RecipientInfo OPTIONAL,

subscriberEquipmentNumber	[4] SubscriberEquipmentNumber OPTIONAL,

eventtimestamp				[5]	TimeStamp,

messageReference			[6] MessageReference,

sMTotalNumber 				[7] INTEGER OPTIONAL,

sMSequenceNumber 			[8] INTEGER OPTIONAL,

messageSize					[9] DataVolume OPTIONAL,

messageClass				[10] MessageClass OPTIONAL,

sMdeliveryReportRequested	[11] BOOLEAN OPTIONAL,

sMDataCodingScheme			[12] INTEGER OPTIONAL,

sMMessageType				[13] SMMessageType OPTIONAL,

sMReplyPathRequested		[14] NULL OPTIONAL,

sMUserDataHeader			[15] OCTET STRING OPTIONAL,

sMSResult					[16] SMSResult OPTIONAL,

userLocationInfo			[17] OCTET STRING OPTIONAL,

rATType						[18] RATType OPTIONAL,

uETimeZone					[19] MSTimeZone OPTIONAL,

pDPAddress					[20] PDPAddress OPTIONAL,

session-Id					[21] Session-Id OPTIONAL,

numberPortabilityRouting	[22] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting		[23] CarrierSelectRouting OPTIONAL,

localSequenceNumber			[24] LocalSequenceNumber OPTIONAL,

recordExtensions			[25] ManagementExtensions OPTIONAL

}

ISMSMTRecord	::= SET

{

recordType					[0] RecordType,

sMSNodeAddress				[1] NodeAddress,

recipientInfo				[2] RecipientInfo OPTIONAL,

originatorInfo				[3] OriginatorInfo OPTIONAL,

subscriberEquipmentNumber	[4] SubscriberEquipmentNumber OPTIONAL,

submissionTime				[5]	TimeStamp OPTIONAL,

eventtimestamp				[6]	TimeStamp,

sMPriority					[7]	PriorityType OPTIONAL,

messageReference			[8] MessageReference,

sMTotalNumber 				[9] INTEGER OPTIONAL,

sMSequenceNumber 			[10] INTEGER OPTIONAL,

messageSize					[11] DataVolume OPTIONAL,

messageClass				[12] MessageClass OPTIONAL,

sMdeliveryReportRequested	[13] BOOLEAN OPTIONAL,

sMDataCodingScheme			[14] INTEGER OPTIONAL,

sMMessageType				[15] SMMessageType OPTIONAL,

sMReplyPathRequested		[16] NULL OPTIONAL,

sMUserDataHeader			[17] OCTET STRING OPTIONAL,

sMSStatus					[18] SMSStatus OPTIONAL,

sMDischargeTime				[19] TimeStamp OPTIONAL,

userLocationInfo			[20] OCTET STRING OPTIONAL,

rATType						[21] RATType OPTIONAL,

uETimeZone					[22] MSTimeZone OPTIONAL,

sMSResult					[23] SMSResult OPTIONAL,

pDPAddress					[24] PDPAddress OPTIONAL,

session-Id					[25] Session-Id OPTIONAL,

numberPortabilityRouting	[26] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting		[27] CarrierSelectRouting OPTIONAL,

localSequenceNumber			[28] LocalSequenceNumber OPTIONAL,

recordExtensions			[29] ManagementExtensions OPTIONAL

}

--

--  SMS DATA TYPES

--

OriginatorInfo	::= SEQUENCE

--

-- OriginatorInfo is used for information about Originator of a Short Message

--

{

originatorIMSI				[0] IMSI OPTIONAL,

originatorMSISDN			[1] MSISDN OPTIONAL,

originatorOtherAddress		[2] SEQUENCE OF SMAddressInfo OPTIONAL,

-- used if type different from IMSI and MSISDN

originatorSCCPAddress		[3] AddressString OPTIONAL,

originatorReceivedAddress	[4] SMAddressInfo OPTIONAL,

sMOriginatorInterface		[5] SMInterface OPTIONAL,

sMOriginatorProtocolID		[6] OCTET STRING OPTIONAL

}

RecipientInfo	::= SEQUENCE

--

-- RecipientInfo is used for information about Recipient of a Short Message

--

{

recipientIMSI				[0] IMSI OPTIONAL,

recipientMSISDN				[1] MSISDN OPTIONAL,

recipientOtherAddress		[2] SEQUENCE OF SMAddressInfo OPTIONAL,

-- used if type different from IMSI and MSISDN

recipientSCCPAddress		[3] AddressString OPTIONAL,

recipientReceivedAddress	[4] SMAddressInfo OPTIONAL,

sMDestinationInterface		[5] SMInterface OPTIONAL,

sMRecipientProtocolID		[6] OCTET STRING OPTIONAL

}

SMAddressDomain		::= SEQUENCE
{

sMDomainName			[0] GraphicString OPTIONAL,

threeGPPIMSI-MCC-MNC	[1] PLMN-Id OPTIONAL

}

SMAddressInfo		::= SEQUENCE
{

sMAddressType		[0] SMAddressType OPTIONAL,

sMAddressData		[1] GraphicString OPTIONAL,

sMAddressDomain		[2] SMAddressDomain OPTIONAL

}

SMAddressType	::= ENUMERATED

{

emailAddress			(0),

mSISDN					(1),

iPv4Address				(2),

iPv6Address				(3),

numericShortCode		(4),

alphanumericShortCode	(5),

other					(6),

iMSI					(7),

nAI						(8),

externalId				(9)

}

SMDeviceTriggerIndicator	::= ENUMERATED

{

notDeviceTrigger		(0),

deviceTriggerRequest	(1),

deviceTriggerReplace	(2),

deviceTriggerRecall		(3)

}

SMDeviceTriggerInformation	::= SEQUENCE

--

-- SMDeviceTriggerInformation is used for information on device triggering from T4

-- as specified in TS 29.337[231]

--

{

mTCIWFAddress			[0] NodeAddress OPTIONAL,

sMDTReferenceNumber		[1] INTEGER OPTIONAL,

sMServingNode			[2] SMServingNode OPTIONAL,

sMDTValidityPeriod		[3] INTEGER OPTIONAL,

sMDTPriorityIndication	[4] SMDTPriorityIndication OPTIONAL,

sMSApplicationPortID	[5] INTEGER OPTIONAL

}

SMDTPriorityIndication	::= ENUMERATED

{

nonpriority	(0),

priority	(1)

}

SMInterface		::= SEQUENCE
{

interfaceId			[0] GraphicString OPTIONAL,

interfaceText		[1] GraphicString OPTIONAL,

interfacePort		[2] GraphicString OPTIONAL,

interfaceType		[3] SMInterfaceType OPTIONAL

}

SMInterfaceType	::= ENUMERATED

{

unkown					(0),

mobileOriginating		(1),

mobileTerminating		(2),

applicationOriginating	(3),

applicationTerminating	(4),

deviceTrigger			(5)

}

SMMessageType	::= ENUMERATED

{

submission			(0),

deliveryReport		(1),

sMServiceRequest	(2),

delivery			(3),

t4DeviceTrigger		(4),

sMDeviceTrigger		(5)

}

SMServingNode		::= SEQUENCE
{

sGSNName			[0] DiameterIdentity OPTIONAL,

sGSNRealm			[1] DiameterIdentity OPTIONAL,

sGSNNumber			[2] AddressString OPTIONAL,

mMEName				[3] DiameterIdentity OPTIONAL,

mMERealm			[4] DiameterIdentity OPTIONAL,

mMENumberForMTSMS	[5] AddressString OPTIONAL,

mSCNumber			[6] AddressString OPTIONAL,

iPSMGWNumber		[7] AddressString OPTIONAL,

iPSMGWName			[8] DiameterIdentity OPTIONAL

}

SMSStatus		::= OCTET STRING (SIZE(1))

.#END

End of changes