3GPP TSG-SA5 Meeting #134-e	S5-206037

# Online, , 16th Nov 2020 - 25th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0412 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | YANG improvements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNRM |  |  |  |  |  | Date: |  |  | 2020-11-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Correction of stage2-stage3 mismatches, of errors and editorial updates to follow TS 32.160. Merging of multiple 5G common type files into one. - S-NSSAI was defined wrongly, this impacts all the users of it. It was also moved to a 5G specific file from the common types file. Another CR will take care of removing the unneeded definition from 28.623. This CR is not dependent on that, the other CR is just a clean-up task. - UPFFunction, one attribute was defined incorrectly, did not follow stage-2. - The sd and sst values were missing from the YANG key statement which restricted their usage in PLMNInfo. The restriction is not present in stage-2. - Merged _3gpp-nr-nrm-common into _3gpp-5g-common-yang-types. The former carried a single type. It is overkill to have a separate file for one type. Also having multiple type defining YANG modules in 5G just leads to confusion as many types are used both in NR-NRM and 5GC. Usage might also change during development of the standard. - Any touched YANG module was also corrected for errors violating the 32.160 like missing contact statements or longer than 80 character lines. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Only the YANG mapping is corrected, Stage-2 not affected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecesarry limitations on the implementers, deviations between stage-2 and stage-3, bad readability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.5.17, E.5.19, E.5.20, E.5.26, E.5.27, H.5. , H.5.1 , H.5.2 , H.5.3 , H.5.4 , H.5.5 , H.5.13 , H.5.14 , H.5.15 , H.5.16 , H.5.17 , H.5.18 , H.5.20 , H.5.22 , H.5.23 , H.5.24 , H.5.25 , H.5.27 , H.5.28 , H.5.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/tree/S5-206037_Rel-17_CR_28.541_YANG_improvements |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## E.5.17	module _3gpp-nr-nrm-gnbcuupfunction.yang

module _3gpp-nr-nrm-gnbcuupfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-gnbcuupfunction";

prefix "gnbcuup3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the GNBCUUPFunction Information

Object Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2020-05-28 { reference "CR-0318"; }

revision 2020-03-12 { reference "SP-200233 S5-201547"; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-08-21 {

reference "Initial revision";

}

grouping TAIGrp {

description "Tracking Area Identity";

list pLMNId {

key "mcc mnc";

description "PLMN IDs for the Tracking area";

uses types3gpp:PLMNId;

}

leaf nRTAC {

type int64;

description "Identity of the common Tracking Area Code for the PLMNs

allowedValues:

a) 	It is the TAC or Extended-TAC.

b) 	A cell can only broadcast one TAC or Extended-TAC.

See TS 36.300, subclause 10.1.7 (PLMNID and TAC relation).

c) 	TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003 and

Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473.

d) 	For a 5G SA (Stand Alone), it has a non-null value.";

}

}

grouping BackhaulAddressGrp {

description "Indicates the backhauladdress of gNB.";

leaf gNBId {

type uint32 {

range "0..4294967295";

}

description "It identifies a gNB within a PLMN. The gNB ID is part of

the NR Cell Identifier (NCI) of the gNB cells.";

reference "gNB Identifier (gNB ID) of subclause 8.2 of TS 38.300.

Global gNB ID in subclause 9.3.1.6 of TS 38.413";

}

list tAI {

key nRTAC;

min-elements 1;

max-elements 1;

description "Tracking Area Identity";

reference "subclause 9.3.3.11 in TS 38.413";

uses TAIGrp;

}

}

grouping MappingSetIDBackhaulAddressGrp {

description "Mapping relationship between setID and backhaulAddress of gNB";

leaf idx {

type uint32 ;

description "ID value";

}

leaf setID {

type uint32;

mandatory true;

description "Indicates the setID of gNB.";

reference "Subclause 7.4.1.6 in TS 38.211";

}

list backhaulAddress {

key gNBId;

min-elements 1;

max-elements 1;

description "Indicates the backhauladdress of gNB.";

uses BackhaulAddressGrp;

}

}

grouping GNBCUUPFunctionGrp {

description "Represents the GNBCUUPFunction IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf gNBCUUPId {

type uint64 {

range "0..68719476735" ;

}

config false;

mandatory true;

description "Identifies the gNB-CU-UP at least within a gNB-CU-CP";

reference "'gNB-CU-UP ID' in subclause 9.3.1.15 of 3GPP TS 38.463";

}

leaf gNBId {

type uint32;

mandatory true;

description "Identifies a gNB within a PLMN. The gNB ID is part of the

NR Cell Identifier (NCI) of the gNB cells. ";

reference "gNB Identifier (gNB ID) of subclause 8.2 of TS 38.300.

Global gNB ID in subclause 9.3.1.6 of TS 38.413";

}

leaf gNBIdLength {

mandatory true;

type int32 { range "22..32"; }

description "Indicates the number of bits for encoding the gNB Id.";

reference "gNB Id in 3GPP TS 38.300, Global gNB ID in 3GPP TS 38.413";

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It

defines which PLMNs that

can be served by the GNBCUUPFunction and

which S-NSSAIs can be supported by the

GNBCUUPFunction for

corresponding PLMN in case of network slicing feature is supported";

key "mcc mnc sd sst";

uses types5g3gpp:PLMNInfo;

}

list mappingSetIDBackhaulAddressList {

key idx;

description "Specifies a list of mappingSetIDBackhaulAddress used to

retrieve the backhaul address of the victim set.

Must be present if Remote Interference Management function is

supported.";

uses MappingSetIDBackhaulAddressGrp;

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the GNBCUUPFunction

supports (is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the GNBCUUPFunction

supports (is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list GNBCUUPFunction {

key id;

description "Represents the logical function CU-UP of gNB or en-gNB.";

reference "3GPP TS 28.541";

uses top3gpp:Top_Grp;

container attributes {

uses GNBCUUPFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

Next Change

## E.5.19	module _3gpp-nr-nrm-nrcellcu.yang

module _3gpp-nr-nrm-nrcellcu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcellcu";

prefix "nrcellcu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRCellCU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-05-08 { reference S5-203316 ; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 {

reference "Initial revision";

}

feature DESManagementFunction {

description "Class representing Distributed SON or Domain-Centralized SON

Energy Saving feature";

}

feature DRACHOptimizationFunction {

description "Class representing D-SON function of RACH optimization

feature";

}

feature DMROFunction {

description "Class representing D-SON function of MRO feature";

}

feature CESManagementFunction {

description "Class representing Cross Domain-Centralized SON Energy Saving

feature";

}

grouping NRCellCUGrp {

description "Represents the NRCellCU IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with corresponding

gNB ID it forms the NR Cell Identifier (NCI).";

mandatory true;

type int32 { range "0..16383"; }

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It defines

which PLMNs

that can be served by the NR cell, and which S-NSSAIs that

can be supported by the

NR cell for corresponding PLMN in case of

network slicing feature is supported.";

// Note: Whether the attribute pLMNId in the pLMNInfo can be writable

// depends on the implementation.

key "mcc mnc sd sst";

min-elements 1;

uses types5g3gpp:PLMNInfo;

}

leaf nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

config false;

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

list NRCellCU {

description "Represents the information required by CU that is

responsible for the management of inter-cell mobility and neighbour

relations via ANR.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellCUGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## E.5.20	module _3gpp-nr-nrm-nrcelldu.yang

module _3gpp-nr-nrm-nrcelldu {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrcelldu";

prefix "nrcelldu3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-nr-nrm-rrmpolicy { prefix nrrrmpolicy3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRCellDU Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-05-08 { reference S5-203316 ; }

revision 2020-02-14 { reference S5-20XXXX ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-09-03 {

reference "Initial revision";

}

feature DPCIConfigurationFunction {

description "Class representing Distributed SON or Domain-Centralized SON

function of PCI configuration feature";

}

feature CPCIConfigurationFunction {

description "Class representing Cross Domain-Centralized SON function of

PCI configuration feature";

}

grouping NRCellDUGrp {

description "Represents the NRCellDU IOC.";

reference "3GPP TS 28.541";

uses mf3gpp:ManagedFunctionGrp;

uses nrrrmpolicy3gpp:RRMPolicy_Grp;

leaf cellLocalId {

description "Identifies an NR cell of a gNB. Together with the

corresponding gNB identifier in forms the NR Cell Identity (NCI).";

reference "NCI in 3GPP TS 38.300";

mandatory true;

type int32 { range "0..16383"; }

}

leaf operationalState  {

description "Operational state of the NRCellDU instance. Indicates

whether the resource is installed and partially or fully operable

(ENABLED) or the resource is not installed or not operable

(DISABLED).";

config false;

type types3gpp:OperationalState;

}

leaf administrativeState  {

description "Administrative state of the NRCellDU. Indicates the

permission to use or prohibition against using the cell, imposed

through the OAM services.";

type types3gpp:AdministrativeState;

default LOCKED;

}

leaf cellState  {

description "Cell state of the NRCellDU instance. Indicates whether the

cell is not currently in use (IDLE), or currently in use but not

configured to carry traffic (INACTIVE), or currently in use and is

configured to carry traffic (ACTIVE).";

config false;

type types3gpp:CellState;

}

list pLMNInfoList {

description "The PLMNInfoList is a list of PLMNInfo data type. It

defines which PLMNs that

can be served by the NR cell, and which

S-NSSAIs that can be supported by the NR cell for

corresponding PLMN

in case of network slicing feature is supported. The plMNId of the

first

entry of the list is the PLMNId used to construct the nCGI for

the NR cell.";

key "mcc mnc sd sst";

min-elements 1;

ordered-by user;

uses types5g3gpp:PLMNInfo;

}

leaf nRPCI {

description "The Physical Cell Identity (PCI) of the NR cell.";

reference "3GPP TS 36.211";

mandatory true;

type int32 { range "0..1007"; }

}

leaf nRTAC {

description "The common 5GS Tracking Area Code for the PLMNs.";

reference "3GPP TS 23.003, 3GPP TS 38.473";

type types3gpp:Tac;

}

leaf arfcnDL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

downlink.";

reference "3GPP TS 38.104";

mandatory true;

type int32;

}

leaf arfcnUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

uplink.";

reference "3GPP TS 38.104";

type int32;

}

leaf arfcnSUL {

description "NR Absolute Radio Frequency Channel Number (NR-ARFCN) for

supplementary uplink.";

reference "3GPP TS 38.104";

type int32;

}

leaf bSChannelBwDL {

description "Base station channel bandwidth for downlink.";

reference "3GPP TS 38.104";

type int32;

units MHz;

}

leaf bSChannelBwUL {

description "Base station channel bandwidth for uplink.";

reference "3GPP TS 38.104";

type int32;

units MHz;

}

leaf bSChannelBwSUL {

description "Base station channel bandwidth for supplementary uplink.";

reference "3GPP TS 38.104";

type int32;

units MHz;

}

leaf ssbFrequency {

description "Indicates cell defining SSB frequency domain position.

Frequency (in terms of NR-ARFCN) of the cell defining SSB transmission.

The frequency identifies the position of resource element RE=#0

(subcarrier #0) of resource block RB#10 of the SS block. The frequency

must be positioned on the NR global frequency raster, as defined in

3GPP TS 38.101-1, and within bSChannelBwDL.";

mandatory true;

type int32 { range "0..3279165"; }

}

leaf ssbPeriodicity {

description "Indicates cell defined SSB periodicity. The SSB periodicity

is used for the rate matching purpose.";

mandatory true;

type int32 { range "5 | 10 | 20 | 40 | 80 | 160"; }

units "subframes (ms)";

}

leaf ssbSubCarrierSpacing {

description "Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz

(< 6 GHz), 120 kHz or 240 kHz (> 6 GHz) are applicable.";

reference "3GPP TS 38.211";

mandatory true;

type int32 { range "15 | 30 | 120 | 240"; }

units kHz;

}

leaf ssbOffset {

description "Indicates cell defining SSB time domain position. Defined

as the offset of the measurement window, in which to receive SS/PBCH

blocks, where allowed values depend on the ssbPeriodicity

(ssbOffset < ssbPeriodicity).";

mandatory true;

type int32 { range "0..159"; }

units "subframes (ms)";

}

leaf ssbDuration {

description "Duration of the measurement window in which to receive

SS/PBCH blocks.";

reference "3GPP TS 38.213";

mandatory true;

type int32 { range "1..5"; }

units "subframes (ms)";

}

leaf-list nRSectorCarrierRef {

description "Reference to corresponding NRSectorCarrier instance.";

min-elements 1;

type types3gpp:DistinguishedName;

}

leaf-list bWPRef {

description "Reference to corresponding BWP instance.";

type types3gpp:DistinguishedName;

}

leaf-list nRFrequencyRef {

description "Reference to corresponding NRFrequency instance.";

type types3gpp:DistinguishedName;

}

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

list NRCellDU {

description "Represents the information of a cell known by DU.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRCellDUGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

Next Change

## E.5.26	module _3gpp-nr-nrm-rrmpolicy.yang

module _3gpp-nr-nrm-rrmpolicy {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-rrmpolicy";

prefix nrrrmpolicy3gpp;

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the RRMPolicy abstract class that

is part of the NR Network Resource Model (NRM).";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-04-28 {

reference "CR0285";

}

revision 2020-02-14 {

reference "Initial revision";

}

grouping rRMPolicyMemberGrp {

description "This data type represents an RRM Policy member that will be

part of a

rRMPolicyMemberList. A RRMPolicyMember is defined by its

pLMNId and sNSSAI (S-NSSAI).

The members in a rRMPolicyMemberList are assigned a specific amount of

RRM resources

based on settings in RRMPolicy.";

uses types5g3gpp:PLMNInfo;

}

typedef CyclicPrefix {

type enumeration {

enum NORMAL;

enum EXTENDED;

}

}

grouping RRMPolicy_Grp {

description "This IOC represents the properties of an abstract RRMPolicy.

The RRMPolicy_ IOC

needs to be subclassed to be instantiated.

It defines two attributes apart from those

inherited from Top IOC, the

resourceType attribute defines type of resource (PRB, RRC

connected users, DRB usage etc.) and the rRMPolicyMemberList attribute

defines the

RRMPolicyMember(s)that are subject to this policy.

An RRM resource (defined in resourceType

attribute) is located in NRCellDU, NRCellCU, GNBDUFunction,

GNBCUCPFunction or in

GNBCUUPFunction. The RRMPolicyRatio IOC is one

realization of a RRMPolicy_ IOC.

This RRM framework allows adding new

policies,

both standardized (like RRMPolicyRatio) or as vendor specific,

by inheriting from the

abstract RRMPolicy_ IOC.";

leaf resourceType {

description "The resourceType attribute defines type of resource (PRB,

RRC connected users,

DRB usage etc.) that is subject to policy.

Valid values are 'PRB', 'RRC' or 'DRB'";

mandatory true;

type string;

}

list rRMPolicyMemberList{

description "It represents the list of RRMPolicyMember (s) that the

managed object

is supporting. A RRMPolicyMember <<dataType>> include

the PLMNId <<dataType>>

and S-NSSAI <<dataType>>." ;

min-elements 1;

key "mcc mnc sd sst";

uses rRMPolicyMemberGrp;

}

} // grouping

grouping RRMPolicyRatioGrp {

description "Represents the RRMPolicyRatio concrete IOC.";

uses RRMPolicy_Grp;    // Inherits RRMPolicy_

leaf rRMPolicyMaxRatio {

description " This attribute specifies the maximum percentage of radio

resource that can be used by the associated rRMPolicyMemberList.

The maximum percentage of radio resource include at least one of

the shared resources, prioritized resources and dedicated resources.

The sum of the rRMPolicyMaxRatio values assigned to all RRMPolicyRatio(s)

name-contained by same ManagedEntity can be greater that 100.";

default 100;

type uint8 { range "0..100"; }

units percent;

}

leaf rRMPolicyMinRatio {

description " This attribute specifies the minimum percentage of radio

resources that can be used by the associated rRMPolicyMemberList.

The minimum percentage of radio resources including at least one of

prioritized resources and dedicated resources. The sum of the

rRMPolicyMinRatio values assigned to all RRM PolicyRatio(s)

name-contained by same ManagedEntity shall be less or equal 100.";

default 0;

type uint8 { range "0..100"; }

units percent;

}

leaf rRMPolicyDedicatedRatio {

description " This attribute specifies the percentage of radio resource

that dedicatedly used by the associated rRMPolicyMemberList. The sum of

the rRMPolicyDeidctaedRatio values assigned to all RRMPolicyRatio(s)

name-contained by same ManagedEntity shall be less or equal 100. ";

default 0;

type uint8 { range "0..100"; }

units percent;

}

}

list RRMPolicyRatio {

description " The RRMPolicyRatio IOC is one realization of a RRMPolicy_ IOC,

see the inheritance in Figure 4.2.1.2-1. This RRM framework allows

adding new policies, both standardized (like RRMPolicyRatio) or as

vendor specific, by inheriting from the

abstract RRMPolicy_ IOC. For details see subclause 4.3.36.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses RRMPolicyRatioGrp;

}

}

}

## E.5.27	Void

Next Change

## H.5.1	module _3gpp-5g-common-yang-types.yang

module _3gpp-5g-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5g-common-yang-types";

prefix "types5g3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The model defines common types for 5G networks and

network slicing.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-20 {

reference "Initial version.";

}

grouping SNssai {

description

"Single Network Slice Selection Assistance Information(S-NSSAI)";

reference "3GPP TS 23.003";

leaf sd {

description "Slice Differentiator

If not needed, the value can be set to FFFFFF.";

type string{

length 6;

pattern '[a-fA-F0-9]*';

}

reference "3GPP TS 23.003";

}

leaf sst {

type uint8;

description "Slice/Service Type.

Values 0 to 127 belong to standardized SST range and are defined in

3GPP TS 23.501. Values 128 to 255 belong to operator-specific range.";

}

}

grouping PLMNInfo {

description "The PLMNInfo data type define a S-NSSAI member in a specific

PLMNId, and it have two attributes PLMNId and S-NSSAI (PLMNId, S-NSSAI).

The PLMNId represents a data type that is comprised of mcc

(mobile country code) and mnc (mobile network code), (See TS 23.003

subclause 2.2 and 12.1) and S-NSSAI represents an data type, that is

comprised of an SST (Slice/Service type) and an optional

SD (Slice Differentiator) field";

uses types3gpp:PLMNId;

uses SNssai;

}

typedef CommModelType {

reference "3GPP TS 23501";

type enumeration {

enum DIRECT_COMMUNICATION_WO_NRF {

value 0;

description "Directly communicate to other pre-configured NF service.";

}

enum DIRECT_COMMUNICATION_WITH_NRF {

value 1;

description "Directly communicate to other NF service discovered

by NRF.";

}

enum INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY {

value 2;

description "Communicate to pre-configured other NF service through

SCP as a proxy.";

}

enum INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY {

value 3;

description "Communication to NF service discovered by NRF through

SCP as a proxy.";

}

}

}

grouping CommModel {

leaf groupId {

type uint16;

}

leaf commModelType {

type CommModelType;

}

leaf-list targetNFServiceList {

type types3gpp:DistinguishedName;

}

leaf commModelConfiguration {

type string;

}

}

grouping SupportedFunc {

leaf function {

type string;

}

leaf policy {

type string;

}

}

}

Next Change

## H.5.2	module _3gpp-5gc-nrm-amffunction.yang

module _3gpp-5gc-nrm-amffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amffunction;

prefix amf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "AMFFunction derived from basic ManagedFunction.";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 {

reference "Ericsson refactoring.";

}

revision 2018-08-07 {

reference "Initial revision";

}

grouping AMFFunctionGrp {

description "Represents the AMFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

container aMFIdentifier {

presence true;

description "An AMF identifier, comprising an AMF Region ID, an

AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

min-elements 1;

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list AMFFunction {

description "5G Core AMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.3	module _3gpp-5gc-nrm-amfregion.yang

module _3gpp-5gc-nrm-amfregion {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfregion;

prefix amfr3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the AMF Region which consists one or

multiple AMF Sets.";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 {

reference "Ericsson refactoring.";

}

grouping AMFRegionGrp {

description "Represents the AMFRegion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC)

and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf aMFRegionId {

description "Represents the AMF Region ID, which identifies the region.";

mandatory true;

type types3gpp:AmfRegionId;

}

leaf-list aMFSet {

description "The AMFSet that the AFMRegion is associated with.";

min-elements 1;

type instance-identifier;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFRegion {

description "5G Core AMFRegion IOC";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFRegionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.4	module _3gpp-5gc-nrm-amfset.yang

module _3gpp-5gc-nrm-amfset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfset;

prefix amfset3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the AMF Set which consists of some AMFs

that serve a given area and Network Slice.";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 {

reference "Ericsson refactoring.";

}

grouping AMFSetGrp {

description "Represents the AMFSet IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed

of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf aMFRegion {

description "The AMFRegion that the AFMSet is associated with.";

type instance-identifier;

}

leaf-list aMFSetMemberList {

description "List of DNs of AMFFunction instances of the AMFSet.";

min-elements 1;

max-elements 1;

type types3gpp:DistinguishedName;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFSet {

description "5G Core AMFSet IOC";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFSetGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.5	module _3gpp-5gc-nrm-ausffunction.yang

module _3gpp-5gc-nrm-ausffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ausffunction;

prefix ausf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the AUSF function in 5GC. For more

information about the AUSF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-22 {

reference "initial revision";

}

grouping AUSFFuntionGrp {

description "Represents the AUSFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list  sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list AUSFFunction {

description "5G Core AUSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AUSFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

Next Change

## H.5.13	module _3gpp-5gc-nrm-nfservice.yang

module _3gpp-5gc-nrm-nfservice {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfservice;

prefix nfs3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-yang-types { prefix yang; }

import ietf-inet-types { prefix inet; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "NF service class.";

reference "3GPP TS 29.510";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-06-17 {

reference "initial revision";

}

grouping NFServiceGrp {

description "Represents the NFService IOC";

leaf serviceInstanceID {

description

"Unique ID of the service instance within a given NF Instance.";

mandatory true;

type string;

}

leaf serviceName {

description "Name of the service instance (e.g. 'nudm-sdm').";

mandatory true;

type ServiceName;

}

list versions { //check in review if key is ok (unique)

description "API versions supported by the NF Service and if available,

the corresponding retirement date of the NF Service.";

min-elements 1;

key "apiVersionInUri apiFullVersion";

uses NFServiceVersion;

}

leaf scheme {

description "URI scheme (e.g. 'http', 'https').";

mandatory true;

type UriScheme;

}

leaf nfServiceStatus {

description "Status of the NF Service Instance.";

mandatory true;

type NFServiceStatus;

}

leaf fqdn {

description "FQDN of the NF Service Instance.";

//optional support

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF service needs to be discoverable by other NFs in a

different PLMN,

then an FQDN that is used for inter PLMN routing.";

//optional support

type inet:domain-name;

}

list ipEndPoints {

description "IP address(es) and port information of the Network Function

(including IPv4 and/or IPv6 address)

where the service is listening

for incoming service requests.";

//optional support

key idx;

leaf idx {

type string;

}

min-elements 1;

uses ipEndPoint;

}

leaf apiPrefix {

description "Optional path segment(s) used to construct the {apiRoot}

variable of the different API URIs.";

//optional support

type string;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

//optional support

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

list allowedPlmns {

description "PLMNs allowed to access the service instance.

The absence of this attribute indicates that any PLMN is allowed to

access the service instance.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the service instance.

The absence of this attribute indicates that any NF type is allowed

to access the service instance.";

min-elements 1;

//optional support

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed to

access the service instance.";

//optional support

min-elements 1;

type string;

}

list allowedNssais {

description "S-NSSAI of the allowed slices to access the service instance.

The absence of this attribute indicates that any slice is allowed to

access the service instance.";

min-elements 1;

//optional support

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf priority {

description "Priority (relative to other services of the same type)

in the range of 0-65535,

to be used for NF Service selection; lower

values indicate a higher priority.";

//optional support

type uint16;

}

leaf capacity {

description "Static capacity information in the range of 0-65535,

expressed as a weight relative to other services of the same type.";

//optional support

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100, indicates

the current load percentage of the NF Service.";

//optional support

type types3gpp:Load;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

//optional support

type yang:date-and-time;

}

list chfServiceInfo { //is the key unique

description "Specific data for a CHF service instance.";

//optional support

max-elements 1;

key "primaryChfServiceInstance secondaryChfServiceInstance";

uses ChfServiceInfo;

}

leaf supportedFeatures {

description "Supported Features of the NF Service instance.";

//optional support

type SupportedFeatures;

}

}

typedef SupportedFeatures {

type string {

pattern '[A-Fa-f0-9]*';

}

}

grouping ipEndPoint {

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

}

leaf transport {

type TransportProtocol;

}

leaf port {

type uint16;

}

}

typedef TransportProtocol {

type enumeration {

enum TCP;

enum STCP;

enum UDP;

}

}

grouping NFServiceVersion {

leaf apiVersionInUri {

mandatory true;

type string;

}

leaf apiFullVersion {

mandatory true;

type string;

}

leaf expiry {

//optional to support

type yang:date-and-time;

}

}

typedef ServiceName {

type enumeration {

enum NNRF_NFM;

enum NNRF_DISC;

enum NUDM_SDM;

enum NUDM_UECM;

enum NUDM_UEAU;

enum NUDM_EE;

enum NUDM_PP;

enum NAMF_COMM;

enum NAMF_EVTS;

enum NAMF_MT;

enum NAMF_LOC;

enum NSMF_PDUSESSION;

enum NSMF_EVENT-EXPOSURE;

enum NAUSF_AUTH;

enum NAUSF_SORPROTECTION;

enum NNEF_PFDMANAGEMENT;

enum NPCF_AM-POLICY-CONTROL;

enum NPCF_SMPOLICYCONTROL;

enum NPCF_POLICYAUTHORIZATION;

enum NPCF_BDTPOLICYCONTROL;

enum NPCF_EVENTEXPOSURE;

enum NPCF_UE_POLICY_CONTROL;

enum NSMSF_SMS;

enum NNSSF_NSSELECTION;

enum NNSSF_NSSAIAVAILABILITY;

enum NUDR_DR;

enum NLMF_LOC;

enum N5G_EIR_EIC;

enum NBSF_MANAGEMENT;

enum NCHF_SPENDINGLIMITCONTROL;

enum NCHF_CONVERGEDCHARGING;

enum NNWDAF_EVENTSSUBSCRIPTION;

enum NNWDAF_ANALYTICSINFO;

}

}

typedef UriScheme {

type enumeration {

enum HTTP;

enum HTTPS;

}

}

typedef NFServiceStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

enum UNDISCOVERABLE;

}

}

grouping ChfServiceInfo {

leaf primaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a

secondary CHF instance of another primary CHF service instance.";

//conditional to support

type string;

}

leaf secondaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a

primary CHF instance of another secondary CHF service instance.";

//conditional to support

type string;

}

}

}

## H.5.14	module _3gpp-5gc-nrm-ngeirfunction.yang

module _3gpp-5gc-nrm-ngeirfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ngeirfunction;

prefix ngeir3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the 5G-EIR function in 5GC. For more

information about the 5G-EIR, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {

reference "initial revision";

}

grouping NGEIRFunctionGrp {

description "Represents the NGEIRFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list NGEIRFunction {

description "5G Core NGEIR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NGEIRFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.15	module _3gpp-5gc-nrm-nrffunction.yang

module _3gpp-5gc-nrm-nrffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nrffunction;

prefix nrf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NRF function in 5GC.

For more information about the NRF, see 3GPP TS 23.501 [2].";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-15 { reference "initial revision"; }

grouping NRFFunctionGrp {

description "Represents the NRFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and subclause 6.1.6.2.7 of 3GPP TS 29.531";

type string;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list nFProfileList {

description "Set of NFProfile(s) to be registered in the NRF instance.";

//optional support

key nfInstanceID;

uses nfp3gpp:NFProfileGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NRFFunction {

description "5G Core NRF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.16	module _3gpp-5gc-nrm-nssffunction.yang

module _3gpp-5gc-nrm-nssffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nssffunction;

prefix nssf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NSSF function in 5GC. For more

information about the NSSF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 { reference "initial revision"; }

grouping NSSFFunctionGrp {

description "Represents the NSSFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least

one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and subclause 6.1.6.2.7 of 3GPP TS 29.531";

type string;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list NSSFFunction {

description "5G Core NSSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NSSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.17	module _3gpp-5gc-nrm-nwdaffunction.yang

module _3gpp-5gc-nrm-nwdaffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nwdaffunction;

prefix nwdaf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NWDAF function in 5GC. For more

information about the NWDAF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {

reference "initial revision";

}

grouping NWDAFFunctionGrp {

description "Represents the NWDAFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list NWDAFFunction {

description "5G Core NWDAF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NWDAFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.18	module _3gpp-5gc-nrm-pcffunction.yang

module _3gpp-5gc-nrm-pcffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-pcffunction;

prefix pcf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the PCF function in 5GC. For more

information about the PCF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-06 { reference "CR-0331"; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-22 {

reference "initial revision";

}

grouping PCFFuntionGrp {

description "Represents the PCFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the PCFFunction supports

(is associated to).";

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the PCFFunction supports

(is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list PCFFunction {

description "5G Core PCF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses PCFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

Next Change

## H.5.20	module _3gpp-5gc-nrm-smffunction.yang

module _3gpp-5gc-nrm-smffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smffunction;

prefix smf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "SMFFunction derived from basic ManagedFunction.";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 {

reference "Ericsson refactoring.";

}

revision 2018-08-07 {

reference "Initial revision";

}

grouping SMFFunctionGrp {

description "Represents the SMFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the SMFFunction supports

(is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the SMFFunction supports

(is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list SMFFunction {

description "5G Core SMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

Next Change

## H.5.22	module _3gpp-5gc-nrm-udmfunction.yang

module _3gpp-5gc-nrm-udmfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udmfunction;

prefix udm3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDM function in 5GC. For more

information about the UDM, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 {

reference "initial revision";

}

grouping UDMFuntionGrp {

description "Represents the UDMFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list UDMFunction {

description "5G Core UDM Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDMFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.23	module _3gpp-5gc-nrm-udrfunction.yang

module _3gpp-5gc-nrm-udrfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udrfunction;

prefix udr3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDR function in 5GC. For more information

about the UDR, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 {

reference "initial revision";

}

grouping UDRFuntionGrp {

description "Representse the UDRFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list UDRFunction {

description "5G Core UDR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDRFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

## H.5.24	module _3gpp-5gc-nrm-udsffunction.yang

module _3gpp-5gc-nrm-udsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udsffunction;

prefix udsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDSF function which can be interacted

with any other 5GC NF defined in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 {

reference "initial revision";

}

grouping UDSFFuntionGrp {

description "Represents the UDSFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Managed Network Function profile";

reference "3GPP TS 23.501";

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list UDSFFunction {

description "5G Core UDSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDSFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;    }

}

}

## H.5.25	module _3gpp-5gc-nrm-upffunction.yang

module _3gpp-5gc-nrm-upffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-upffunction;

prefix upf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "UPFFunction derived from basic ManagedFunction.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 {

reference "Ericsson refactoring.";

}

revision 2018-08-07 {

reference "Initial revision";

}

grouping UPFFunctionGrp {

description "Represents the UPFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types3gpp:Tac;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

reference "3GPP TS 23.003";

uses types3gpp:ManagedNFProfile;

}

leaf-list supportedBMOList {

type string;

description "List of supported BMOs (Bridge Managed Objects) required

for integration with TSN system.";

}

}

augment /me3gpp:ManagedElement {

list UPFFunction {

description "5G Core UPF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UPFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

Next Change

## H.5.27	module _3gpp-5gc-nrm-neffunction.yang

module _3gpp-5gc-nrm-neffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-neffunction;

prefix nef3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NEF function in 5GC. For more

information about the NEF, see 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-20 {

reference "initial revision";

}

grouping NEFFunctionGrp {

description "Represents the NEFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf-list capabilityList {

description "List of supported capabilities of the NEF.";

reference "3GPP TS 23.003";

type string;

}

leaf isINEF {

type boolean;

}

leaf isCAPIFSup {

type boolean;

}

}

augment "/me3gpp:ManagedElement" {

list NEFFunction {

description "5G Core NEF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NEFFunctionGrp;

}

}

}

}

## H.5.28	module _3gpp-5gc-nrm-QFQoSMonitoringControl.yang

module _3gpp-5gc-nrm-QFQoSMonitoringControl {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-QFQoSMonitoringControl;

prefix qFQMCtrl3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the capabilities and properties for control

of QoS monitoring per QoS flow per UE for URLLC service defined

in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-04-10 { reference "S5-202101";   }

grouping QFPacketDelayThresholdsTypeGrp {

description "Represents the QFPacketDelayThresholdsType";

leaf thresholdDl {

type uint32;

units milliseconds;

description "Downlink threshold";

}

leaf thresholdUl {

type uint32;

units milliseconds;

description "Uplink threshold";

}

leaf thresholdRtt {

type uint32;

units milliseconds;

description "Round trip threshold";

}

}

grouping QFQoSMonitoringControlGrp {

description "Represents the QFQoSMonitoringControl IOC.";

reference "3GPP TS 28.541";

leaf qFQoSMonitoringState {

description "The state of QoS monitoring per QoS flow per UE.";

mandatory true;

type enumeration {

enum ENABLED;

enum DISABLED;

}

}

list qFMonitoredSNSSAIs {

description "The S-NSSAIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf-list qFMonitored5QIs {

description "The 5QIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "3GPP TS 23.501";

type uint32 {

range "0..255";

}

}

leaf isEventTriggeredQFMonitoringSupported {

description "It indicates whether the event based QoS monitoring

reporting per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isPeriodicQFMonitoringSupported {

description "It indicates whether the periodic QoS monitoring reporting

per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isSessionReleasedQFMonitoringSupported {

description "It indicates whether the session release based QoS monitoring

reporting  per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

list qFPacketDelayThresholds {

key "idx";

min-elements 1;

max-elements 1;

description "It specifies the thresholds for reporting the packet delay

between PSA and UE for QoS monitoring per QoS flow per UE.";

leaf idx { type uint32 ; }

uses QFPacketDelayThresholdsTypeGrp;

}

leaf qFMinimumWaitTime {

description "It specifies the minimum waiting time (in seconds) between

two consecutive reports for event triggered QoS monitoring reporting

per QoS flow per UE.";

type uint32;

}

leaf qFMeasurementPeriod {

description "It specifies the period (in seconds) for reporting the

packet delay for QoS monitoring per QoS flow per UE.";

type uint32;

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list QFQoSMonitoringControl {

description "Represents the QFQoSMonitoringControl IOC.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses QFQoSMonitoringControlGrp;

}

}

}

}

## H.5.29	module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl.yang

module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-GtpUPathQoSMonitoringControl;

prefix gtpUPathQMCtrl3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the capabilities and properties for control

of GTP-U path QoS monitoring defined in 3GPP TS 23.501.";

reference "3GPP TS 28.541";

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-04-10 { reference "S5-202103"; }

grouping GtpUPathDelayThresholdsType {

leaf n3AveragePacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n3MinPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n3MaxPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9AveragePacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9MinPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9MaxPacketDelayThreshold {

mandatory true;

type uint32;

}

}

grouping GtpUPathQoSMonitoringControlGrp {

description "Represents the GtpUPathQoSMonitoringControl IOC.";

leaf gtpUPathQoSMonitoringState {

description "The state of GTP-U path QoS monitoring.";

mandatory true;

type enumeration {

enum ENABLED;

enum DISABLED;

}

}

list gtpUPathMonitoredSNSSAIs {

key "sd sst";

description "The S-NSSAIs for which the the GTP-U path QoS monitoring is

to be performed.";

reference "3GPP TS 23.003";

uses types5g3gpp:SNssai;

}

leaf-list monitoredDSCPs {

description "The DSCPs for which the GTP-U path QoS monitoring is to be

performed.";

reference "3GPP TS 29.244";

type uint32;

}

leaf isEventTriggeredGtpUPathMonitoringSupported {

description "It indicates whether the event triggered GTP-U path QoS

monitoring reporting based on thresholds is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isPeriodicGtpUMonitoringSupported {

description "It indicates whether the periodic GTP-U path QoS monitoring

reporting is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isImmediateGtpUMonitoringSupported {

description "It indicates whether the immediate GTP-U path QoS monitoring

reporting is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

list gtpUPathDelayThresholds {

key n3AveragePacketDelayThreshold;

// if max-elements is increased later, the key may need to be modified

min-elements 1;

max-elements 1;

description "It specifies the thresholds for reporting the packet delay

for the GTO-U path QoS monitoring.";

uses GtpUPathDelayThresholdsType;

}

leaf gtpUPathMinimumWaitTime {

description "It specifies the minimum waiting time (in seconds) between

two consecutive reports for event triggered GTP-U path QoS monitoring

reporting.";

type uint32;

}

leaf gtpUPathMeasurementPeriod {

description "It specifies the period (in seconds) for reporting the packet

delay for GTP-U path QoS monitoring.";

type uint32;

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list GtpUPathQoSMonitoringControl {

description "Specifies the capabilities and properties for control of

GTP-U path QoS monitoring. For more information about the GTP-U path

QoS monitoring.";

reference "3GPP TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GtpUPathQoSMonitoringControlGrp;

}

}

}

}

End of Change