3GPP TSG SA WG5 Meeting 134-e		TDoc S5-206132

# electronic meeting, online, 16th - 25th November 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0275 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Registration mesurements for SMSF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_SLICE_ePA |  |  |  |  |  | Date: |  |  | 2020-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Measurements for monitoring of SMSF registration procedure are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add TS 29.540 in clause 2, add “Performance measurements for SMSF” in clause 5.x, and add “Monitoring of registration and de-registration procedure for SMS” in A.y. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no measurements for SMSF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.x(new), A.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[3]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[4]	3GPP TS 23.501: "System Architecture for the 5G System".

[5]	IETF RFC 5136: "Defining Network Capacity".

[6]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[9]	3GPP TS 32.425: "Performance Management (PM); Performance measurements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[10]	3GPP TS 32.451: "Key Performance Indicators (KPI) for Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Requirements".

[11]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[12]	Void.

[13]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".[14]	3GPP TS 29.502: "5G System; Session Management Services; Stage 3".

[15]	Void.

[16]	3GPP TS 29.244: "Technical Specification Group Core Network and Terminals; Interface between the Control Plane and the User Plane Nodes; Stage 3".

[17]	ETSI GS NFV-IFA027 v2.4.1: "Network Functions Virtualisation (NFV); Management and Orchestration; Performance Measurements Specification".

[18]	Void.

[19]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[20]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[21]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[22]	3GPP TS 29.413: "Application of the NG Application Protocol (NGAP) to non-3GPP access".

[23]	3GPP TS 29.122: "Technical Specification Group Core Network and Terminals; T8 reference point for Northbound APIs".

[24]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[25]	ETSI ES 202 336-12 V1.2.1: "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[26]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[27]	3GPP TS 29.274: "Evolved General Packet Radio Service (GPRS); Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[28]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[29]	3GPP TS 38.314: "NR; layer 2 measurements".

[30]	3GPP TS 38.313: "Self-Organizing Networks (SON) for 5G networks".

[31]	3GPP TS 38.415: "NG-RAN; PDU session user plane protocol".

[32]	3GPP TS 38.321: "NR MAC protocol specification".

[33]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[34]	3GPP TS 38.215: "NR; Physical layer measurements".

[35]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[36]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[37]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[38]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[X]					3GPP TS 29.540: "5G System; SMS Services; Stage 3".

2nd Modified Section

5.x Performance measurements for SMSF

5.x.y Registration procedure related measurements

5.x.y.1 Number of registration requests

a)	This measurement provides the number of registration requests received by the SMSF from AMF.

b)	CC

c)	SMSF receives the registration (PUT) operation request sent by AMF for the " /ue-contexts/{supi}" resource URL (see clause 5.2.2.2 of 3GPP TS 29.540 [X]). Each registration request is added to the relevant subcounter per S-NSSAI.

d)	Each subcounter is an integer value

e)	SMSF.ActivateReq.SNSSAI

Where SNSSAI identifies the S-NSSAI;

f)	SMSFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

5.x.y.2 Number of successful registrations

a)	This measurement provides the number of successful registrations at the SMSF.

b)	CC

c)	SMSF returns "201 Created" or "204 No Content" response message (see clause 5.2.2.2 of 3GPP TS 29.540 [X]). Each successful registration is added to the relevant subcounter per S-NSSAI.

d)	Each subcounter is an integer value

e)	SMSF.ActivateSucc.SNSSAI

Where SNSSAI identifies the S-NSSAI;

f)	SMSFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

5.x.y.3 Number of de-registration requests

a)	This measurement provides the number of de-registration requests received by the SMSF from AMF.

b)	CC

c)	SMSF receives the de-registration (DELETE) operation request sent by AMF for the " /ue-contexts/{supi}" resource URL (see clause 5.2.2.3 of 3GPP TS 29.540 [X]). Each de-registration request is added to the relevant subcounter per S-NSSAI.

d)	Each subcounter is an integer value

e)	SMSF.DeactivateReq.SNSSAI

Where SNSSAI identifies the S-NSSAI;

f)	SMSFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

5.x.y.4 Number of successful de-registrations

a)	This measurement provides the number of successful de-registrations at the SMSF.

b)	CC

c)	SMSF returns "204 No Content" response message (see clause 5.2.2.3 of 3GPP TS 29.540 [X]). Each successful de-registration is added to the relevant subcounter per S-NSSAI.

d)	Each subcounter is an integer value

e)	SMSF.DeactivateSucc.SNSSAI

Where SNSSAI identifies the S-NSSAI;

f)	SMSFFunction

g)	Valid for packet switched traffic

h)	5GS

i) 	One usage of this performance measurements is for performance assurance.

3rd Modified Section

A.y	Monitoring of registration and de-registration procedure for SMS

In order to activate SMS service for a given service user, a registration procedure for SMS over NAS is needed. Registration procedure related measurements can reflect the success ratio for access to 5GS of SMS users.

De-registration procedure is invoked to deactivate SMS service for a given service user, which results in deleting an individual UE Context for SMS in the SMSF. De-registration procedure related measurements can reflect the success ratio for deactivating SMS service.