3GPP TSG-SA5 Meeting #136e	S5-211103

# 25 January to 03 February 2021, E-meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add measurements on service specific parameter provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2021-01-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | AF may need to provide service specific parameters to 5G system via NEF in order to support the service not provided by the PLMN. The 5GS, after receiving via NEF, delivers the service specific parameters to the target UEs. The service specific parameter provisioning service provided by NEF is critical for the 5GS (including the UEs) to support these services. Therefore, it would be necessary for the operators to monitor the performance of the service specific parameter provisioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the measurements related to service specific parameter provisioning on NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance of service specific parameter provisioning cannot be monitored. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 5.9.x (new), A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This contribution is to be incorporated into the Draft CR for ePM_KPI_5G (see the latest version in S5-205282). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

## 3.3	Measurement family

The measurement names defined in the present document are all beginning with a prefix containing the measurement family name. This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration.

The list of families currently used in the present document is as follows:

-	DRB (measurements related to Data Radio Bearer).

-	RRC (measurements related to Radio Resource Control).

-	UECNTX (measurements related to UE Context).

-	RRU (measurements related to Radio Resource Utilization).

-	RM (measurements related to Registration Management).

-	SM (measurements related to Session Management).

-	GTP (measurements related to GTP Management).

-	IP (measurements related to IP Management).

-	PA (measurements related to Policy Association).

-	MM (measurements related to Mobility Management).

-	VR (measurements related to Virtualized Resource).

-	CARR (measurements related to Carrier).

-	QF (measurements related to QoS Flow).

-	AT (measurements related to Application Triggering).

-	SMS (measurements related to Short Message Service).

-	PEE (measurements related to Power, Energy and Environment).

-	NFS (measurements related to NF service).

-	PFD (measurements related to Packet Flow Description).

-	RACH (measurements related to Random Access Channel)

-	MR (measurements related to Measurement Report)

- 	L1M (measurements related to Layer 1 Measurement)

-	NSS (measurements related to Network Slice Selection)

-	PAG (measurements related to Paging)

-	SPP (measurements related to Service Parameter Provisioning)

Next modified section

### 5.9.x	Service specific parameters provisioning related measurements

#### 5.9.x.1	Service specific parameters creation

##### 5.9.x.1.1	Number of service specific parameters creation requests

a)	This measurement provides the number of service specific parameters creation requests received by the NEF from AF.

b)	CC.

c)	Receipt by the NEF of an Nnef_ServiceParameter_Create request message from AF (see 3GPP TS 23.502 [7]).

d)	A single integer value.

e)	SPP.NbrCreatReq

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.x.1.2	Number of successful service specific parameters creations

a)	This measurement provides the number of successful service specific parameters creations by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ServiceParameter_Create response message to SMF indicating a successful service specific parameters creation (see 3GPP TS 29.522 [a]).

d)	A single integer value.

e)	SPP.NbrCreatSucc

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.x.1.3	Number of failed service specific parameters creations

a)	This measurement provides the number of failed service specific parameters creations by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ServiceParameter_Create response message to AF indicating a failed service specific parameters creation (see 3GPP TS 29.522 [a]), each message increments the relevant subcounter per failure cause by 1.

d)	Each measurement is an integer value.

e)	SPP.NbrCreatFail.cause
Where cause indicates the failure cause of the service specific parameters creation.

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.9.x.2	Service specific parameters update

##### 5.9.x.2.1	Number of service specific parameters update requests

a)	This measurement provides the number of service specific parameters update requests received by the NEF from AF.

b)	CC.

c)	Receipt by the NEF of an Nnef_ServiceParameter_Update request message from AF (see 3GPP TS 23.502 [7]).

d)	A single integer value.

e)	SPP.NbrUpdateReq

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.x.2.2	Number of successful service specific parameters updates

a)	This measurement provides the number of successful service specific parameters updates by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ServiceParameter_Update response message to AF indicating a successful service specific parameters update (see 3GPP TS 29.522 [a]).

d)	A single integer value.

e)	SPP.NbrUpdateSucc

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.x.2.3	Number of failed service specific parameters updates

a)	This measurement provides the number of failed service specific parameters updates by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ServiceParameter_Update response message to AF indicating a failed service specific parameters update (see 3GPP TS 29.522 [a]), each message increments the relevant subcounter per failure cause by 1.

d)	Each measurement is an integer value.

e)	SPP.NbrUpdateFail.cause
Where cause indicates the failure cause of the service specific parameters update.

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.9.x.3	Service specific parameters deletion

##### 5.9.x.3.1	Number of service specific parameters deletion requests

a)	This measurement provides the number of service specific parameters deletion requests received by the NEF from AF.

b)	CC.

c)	Receipt by the NEF of an Nnef_ServiceParameter_Delete request message from AF (see 3GPP TS 23.502 [7]).

d)	A single integer value.

e)	SPP.NbrDelReq

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.x.3.2	Number of successful service specific parameters deletions

a)	This measurement provides the number of successful service specific parameters deletions by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ServiceParameter_Delete response message to AF indicating a successful service specific parameters deletion (see 3GPP TS 29.522 [a]).

d)	A single integer value.

e)	SPP.NbrDelSucc

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

##### 5.9.x.3.3	Number of failed service specific parameters deletions

a)	This measurement provides the number of failed service specific parameters deletions by the NEF.

b)	CC.

c)	Transmission by the NEF of an Nnef_ServiceParameter_Delete response message to AF indicating a failed service specific parameters deletion (see 3GPP TS 29.522 [a]), each message increments the relevant subcounter per failure cause by 1.

d)	Each measurement is an integer value.

e)	SPP.NbrDelFail.cause
Where cause indicates the failure cause of the service specific parameters deletion.

f)	NEFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

Next modified section

# A.x	Monitoring of service specific parameters provisioning

AF may need to provide service specific parameters to 5G system via NEF in order to support the service not provided by the PLMN. The 5GS, after receiving via NEF, delivers the service specific parameters to the target UEs.

The service specific parameter provisioning service provided by NEF is critical for the 5GS (including the UEs) to support these services. Therefore, it would be necessary for the operators to monitor the performance of the service specific parameter provisioning.

End of modified section