3GPP TSG-SA5 Meeting #135e 	S5-211106

# e-meeting, 25 January – 3 February 2021

# Source:	Telefónica S.A.

# Title:	pCR 28.557 Add CAG management

# Document for:	Approval

Agenda Item:	6.4.1

# 1	Decision/action requested

Discuss and approve on the proposal

# 2	References

[1]	TS 28.557 Management of non-public networks; Stage 1 and stage 2 v0.2.0

[2]	TS 23.501 System architecture for the 5G System (5GS); Stage 2 v16.7.0

# 3	Rationale

It is proposed to add concept of CAG management in public integrated network non-public networks in draft TS 28.557 [1].

# 4	Detailed proposal

This document proposes the following changes in TS 28.557 [1].

1st Change

## 4.5	Management of PNI-NPNs

A PNI-NPN is a NPN made available via a PLMN, by means of dedicated DNNs, or by one (or more) Network Slice instances allocated for the NPN [2]. In order to access PNI-NPN, the UE shall have a subscription for the PLMN.

PNI-NPN operation may optionally make use of the concept of Closer Access Group (CAG), which enables the control of UE’s access to PNI-NPN on a per cell basis (CAG cells). The CAG concept is used to prevent UEs which are not allowed to access the PNI-NPN from automating selecting and accessing the associated cell(s). The CAG cell broadcasts information such that only UEs supporting CAG are accessing the cell. This is not possible with network slicing unless an operator specific barring is used.

The PLMN ID identifies the network and the CAG ID identifies the CAG cells. Network selection and reselection is performed based on PLMN ID. Cell selection and reselection, and access control are done based on the CAG ID.

In a PNI-NPN scenario, the CAG management aspects include:

- Assignment and maintenance of CAG IDs.

## Managing the actual list of UEs that are allowed on the CAG. This list must be shared between the MNO (NPN operator and NPN-SP) and the enterprise (NPN-SC).