3GPP TSG-SA5 Meeting #135e	S5-211312

# e-meeting, 25 January – 3 February 2021

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.530 | CR | 0041 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | cleanup concepts of network slice business models |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Telefónica S.A. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The diagrams and description for concept of network slice business model were unclear that caused confusion and misleading. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix diagrams and description for concept of network slice business models |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network slice in NOP internal model and network slice as a service model cannot be easily distinguished that incurred wrong implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.6, 4.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st modification

### 4.1.6	Network Slice as a Service (NSaaS)

Network Slice as a Service (NSaaS) represents a model whereby a NOP offers to its customer a network slice, in the form of a service. In NSaaS, the NOP service offering is a network slice that the customer can use to build a communication service. The customer can provide the communication service based on the sole use of the network slice obtained from the NOP. Alternatively, the customer may require additional NFs to provide the communication service; in such a case, the customer defines its own network/network slice containing the network slice obtained from the NOP as a "building block".

In NSaaS, the customer can have limited level of control over the obtained network slice, according to the NOP exposed management capabilities agreed between the NOP and the customer. The NSaaS offered by the NOP could be characterized by certain properties (capabilities to satisfy service level requirements), e.g.

- 	radio access technology,

-	 bandwidth,

- 	end-to-end latency,

-	 reliability,

- 	guaranteed / non-guaranteed QoS,

- 	security level, etc.

Figure 4.1.6.1 illustrates an example on NSaaS, with a NOP (business entity X) offering a network slice to its customer (business entity Y). As seen, the customer takes here both NOP and CSP roles, combining the received network slice with additional NFs (the customer takes here the NOP role) to build and provide communication services (the customer takes here the CSP role). For simplicity this figure omits the details of how NFs are being managed and does not show their groupings into network slice subnet.

Figure 4.1.6.1: Examples of Network Slice as a Service (NSaaS) being utilized to deliver communication services to customers

NOTE 1: “network slice” shown in left side of the figure is a NetworkSlice instance created by the business entity X,  which offered to the business entity Y as “network slice” shown in the right side of the figure. The business entity Y uses Distinguished Name to refer to and access the network slice created in the business entity X.

NOTE 2: Depending on the use case, the business entity Y could represent a vertical customer or another MNO.

### 4.1.7	Network slices as NOP internals

In the "network slices as NOP internals" model, network slices are not part of the service offering and hence are not visible to its external customers. However, the NOP, to provide support to communication services, may decide to deploy network slices, e.g. for internal network optimization purposes. This model allows CSC to use the network as the end user or vertical customer, and optionally allows CSC to monitor the service status (assurance of the SLA associated with the internally offered network slice) via the management exposure interface provided by the CSP.

Figure 4.1.7.1 illustrates an example on how network slices can be utilized to deliver communication services. In this example, one business entity (X) takes the role of both NOP and CSP to provide communication services to a CSC based on a network slice. Only communication service is exposed to the CSC. The CSC should be able to monitor the communication service status information (e.g. service performance, fault information, traffic data, etc.).

Figure 4.1.7.1: Examples of network slice as NOP internals

End of 1st modification