3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211107

- electronic meeting, online, 25 January - 3 February 2021

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Introduction of background on NS management charging information

- Document for:	Approval

Agenda Item:	7.5.5

# 1	Decision/action requested

This pCR is to introduce some background on NS management charging information

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[3]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[4]	GSMA 5GJA NG.116: "Generic Network Slice Template".

# 3	Rationale

The pCR proposes to introduce some background on NS charging information focusing on charging information obtained per TS 28.202 [2].

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

# 4	General background

## 4.x	Network Slice Management Charging

Network Slice Management Charging specified by 3GPP TS 28.202 [x], enables the Converged Charging System (CCS) to capture for each NetworkSlice instance per TS 28.530 [x] definition, the supported list of service profiles representing the Service Level Specification (SLS) between Network Slice Customer (NSC) and Network Slice Provider (NSP).

For each S-NSSAI, associated attributes and their values representing the Service Level Specification (SLS) are available to the Converged Charging system, via service profiles captured with the NetworkSlice instance.

These informations are captured in NSM charging CHF record generated by CHF upon NetworkSlice instance creation,  modification and termination.

End of changes