- 3GPP TSG SA WG5 Meeting #135-e	S5-211117

- January 25 – Feburary 3, 2021, e-Meeting	revision of S5-21abcd

- Source:	Intel

- Title:	pCR 28.814 UC of UPF selection to support EAS deployment

- Document for:	Approval

Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 	TR 28.814 “Study on enhancements of edge computing management”

# 3	Rationale

An EAS deployed in the EDN needs to be connected to the UPF in 5GC via the N6 interface, as shown in Figure 4.1-2 in [1], in order to supporting UEs to communicate with EAS. This contribution proposes to add a use case to TR 28.814 [1] to enable ECSP to request 3GPP management system to select a UPF, based on the service area and QoS requirements.

# 4	Detailed proposal

1st modified section

## 6.2	Specification level use cases and requirements

### 6.2.x	UPF selection for EAS

#### 6.2.x.1	Goal

The goal is to enable ECSP management system to query 3GPP management system to select a UPF among the UPF(s) being deployed, or instantiate a new UPF if nothing available, based on the service area and QoS requirements. The UPF being selected will be connected to the EAS via the N6 interface to support edge computing applications.

#### 6.2.x.2	Description

1. ECSP management system queries 3GPP management system about the information of a UPF that will be connected to the EAS, by providing the UPF selection requirements, including service area requirements (e.g. Topological Service Area, Geographical Service Area, (see subclause 7.3.3 in TS 28.558 [2])) and list of N6 traffic routing requirements (see Table 8.2.4.1 in Ts 23.558 [2]).

2. 3GPP management system finds a UPF among the UPF(s) being deployed that meets the EAS service area and QoS requirements.

3. 3GPP management system deploys a new UPF if no UPF can be found.

4. 3GPP management system provides the UPF information to the ECSP management system.

#### 6.2.x.3	Requirements

REQ-UPF-FUN-1 3GPP management service producer should have the capability allowing authorized consumer to request for the information of a UPF that will be connected to EAS, based on UPF selection requirements (e.g. service area and QoS requirements).

REQ-UPF-FUN-2 3GPP management service producer should have the capability to provide the UPF information to the consumer.

End of modified section