3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211163rev1

- electronic meeting, online, 25 January - 3 February 2021

- Source:	Huawei

- Title:	Add use case NIDD for control plane data transfer charging

- Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TR 28.816

# 3	Rationale

This contribution is to add a use case for NIDD regarding control plane data transfer.

# 4	Detailed proposal

(For pseudo CR, include the complete clause(s) or subclause(s) of the latest draft TS/TR to be modified, with clear clause and sub-clause headings included and all modifications shown with revision marks, unambiguously showing where the changes shall be made or inserted in the draft TS/TR. It is not sufficient to just state, for example, “add the following text to the draft TS/TR…”.)

The first chagne

### 5.1.y	Use case #y: Non-IP Data Delivery (NIDD) in control plane data transfer domain charging

Functions of NIDD is a 5GS CIoT to handle Mobile Originated (MO) and Mobile Terminated (MT) communication for unstructured data (also referred to as Non-IP), that delivery to the AF using the NIDD API or using the UPF via a PtP N6 tunnel.

The use case begins during the PDU session establishment, the UE may obtain an Unstructrured PDU session to the NEF and a NEF Identity for NIDD may be presented. The determination of NIDD API invoking for a PDU session depends on the presence of NEF Identity for NIDD for the DNN/S-NSSAI combination in the subscription. SMF selects that NEF and uses the NIDD API for the PDU session if determines.

The NEF is responsible to expose the NIDD API on N33/Nnef reference point.

In control plan data transfer domain charging, offline charging is supported by Control Plane data transfer Charging Nodes (CPCN), with the CTF. The 5GC NFs (e.g., AMF, SMF and NEF) may collect the information on the use of NIDD API and generating the charging information.

The 5GS CIoT charging may require 5GS NFs to support collecting the uses of NIDD API during a PDU session.

The potential charging requirements for control plane data transfer domain charging are:  REQ-3GPPCH-5GSCIoT-xx, REQ-3GPPCH-5GSCIoT-yy.

Second change

## 5.2 	Potential charging requirements

REQ-3GPPCH-5GSCIoT-01	The charging aspect of 5G data connectivity domain charging shall support charging of Control Plane CIoT 5GS Optimisation.

REQ-3GPPCH-5GSCIoT-xx  The 5GS CIoT charging should support charging of control plane data transfer based on 5G system architecture.

REQ-3GPPCH-5GSCIoT-yy  The charging aspect of control plane data transfer should support collecting charging information related to Non-IP Data Delivery (NIDD) and support charging the use of NIDD API based on 5G system architecture.

End of change