3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211278

- electronic meeting, online, 25 January - 3 February 2021

- Source:	Ericsson LM

- Title:	Requirements for 5GS usage based charging

- Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.815.

# 2	References

[1]	3GPP TR 28.815: "Charging management; Study on charging aspects of edge computing ".

# 3	Rationale

The clause 7.1.1 handles 5GS capability usage based charging and collecting information for this, therefor requirements not related to 5GS capability usage should be removed and 5GS capability usage for other parties than the owner of the UE should be added.

# 4	Detailed proposal

First change

## 7.1	5GS capability usage based charging for edge computing

### 7.1.1	Use cases

#### 7.1.1.x	Use case #x: MNO charging subscriber for using edge application based on 5GS capability usage

This use case focuses on MNO and subscriber business roles.

A UE has a subscription with the MNO which allow usage of edge applications.

5GS provides the capabilities to support edge computing, and the MNO may charge these 5GS capabilities in the following ways:

- 	Charge the individual subscribers for using the 5G capabilities supporting edge computing for accessing edge applications.

The subscriber charging could be based on data volume or in-session (PDU session) time that the UE has been connected to the edge applications, and potentially with consideration of the following factors:

-	User’s location;

-	Access RAT type;

-	Subscribed QoS information;

-	Authorized QoS information;

7.1.1.y	Use case #y: MNO charging ECSP for 5GS capabilities usage

This use case focuses on MNO and ECSP business roles.

The ECSP has an agreement to use the 5GS capabilities of the MNO.

-	Charge the ECSP for the 5GS capabilities provided for edge computing.

The inter-provider charging by MNO could be based on the total data volume transferred in the PLMN for the edge data network.

#### 7.1.1.z	Use case #z: ECSP charging ASP for edge enabling service including 5GS capabilities usage

This use case focuses on ECSP and ASP business roles.

The ASP has an agreement to use the edge enabling service of the ECSP, including the 5GS capabilities.

-	Charge the ASP for the edge enabling service including the 5GS capabilities.

The inter-provider charging by ECSP could be based on the total data volume transferred for the edge application.

### 7.1.2	Potential charging requirements

REQ-CH_EC_5GS-01: The SMF should support the following extensions to the charging requirements defined in TS 32.255 [6]:

-	The SMF should support converged charging for each edge application, i.e., charging per rating group per PDU session with information about edge application.

-	The charging information of PDU session, service flow or QoS flow provided by SMF should be able to indicate or be correlated with the edge application that the user accesses to.

-	The SMF should support converged charging and charging information reporting for each edge application optionally based on the following criteria:

-	User’s location;

-	Access RAT type;

-	Subscribed QoS information;

-	Authorized QoS information.

REQ-CH_EC_5GS-02:	The 5G system should support collecting charging information per UE for usage of edge application.

REQ-CH_EC_5GS-03:	The 5G system should support collecting charging information per edge data network.

REQ-CH_EC_5GS-04:	The 5G system should support collecting charging information per edge application.

7.1.3	Key issues

The following key issues are identified:

-	Key Issue #1a: The charging information reported by SMF needs to contain the parameter for correlation between 5GS and edge application that the user has accessed to.

-	Key Issue #1b: The charging mechanism for 5GS (including the management system) needs to support converged inter-Provider charging for the 5G capabilities provided to support edge computing to its consumer based on the total UL/DL data volume transferred for the edge application.

### 7.1.4	Possible solutions

#### 7.1.4.1	Possible solutions for subscriber based charging

The possible solutions to support the potential requirements REQ-CH_EC_5GS-01, REQ-CH_EC_5GS-02 and Key Issue #1a could be the following enhancements or extensions to the PDU session charging solutions defined in TS 32.255 [6]:

Solution #1a: Reuse the “AF Charging Identifier” (see TS 32.255 [6] and TS 29.512 [8]) or “afChargId” (see TS 29.512 [8])  parameter for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “AF Charging Identifier” or “afChargId” parameter is provided to SMF in the PCC rule for one or more service flows and could be contained in the “PDU Container Information” that is provided in the Charging Data Request sent by SMF and CHF record data.

Solution #1b: Reuse the “Network Slice Instance Identifier” parameter (see TS 32.255 [6]) for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Network Slice Instance Identifier” parameter is contained in the “PDU Session Charging Information” and needs to be provided in the Charging Data Request sent by SMF and CHF record data.

The assumption for this solution is that each edge application is associated with a dedicated network slice.

Solution #1c: Use the “Application Identifier”, “flowDescription” or “ethFlowDescription”  parameter (see “appId” in TS 29.512 [8]) for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Application Identifier”, “flowDescription” or “ethFlowDescription” parameter is provided to SMF in the PCC rule for one or more service flows. Including either of these parameters in the Charging Data Request sent by SMF and CHF record data would enable the CHF to correlate them with the edge application.

Solution #1d: Use the “Rating Group” parameter for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Rating Group” is provided to SMF in the PCC rule for one or more service flows.

Solution #1e: Use the “Service Identifier” parameter for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Service Identifier” is provided to SMF in the PCC rule for one or more service flows.

Solution #1f: Use the “Sponsor Identity” parameter for correlating the Charging Data Request, charging information and CHF record data with the edge application for PDU session charging.

The “Sponsor Identity” are provided to SMF in the PCC rule for one or more service flows.

#### 7.1.4.x	Possible solutions for inter-provider based charging

The possible solutions to support the potential requirements REQ-CH_EC_5GS-03, REQ-CH_EC_5GS-04 and Key Issue #1b

Solution #2a could be the reusing the information from Key Issue 1a and with the addition of the edge data network and having the CHF or CGF aggregating the usage per edge application and/or edge data network.

End of changes