3GPP TSG SA WG5 Meeting 135-e		TDoc S5-211283rev3

- electronic meeting, online, 25 January - 3 February 2021

- Source:	Ericsson

- Title:	pCR 28.811 Multi-operator concepts

- Document for:	Approval

Agenda Item:	6.5.5

# 1	Decision/action requested

The group is asked to agree on the proposal.

# 2	References

[1]	3GPP TR 28.811 v 0.2.0 : "Management and orchestration; Network Slice Management Enhancement"

# 3	Rationale

To be able to study multi-operator network slice management scenarios, it is important to understand and take into consideration the existing multi-operator scenarios already deployed pre network slicing.

Further it is important to study how those existing scenarios (and new ones) can be enhanced to also support network slicing in multi-operator slice scenarios.

# 4	Detailed proposal

To add updates to clause 2, and to add concepts into clause 4 of TR 28.811 v0.2.0.

Start of 1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.501 v16.6.0: "System architecture for the 5G System (5GS); Stage 2"

[2]	3GPP TS 28.541 v17.0.0: "5G Network Resource Model (NRM); Stage 2 and stage 3"

[3]	3GPP TS 28.531 v16.7.0: "Management and orchestration; Provisioning"

[4]	3GPP TS 28.530 v16.3.0: "Management and orchestration; Concepts, use cases and requirements"

[x]	3GPP TS 22.011 v17.3.0 “Service accessibility”

Next Change

## 4.x	Multi-operator concepts

In this clause different Multi-operator concepts are described.

#### 4.x.1	National roaming concept

National Roaming is defined in TS 22.011 [x] as a service whereby a UE of a given PLMN is able to obtain service from another PLMN of the same country, anywhere, or on a regional basis. It is used today e.g between EPS networks. In the following figure, Network operator A and B have a national roaming agreement and subscribers of network operator A can use the 5GS network of network operator B, while using the communication services of network operator A.

Provider

Provider

Client

Client

Communication Service Customer

Communication Service Provider A

Communication Service Provider B

Network

Operator A

Communication Service Customer

Client

Provider

Radio Network

A

Radio Network

B

Communication Service Provider B

Network

Operator B

Figure 4.x.1.1: National roaming concept

Management interactions between and by network operators involved here: both the network operator A and B have instantiated network slices. Network operator B determines which network slices are allowed to be used by inbound roamers from network operator A, and if needed, will configure its network to support mapping of S-NSSAI as defined in TS 23.501 [1].

If subscribers of network operator A are visiting the network of network operator B the roaming principles as standardized in TS 23.501 [1] apply, i.e., the UE of network operator A will register into 5GC with an AMF in network operator B, and will receive Allowed NSSAI and other network slicing related information as needed. In case of home routing, PDU sessions with an S-NSSAI are established with V-SMF in network operator B and H-SMF in network operator A.

End of Change