3GPP TSG SA WG5 Meeting 136-e		TDoc S5-212098

# electronic meeting, online, 1 - 9 March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0003 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Registration flows and message content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_AMFCH |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In some steps description for registration flows Accounting word is used instead of Account Registration with AMF re-allocation flow not aligned with TS 23.502 Charging Data Request description misaligned with TS 32.290 The fields setting in Charging Data Response description does not reflect the list of Presence Reporting Area(s) subscription by the CHF. The User Location Information includes the User location Time |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Accounting to Account Align Registration with AMF re-allocation flow with TS 23.502 Add missing information in Charging Data Request description Add Location Reporting Charging information in Charging Data Response for the list of Presence Reporting Area(s) subscription by the CHF Remove the "User location Time" explicit field from Registration, N2 connection and Location reporting charging information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PRA functionality is not complete for AMF charging, and some missing information will lead to incorrect AMF charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 3.3, 5.2.2.2.3, 5.2.2.2.4, 5.2.2.2.5, 5.2.2.5.4, 6.1.1.2, 6.1.1.3, 6.2.1.2, 6.2.1.3, 6.2.1.4, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bam	Reference point for the CDR file transfer from the 5G connection and mobility CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and the CGF.

Nchf	Service based interface exhibited by CHF.

N41	Reference point between AMF and the H-CHF.

N42	Reference point between AMF and the V-CHF.

Next change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

5GC	5G Core Network

5GS	5G System

5G-AN	5G Access Network

AMF	Access and Mobility Management Function

BD	Billing Domain

CCS	Converged Charging System

CDF	Charging Data Function

CGF	Charging Gateway Function

CHF	Charging Function

CTF	Charging Trigger Function

DNN	Data Network Name

ECUR	Event Charging with Unit Reservation

GPSI	Generic Public Subscription Identifier

H-CHF	A CHF in HPLMN

IEC	Immediate Event Charging

GUAMI	Globally Unique AMF Identifier

MICO	Mobile Initiated Connection Only

N3IWF	Non-3GPP InterWorking Function

NF	Network Function

NRF	Network Repository Function

PEC	Post Event Charging

PEI	Permanent Equipment Identifier

SUPI	Subscription Permanent Identifier

UDM	Unified Data Management

UDR	Unified Data Repository

V-CHF	A CHF in VPLMN

Next change

##### 5.2.2.2.3	General Registration – IEC

The following figure 5.2.2.2.3.1 describes a Registration charging message flows in IEC, based on the clause 5.2.2.2.2 Registration description:

Figure 5.2.2.2.3.1: Registration – IEC

1-14c.	Registration procedure initiated by UE.

15-16: AM policy association procedure.

16ch-a. The AMF sends Charging Data Request [Event] to CHF for the UE registration to be granted authorization, rated and accounted.

16ch-b. Account and Rating control by the CHF for the registration and CDR creation.

16ch-c. The CHF acknowledges by sending Charging Data Response [Event] and the AMF, granting authorization to the registration.

17 to 24. Same steps as in Figure 5.2.2.2.2.1.

Next change

##### 5.2.2.2.4	General Registration – ECUR

The following figure 5.2.2.2.4.1 describes a Registration charging message flows in ECUR, based on the clause 5.2.2.2.2 Registration description:

Figure 5.2.2.2.4.1: Registration – ECUR

1-14c.	Registration procedure initiated by UE.

15-16: AM policy association procedure.

16ch-a. The AMF sends Charging Data Request [Initial] to CHF for the UE registration to be granted authorization.

16ch-b. Account and Rating control by the CHF for the registration and open a CDR.

16ch-c. The CHF acknowledges by sending Charging Data Response [Initial] and the AMF, granting authorization to the registration.

17 to 21. Same steps as in Figure 5.2.2.2.2.1.

21ch-a. The AMF sends Charging Data Request [Termination] to CHF for the UE successful registration.

21ch-b. The CHF closes the CDR for this registration.

21ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the AMF.

Next change

##### 5.2.2.2.5	Registration with AMF re-allocation – PEC charging

The following figure 5.2.2.2.5.1 describes a registration with AMF re-allocation PEC charging, based on figure 4.2.2.2.3-1 TS 23.502 [202] description:

Figure 5.2.2.2.5.1: Registration with AMF re-allocation– PEC charging

1-7b.	Registration with AMF re-allocation procedure.

21ch-a to 21ch-c: same steps as per Figure 5.2.2.2.2.1.

4-24b. Registration procedure in PEC charging in clause 5.2.2.2.2.

Next change

##### 5.2.2.5.4	Registration – ECUR to HPLMN

The following figure 5.2.2.5.4.1 describes a Registration charging message flows in ECUR to HPLMN scenario for roaming, based on the clause 5.2.2.2.2 Registration description:

Figure 5.2.2.5.4.1: Roaming Registration – ECUR to HPLMN

1-16.	Registration procedure initiated by UE per steps 1 to 16 of figure 4.2.2.2.2-1 of TS 23.502, with PCF, AUSF and UDM in HPLMN.

16ch-a. Upon Registration checked trigger, the AMF sends Charging Data Request [Initial] to H-CHF for the UE registration to be granted authorization, indicating "roamer out".

16ch-b. Account and Rating control by the H-CHF for the registration and open a CDR.

16ch-c. The H-CHF acknowledges by sending Charging Data Response [Initial] to the AMF in VPLMN, granting authorization to the registration.

17 to 21. Same steps as in Figure 5.2.2.2.2.1.

21ch-a. The AMF in VPLMN sends Charging Data Request [Termination] to H-CHF for the UE successful registration.

21ch-b. The H-CHF closes the CDR for this registration.

21ch-c. The H-CHF acknowledges by sending Charging Data Response [Termination] to the AMF in VPLMN.

21ch-d. The AMF in VPLMN sends Charging Data Request [Event] to V-CHF for the UE successful registration, indicating "roamer in".

21ch-e. The V-CHF creates the CDR for this registration.

21ch-f. The V-CHF acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

Next change

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the AMF as used for 5G connection and mobility converged charging.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57] In case Subscriber Identifier (i.e. SUPI) is not present (for emergency service), the User Equipment Info in table 6.2.1.2.1. shall be present for identifying the user. |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | M | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Charging Identifier | - | This field is not applicable. |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in TS 32.290 [57] |
| One-time Event Type | OC | Described in TS 32.290 [57] |
| Notify URI | - | This field is not applicable. |
| Service Specification Information | OC | Described in TS 32.290 [57] |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G connection and mobility specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57]. |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| AMF Identifier | OC | This field holds the AMF identifier. |
| AMF Charging Profile | OC | This field holds the applicable AMF Charging Profile |
| Registration Charging Information | OM | This field holds the registration specific information described in clause 6.2.1.2 |
| N2 connection Charging Information | OM | This field holds the N2 connection specific information described in clause 6.2.1.3 |
| Location Reporting Charging information | OM | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

Next change

#### 6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for 5G connection and mobility converged charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | This field is not applicable. |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the 5G connection and mobility specific triggers described in clause 5.2.1. |
| Multiple Unit Information | OC | This field is applicable for ECUR. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | OM | Described in TS 32.290 [57] |
| Granted Unit | OC | Described in TS 32.290 [57] |
| Validity Time | OC | Described in TS 32.290 [57] |
| AMF Charging Profile | OC | This field holds the AMF Charging Profile to be applied |
| Location Reporting Charging information | Oc | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

Next change

#### 6.2.1.2	Definition of Registration charging information

Specific charging information used for 5G registration charging is provided within the Registration Charging Information.

The detailed structure of the Registration Charging Information can be found in table 6.2.1.2.1.

Table 6.2.1.2.1: Structure of Registration Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Registration message type | M | This field holds the message type received by the AMF associated to registration: registration (initial, mobility, periodic, emergency), deregistration. |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It may be used for identifying the user and shall be present in case Subscriber Identifier is not present. |
| unauthenticatedFlag | OC | This field indicates the served SUPI is not authenticated. |
| Roamer In Out | OC | This field holds an indication if the roamer is in-bound or out-bound. This field is present only if UE is identified as a roamer. |
| UE MM Core Network Capability | OC | This field holds the UE MM Core network capabilities, as specified in clause 5.4.4a TS 23.501 [200] |
| MICO Mode Indication | OC | This field indicates whether the MICO Mode is applicable or not. |
| SMS Supported Indication | OC | This field indicates whether SMS is supported or not. |
| RAT Type | OC | This field holds the Radio Access Technology (RAT) currently serving the UE. |
| TAI List | OC | This field holds a set of tracking areas composing the Registration Area. |
| User Location Info | OC | This field indicates details of where the UE is currently located (access-specific user location information). |
| PSCell Information | OC | This field holds the PSCell: Primary SCG (Secondary Cell Group) Cell |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| Mobility Restrictions | OC | This field holds the Mobility Restrictions applicable to the UE: RAT restriction, Forbidden area, Service area restrictions and Core Network type restriction. |
| Requested NSSAI | OC | This field holds the requested NSSAI. |
| Allowed NSSAI | OC | This field holds the allowed NSSAI consisting of one or more S-NSSAIs for serving PLMN in the present Registration Area. |
| Rejected NSSAI | OC | This field holds the rejected NSSAI. |
| NSSAI mapping list | OC | This field holds mapping of each S-NSSAI of the Allowed NSSAI to the S-NSSAIs of the Subscribed S-NSSAIs. |
| AMF UE NGAP ID | OM | This fields holds the UE association over the N2 interface within the AMF. |
| RAN UE NGAP ID | OM | This fields holds the RAN UE NGAP ID over N2 interface. |
| RAN Node Id | OM | This fields holds the Global RAN Node ID. |

Next change

#### 6.2.1.3	Definition of N2 Connection charging information

Specific charging information used for 5G N2 Connection charging is provided within the N2 connection charging Information.

The detailed structure of the N2 connection Charging Information can be found in table 6.2.1.3.1.

Table 6.2.1.3.1: Structure of N2 Connection Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| N2 Connection message type | M | This field holds the N2 message type received by the AMF. |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. |
| unauthenticatedFlag | OC | This field indicates the served SUPI is not authenticated. |
| User Location Info | OC | This field indicates details of where the UE is currently located (access-specific user location information). |
| PSCell Information | OC | This field holds the PSCell: Primary SCG (Secondary Cell Group) Cell |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| RAT Type | OC | This field holds the Radio Access Technology (RAT). |
| AMF UE NGAP ID | OM | This fields holds the UE association over the N2 interface within the AMF. |
| RAN UE NGAP ID | OM | This fields holds the RAN UE NGAP ID over N2 interface. |
| RAN Node Id | OM | This fields holds the Global RAN Node ID. |
| Mobility Restrictions | OC | This field holds the Mobility Restrictions applicable to the UE: RAT restriction, Forbidden area, Service area restrictions and Core Network type restriction. |
| Allowed NSSAI | OC | This field holds the allowed NSSAI consisting of one or more S-NSSAIs for serving PLMN in the present Registration Area. |
| RRC Establishment Cause | OC | This fields holds the reason for RRC Connection Establishment as received from the UE. |

Next change

#### 6.2.1.4	Definition of Location Reporting charging information

Specific charging information used for 5G Location Reporting charging is provided within the Location reporting charging Information.

The detailed structure of the Location reporting charging Information can be found in table 6.2.1.4.1.

Table 6.2.1.4.1: Structure of Location Reporting Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Location reporting message type | M | This field holds the location reporting message type received by the AMF. |
| User Information | OM | Group of user information. |
| User Identifier | OC | This field contains the identification of the user (i.e. GPSI). |
| User Equipment Info | OC | This field holds the identification of the terminal (i.e. PEI, MAC Address) It is used for identifying the user in case SUPI is not present during emergency service. |
| unauthenticatedFlag | OC | This field indicates the served SUPI is not authenticated. |
| User Location Info | OM | This field indicates details of where the UE is currently located under the access type. |
| PSCell Information | OC | This field holds the PSCell: Primary SCG (Secondary Cell Group) Cell |
| UE Time Zone | OC | This field holds the Time Zone of where the UE is located, if available where the UE currently resides. |
| Presence Reporting Area Information | OC | This field contains the Presence Reporting Area Information of UE as defined in TS 23.501 [200], comprising the Presence Reporting Area identifier(s) and an indication on whether the UE is inside or outside the Presence Reporting Area, if available. |
| RAT Type | OC | This field holds the Radio Access Technology (RAT). |

Next change

### 6.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by AMF for 5G connection and mobility converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.3.1 defines the basic structure of the supported fields in the Charging Data Request message for AMF converged charging.

Table 6.2.3.1: Supported fields in Charging Data Request message

| Information Element | Functionality of AMF | Reg. | N2 cnt | Loc. Report. |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | I/T/E | E | E |
| Session Identifier |  | ITE | E | E |
| Subscriber Identifier |  | ITE | E | E |
| NF Consumer Identification |  | ITE | E | E |
| Charging Identifier |  | - | - | - |
| Invocation Timestamp |  | ITE | E | E |
| Invocation Sequence Number |  | ITE | E | E |
| Retransmission Indicator |  | - | - | - |
| One-time Event |  | --E | E | E |
| One-time Event Type |  | --E | E | E |
| Notify URI |  | - | - | - |
| Service Specification Information |  | ITE | E | E |
| Triggers |  | ITE | - | - |
| Multiple Unit Usage |  | ITE | - | - |
| AMF Identifier |  | ITE | E | E |
| AMF Charging Profile |  | I-E | E | E |
| Registration Charging Information |  |  |  |  |
| Registration Message type |  | ITE | - | - |
| User Information |  | ITE | - | - |
| UE MM Core Network Capability |  | ITE | - | - |
| MICO Mode Indication |  | ITE | - | - |
| SMS Supported Indication |  | ITE | - | - |
| Access Type |  | ITE | - | - |
| RAT Type |  | ITE | - | - |
| TAI List |  | ITE | - | - |
| User Location Info |  | ITE | - | - |
| PSCell Information |  | ITE | - | - |
| UE Time Zone |  | ITE | - | - |
| Mobility Restrictions |  | ITE | - | - |
| Requested NSSAI |  | ITE | - | - |
| Allowed NSSAI |  | ITE | - | - |
| Rejected NSSAI |  | ITE | - | - |
| NSSAI mapping list |  | ITE | - | - |
| AMF UE NGAP ID |  | ITE | - | - |
| RAN UE NGAP ID |  | ITE | - | - |
| RAN Node Id |  | ITE | - | - |
| N2 Connection Charging Information |  |  |  |  |
| N2 Connection Message type |  | - | E | - |
| User Information |  | - | E | - |
| User Location Info |  | - | E | - |
| PSCell Information |  | - | E | - |
| UE Time Zone |  | - | E | - |
| RAT Type |  | - | E | - |
| AMF UE NGAP ID |  | - | E | - |
| RAN UE NGAP ID |  | - | E | - |
| RAN Node Id |  | - | E | - |
| Mobility Restrictions |  | - | E | - |
| Allowed NSSAI |  | - | E | - |
| RRC Establishment Cause |  | - | E | - |
| Location Reporting Charging information |  |  |  |  |
| Location reporting Message type |  | - | - | E |
| User Information |  | - | - | E |
| User Location Info |  | - | - | E |
| PSCell Information |  | - | - | E |
| UE Time Zone |  | - | - | E |
| Presence Reporting Area Information |  | - | - | E |
| RAT Type |  | - | - | E |

Table 6.2.3.2 defines the basic structure of the supported fields in the Charging Data Response message for AMF converged charging.

Table 6.2.3.2: Supported fields in Charging Data Response message

| Information Element | Functionality of AMF | Reg. | N2 cnt | Loc. Report. |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | ITE | E | E |
| Session Identifier |  | ITE | E | E |
| Invocation Timestamp |  | ITE | E | E |
| Invocation Result |  | ITE | E | E |
| Invocation Sequence Number |  | ITE | E | E |
| Session Failover |  | I-- | - | - |
| Triggers |  | - | E | E |
| Multiple Unit information |  | - | - | - |
| AMF Charging Profile |  | I-E | E | E |
| Location Reporting Charging information |  |  |  |  |
| Location reporting Message type |  | - | - | - |
| User Information |  | - | - | - |
| User Location Info |  | - | - | - |
| PSCell Information |  | - | - | - |
| UE Time Zone |  | - | - | - |
| Presence Reporting Area Information |  | - | - | E |
| RAT Type |  | - | - | - |

End of changes