3GPP TSG SA WG5 Meeting 136-e		TDoc S5-212149

- electronic meeting, online, 1 - 9 March 2021

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Introduction of NS converged charging sol#1.1 single CCS for KI#1

- Document for:	Approval

Agenda Item:	7.5.5

# 1	Decision/action requested

This pCR is to introduce Network slice converged charging sol#1.1 single CCS for KI#1

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 32.845 Study on Charging Aspects of Network Slicing

[3]	3GPP TS 28.202 "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[4]	3GPP TS 28.201 "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2 ".

[5]	GSMA 5GJA NG.116: "Generic Network Slice Template".

# 3	Rationale

This solution sol#1.1 is for Key issue#1 considering REQ-NSCH-01, REQ-NSCH-02, based on a single CCS.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

### 6.1.2	Solution#1-1 NB of UEs based Network slice converged charging -  Single CCS

#### 6.1.2.1	General description

This solution addresses the Key Issue#1 for REQ-NSCH-01, REQ-NSCH-02 and REQ-NSCH-08 potential requirements and is based on CHF/5G Converged Charging System capabilities.

In this solution there is a single Converged Charging System (CCS) with its functional modules:

-	Charging Function (CHF) exposing Nchf_ConvergedCharging service

-	Account Balance Management Function (ABMF) hosting subscribers’s account balance

-	Charging Gateway Function (CGF) for CDRs files transfer to the BD.

-	Rating Function (RF) for rating the resource usage.

Both types of subscriber are handled by the CCS, i.e Network Slice tenant and end-user, respectively referred-to as NS-Tenant and UE for the sake of simplification:

-	Nchf_ConvergedCharging service is exposed by CHF for individual UE.

-	ABMF contains NS-Tenant account balance as well as UE account balance.

-	CDRs relate to both UE and NS-Tenant.

-	Rating Function (RF) is for rating the resource usage by the NS-Tenant as well as the resource usage by the UE.

How the account balance is provisioned with the subscriber's profile (for both NS-tenant and individual end-user) and various subscribed-to services is unspecified and expected to be covered at BSS level.

The NS-Tenant account balance is assumed to be provisioned with all necessary information related to the Network Slice matching SLS requirements the NS-Tenant has paid for. The subscription description includes in particular the list of service profiles subscribed-to by the NS-Tenant, each service profile with assigned group of S-NSSAI(s).

The individual UE account balance is assumed to be provisioned with all necessary information related to subscribed-to services the UE has paid for. The subscription description includes in particular the subscribed NSSAI (set of S-NSSAI(s)).

The Registration management charging from AMF specified in TS 32.256 [x] is used for UE registration control by CCS.

The converged charging on Nb of UEs per S-NSSAI is based on CCS aggregating individual UEs registration under converged charging.

The converged charging on Nb of UEs per NS-Tenant is based on CCS aggregating S-NSSAI(s) from individual UEs registration under converged charging.

Editor's Note: the case of S-NSSAi shared between NS-tenants needs to be clarified

#### 6.1.2.2	Architecture description

The architecture for this solution is the existing TS 32.256 [x] 5G connection and mobility converged charging:

Figure 6.1.2.2.1: "Nb of UEs" based Network Slice Converged charging architecture

#### 6.1.2.3	Flow description

The figures 6.1.2.3.1 and 6.1.2.3.2 describe the high level charging procedure for Network slice converged charging based on number of UEs determined by registration procedures with a single CCS.

Figure 6.1.2.3.1: "Nb of UEs" based Network slice converged charging - Registration

The steps are according to TS 32.256 [x] clause 5.2.2.2.3 with the following additions:

0ch.	CCS ABMF maintains per NS-Tenant S-NSSAI the whole "Nb of UEs" quota when relevant.

16ch-a and b1.	Same as 16ch-a and b TS 32.256 [x]

16ch-b2.	For each allowed S-NSSAI in the received Allowed NSSAI for the individual UE: if sufficient quota of Nb of UEs for this S-NSSAI on the NS-Tenant's account balance, the CHF decreases the "Nb of UEs" quota by one, otherwise the S-NSSAI is set as "rejected" by the CHF for the UE. If the full set of S-NSSAI received under the Allowed NSSAI are "rejected", the individual UE registration is rejected.

16ch-b3.	UE CDR creation for the registration

16ch-c.	CHF provides appropriate answer: either authorizes the registration with the "Allowed NSSAI" updated with "rejected" S-NSSAI(s) or rejects the registration with appropriate cause.

Figure 6.1.2.3.2: "Nb of UEs" based Network slice converged charging - Deregistration

The steps are according to TS 32.256 [x] clause 5.2.2.2.8 with the following additions:

0ch.	CCS ABMF maintains per NS-Tenant S-NSSAI the whole "Nb of UEs" quota when relevant.

1ch-b1.	Same as 1ch-b TS 32.256 [x]

1ch-b2.	For each allowed S-NSSAI in the received Allowed NSSAI: the CHF increases the "Nb of UEs" quota by one in the NS-Tenant's account balance for this S-NSSAI.

End of changes